| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | The Clarification of MIMO Feedback (16.3.9) |  |
| Date Submitted | 2010-03-05 |  |
| Source(s) | Jinyoung Chun, Wookbong Lee LG Electronics | jiny.chun@lge.com, wookbong.lee@lge.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE 802.16 Working Group Letter Ballot Recirc #31 |  |
| Abstract | This contribution provides the clarification of MIMO feedback. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m draft. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

The Clarification of MIMO Feedback (16.3.9)

Jinyoung Chun, Wookbong Lee

LG Electronics

- Introduction

In this contribution, I would like to clarify the MIMO feedback formats in views of the follows:

- Delete the duplicated words.

- Clarify the naming to Best_Subbands_index

- Clarify the period of Event-driven information

- Define the location of PFBCH indicator

- Text proposal in the P802.16m/D4

Proposed text has been underlined in blue and deleted text has been struck through in red. Existing draft text is shown in black.

------------------------------------------------   Text Start  -----------------------------------------------------------

[Modify the text in line 6 of page 636 in the section 16.3.9.3.11 as below:]

16.3.9.3.1.1 Primary fast feedback control channel

The UL PFBCH carries 6 bits of information, providing feedback contents in Table 904.

Table 904 – PFBCH Feedback Content

| PFBCH Feedback Content | Related MIMO feedback mode | Description/Notes |
| --- | --- | --- |
| CQI | 0,1,2,3,4,5,6,7 | 1) Wideband CQI 2) Subband CQI for Best -1 subband |
| STC Rate Indicator | 0,1,2,3 |  |
| Subband index | 2,3,5,6 | Subband selection for best-1 subband |
| PMI | 3,4,6,7 | 1) Wwideband PMI 2) Ssubband PMI for best-1 subband |
| EDI type 0 Event-driven Indicator (EDI) for request for switching MFM | N/A | Event-driven indicator to Indicate request to switch MIMO feedback mode between distributed and localized allocations. |
| EDI type 1 Event-driven Indicator (EDI) for Bandwidth Request Indicator | N/A | Event-driven indicator This is used to request UL bandwidth. 2 sequences (two services) |
| EDI type 2 Event-driven Indicator (EDI) for Frequency partition selection (FPS) | N/A | Event-driven indicator in order that AMS informs ABS about the frequency partition index (for MIMO feedback modes (0,1,4,7) |
| EDI type 3 Event-driven Indicator (EDI) for Buffer management | N/A | Event-driven indicator to iIndicates occupancy status of HARQ soft buffer |

[Add the text in line 7 of page 643 in the section 16.3.9.3.1.2 as below:]

16.3.9.3.1.2 Secondary fast feedback control channel

The UL SFBCH carries narrowband CQI and MIMO feedback information. The number of information bits carried in the SFBCH ranges from 7 to 24. The number of bits carries in the fast feedback channel can be adaptive. The feedback contents in Table 908 are carried from LSB in order of Feedback Fields in Feedback formats.

[Modify the text from line 49 of page 647 in the section 16.3.9.3.1.5 Feedback Formats as below:]

Feedback format for MFM 2

The detailed format is listed in Table 912. Short term report happens in every reporting period as defined in feedback allocation A-MAP IE, long term report will puncture short term report according to long term feedback period in feedback allocation A-MAP IE. The long period report shall start by puncturing the first short period report. If the PFBCH indicator in SFBCH is set to '1', PFBCH is transmitted by puncturing SFBCH in the next feedback opportunity regardless of short-term and long-term feedback using encoding type 0. When q = 0, only the long period reports shall be sent. q = 0 shall not be set if Measurement Method Indication = 0b1.

The CQI of subband m m shall be computed as follows: (Subband m CQI index) = (Subband avg CQI index)

+ (Subband m differential CQI), using the latest available reports of Subband avg CQI and Subband differential CQI. Subband avg CQI index is an average measure of the CQI over the M reported subbands. The possible differential CQI values are {-1, 0, +1, +2}, which is the unit of the MCS level. The AMS shall ensure that the reported differential CQI will produce a value of subband m CQI in the range of 0 to 15.

Table 912 shows feedback formats for MIMO feedback mode 2 when Measurement Method Indication = 0b0 in Feedback Allocation A-MAP IE (operation outside the open-loop region).

Table 912 Feedback formats for MFM 2 (outside OL region)

| Feedback Format | FBCH | Number of reports | Report Period | Feedback Fields | Description/Notes |
| --- | --- | --- | --- | --- | --- |
| 0 (MM =1) | PFBCH | 2 | Short | Subband CQI and STC rate or any type of EDI | Joint encoding of CQI and STC rate with PFBCH encoding Type 0 Encoding type 0 No short term report when q=0 |
|  |  |  | Long | Best_subbands_index | PFBCH eEncoding Type 1 |
| 1 (MM =1) | SFBCH | 1 | Long | Best_subbands_index, Subband CQI, STC rate, PFBCH indicator | Support of STC rate 1 to 8 |
| 2 (MM =min{5, YSB} | SFBCH | 2 | Short | Subband avg CQI, fFor (mm=1:MM){ Subband differential CQI } PFBCH indicator | No short term report when q=0 |
|  |  |  | Long | Best_subbands_index, Wideband STC rate, PFBCH indicator | Support of STC rate 1 to 8 |
| 3 (MM =min{10, YSB} | SFBCH | 2 | Short | Subband avg CQI, fFor (mm=1:MM){ Subband differential CQI } | No short term report when q=0 |
|  |  |  | Long | Best_subbands_index, Wideband STC rate, PFBCH indicator | Support of STC rate 1 to 8 |

Table 913 shows Feedback formats for MIMO feedback mode 2 when Measurement Method Indication = 0b1 in Feedback Allocation A-MAP IE (operation inside the open-loop region).

Table 913 Feedback formats for MFM 2 (inside OL region)

| Feedback Format | FBCH | Number of reports | Report Period | Feedback Fields | Description/Notes |
| --- | --- | --- | --- | --- | --- |
| 0 (MM =1) | PFBCH | 2 | Short | Subband CQI for STC rate = 1/2 or any type of EDI | Joint encoding of CQI with STC rate = 1/2 PFBCH eEncoding Type 3 |
|  |  |  | Long | Best_subbands_index | PFBCH eEncoding Type 1 |
| 1 (MM =1) | SFBCH | 1 | Long | Best_subbands_index, Subband CQI, PFBCH indicator | STC rate = 1/2 |
| 2 (MM =min{5, YSB} | SFBCH | 2 | Short | Subband avg CQI, fFor (mm=1:MM){ Subband dDifferential CQI } PFBCH indicator | STC rate = 1/2 |
|  |  |  | Long | Best_subbands_index, PFBCH indicator |  |
| 3 (MM =min{10, YSB} | SFBCH | 2 | Short | Subband avg CQI, fFor (mm=1:MM){ Subband dDifferential } |  |
|  |  |  | Long | Best_subbands_index, PFBCH indicator |  |

Feedback format for MFM 3

The detailed format is listed in Table 914. Short term report happens in every reporting period as defined in feedback allocation A-MAP IE, long term report will puncture short term report according to long term feedback period in feedback allocation A-MAP IE. The long period report shall start by puncturing the first short period report. For feedback format 0 with 3 reports using the PFBCH, Subband PMI shall be transmitted first after every report of Best_subbands_index. Subband CQI and subband PMI are then transmitted alternately and Best_subbands_index is transmitted in the long term period. If the PFBCH indicator in SFBCH is set to '1', PFBCH is transmitted using encoding type 0 by puncturing SFBCH in the next feedback opportunity regardless of short-term and long-term feedback. When q=0, only the long period reports shall be sent. q=0 shall not be set with the differential feedback mode.

The CQI of subband m m shall be computed as follows for the first short period report following a long period report: (Subband m CQI index) = (Subband avg CQI index) + (Subband m differential CQI). Subband avg CQI index is an average measure of the CQI over the M reported subbands. The possible differential CQI values are {-1, 0, +1, +2}, which is the unit of the MCS level. The AMS shall ensure that the reported differential CQI will produce a value of subband m CQI in the range of 0 to 15.

If the ABS has 4 transmit antennas and assigns MFM3 with feedback format 3, then it shall set Codebook_subset = 0b1 except if Codebook_mode indicates the differential codebook-based feedback mode.

Table 914 Feedback formats for MFM 3

| Feedback Format | FBCH | Number of reports | Report Period | Feedback Fields | Description/Notes |
| --- | --- | --- | --- | --- | --- |
| 0 (MM =1) | PFBCH | 3 | Short | Subband CQI or any type of EDI | PFBCH eEncoding Type 0 No short term report when q=0 |
|  |  |  | Short | Subband PMI | PFBCH eEncoding Type 2 STC rate = 1 No short term report when q=0 |
|  |  |  | Long | Best_subbands_index | PFBCH eEncoding Type 1 |
| 1 (MM =2) | SFBCH | 2 | Short | fFor (mm=1:MM){ Subband differential CQI, Subband PMI } PFBCH indicator | No short term report when q=0 |
|  |  |  | Long | Best_subbands_index, Wideband STC rate, Subband avg CQI, PFBCH indicator |  |
| 1 (M=2) 2(M=3) | SFBCH | 2 | Short | fFor (mm=1:MM){ Subband differential CQI, Subband PMI } | No short term report when q=0 |
|  |  |  | Long | Best_subbands_index, Wideband STC rate, Subband avg CQI, PFBCH indicator |  |
| 3 (MM =4) | SFBCH | 2 | Short | fFor (mm=1:MM){ Subband differential CQI, Subband PMI } | No short term report when q=0 |
|  |  |  | Long | Best_subbands_index, Wideband STC rate, Subband avg CQI, PFBCH indicator |  |

The detailed format for MIMO feedback mode 3 with differential codebook is in Table 915. Short term report happens in every reporting period as defined in Feedback Allocation A-MAP IE. For M > 1, two long term reports will puncture two short term reports continuously according to long term feedback period in Feedback Allocation A-MAP IE. The long term feedback which conveys Best_subband_index shall be transmitted first. For M = 1, the long term report will puncture one short term report according to long term feedback period in Feedback Allocation A-MAP IE.

Table 915 Feedback formats for MFM 3 for differential codebook

| Feedback Format | FBCH | Number of reports | Report Period | Feedback Fields | Description/Notes |
| --- | --- | --- | --- | --- | --- |
| 0 (MM =1) | SFBCH | 2 | Short | for (m=1:M){ Subband differential CQI, Subband differential PMI, } Padding PFBCH indicator | Padding 1 bit: if Nt = 2 0 bit: otherwise |
|  |  |  | Long | Best_subbands_index, STC rate, Base PMI, PFBCH indicator | PMI for reset. Base PMI is a PMI from the base codebook or the base codebook subset. |
| 1 (MM =2) | SFBCH | 3 | Short | fFor (mm=1:MM){ Subband differential CQI, Subband differential PMI } PFBCH indicator |  |
|  |  |  | Long | Best_subbands_ index, Subband avg_CQI, PFBCH indicator |  |
|  |  |  | Long | fFor (mm=1:MM){ Base PMI }, Wideband STC rate | PMI for reset. Base PMI is a PMI from the base codebook or the base codebook subset. |
| 2(MM =3) | SFBCH | 3 | Short | fFor (mm=1:MM){ Subband differential CQI, Subband differential PMI } PFBCH indicator |  |
|  |  |  | Long | Best_subbands_ index, Wideband STC rate, Subband avg_CQI, PFBCH indicator |  |
|  |  |  | Long | fFor (mm=1:MM){ Base PMI } | PMI for reset. Base PMI is a PMI from the base codebook or the base codebook subset. |
| 3 (MM =4) | SFBCH | 3 | Short | fFor (mm=1:MM){ Subband differential CQI, Subband differential PMI } |  |
|  |  |  | Long | Best_subbands_ index, Wideband STC rate, Subband avg_CQI, PFBCH indicator |  |
|  |  |  | Long | fFor (mm=1:MM){ Base PMI } | PMI is a PMI from the base codebook or the base codebook subset. |

Feedback format for MFM 5

The detailed format is listed in Table 916. Short term report happens in every reporting period as defined in Feedback Allocation A-MAP IE, long term report will puncture short term report according to long term feedback period in Feedback Allocation A-MAP IE. The long period report shall start by puncturing the first short period report. If the PFBCH indicator in SFBCH is set to '1', PFBCH is transmitted by puncturing SFBCH in the next feedback opportunity regardless of short-term and long-term feedback using encoding type 0. When q = 0, only the long period reports shall be sent. q = 0 shall not be set if Measurement Method Indication = 0b1.

The CQI of subband m m shall be computed as follows: (Subband m CQI index) = (Subband avg CQI index) + (Subband m differential CQI), using the latest available reports of Subband avg CQI and Subband differential CQI. Subband avg CQI index is an average measure of the CQI over the M reported subbands. The possible differential CQI values are {-1, 0, +1, +2}, which is the unit of the MCS level. The AMS shall ensure that the reported differential CQI will produce a value of subband m CQI in the range of 0 to 15.

The feedback formats for MIMO feedback mode 5 are the same for operation inside or outside the open-loop region.

Table 916 Feedback formats for MFM 5

| Feedback Format | FBCH | Number of reports | Report Period | Feedback Fields | Description/Notes |
| --- | --- | --- | --- | --- | --- |
| 0 (MM =1) | SFBCH | 1 | Long | Best_subbands_index, Subband CQI, Stream Index, PFBCH indicator |  |
| 1 (MM =2) | SFBCH | 2 | Short | Subband avg_CQI, fFor (mm=1:MM){ Subband differential CQI, Stream Index } PFBCH indicator | No short term report when q=0 |
|  |  |  | Long | Best_subbands_index, PFBCH indicator |  |
| 2 (MM =3) | SFBCH | 2 | Short | Subband avg_CQI fFor (mm=1:MM){ Subband differential CQI, Stream Index } PFBCH indicator | No short term report when q=0 |
|  |  |  | Long | Best_subbands_index, PFBCH indicator |  |
| 3 (MM =5) | SFBCH | 2 | Short | Subband avg_CQI, fFor (mm=1:MM){ Subband differential CQI, Stream Index } | No short term report when q=0 |
|  |  |  | Long | Best_subbands_index, PFBCH indicator |  |

Feedback format for MFM 6

The detailed format is listed in Table 917. Short term report happens in every reporting period as defined in feedback allocation A-MAP IE, long term report will puncture short term report according to long term feedback period in feedback allocation A-MAP IE. The long period report shall start by puncturing the first short period report. For feedback format 0 with 3 reports using the PFBCH, Subband PMI shall be transmitted first after every report of Best_subbands_index. Subband CQI and subband PMI are then transmitted alternately and Best_subbands_index is transmitted in the long term period. If the PFBCH indicator in SFBCH is set to '1', PFBCH is transmitted using encoding type 0 by puncturing SFBCH in the next feedback opportunity regardless of short-term and long-term feedback. When q=0, only the long term reports shall be sent. q = 0 shall not be set with the differential feedback mode.

The CQI of subband m m shall be computed as follows: (Subband m CQI index) = (Subband avg CQI index) + (Subband m differential CQI), using the latest available reports of Subband avg CQI and Subband differential CQI. Subband avg CQI index is an average measure of the CQI over the M reported subbands. The possible differential CQI values are {-1, 0, +1, +2}, which is the unit of the MCS level. The AMS shall ensure that the reported differential CQI will produce a value of subband m CQI in the range of 0 to 15.

Table 917 Feedback formats for MFM 6

| Feedback Format | FBCH | Number of reports | Report Period | Feedback Fields | Description/Notes |
| --- | --- | --- | --- | --- | --- |
| 0 (MM=1) | PFBCH | 3 | Short | Subband CQI or any type of EDI | PFBCH eEncoding Type 0 No short term report when q=0 |
|  |  |  | Short | Subband PMI | PFBCH eEncoding Type 2 No short term report when q=0 |
|  |  |  | Long | Best_subbands_index | PFBCH eEncoding Type 1 |
| 1 (MM =2) | SFBCH | 2 | Short | fFor (mm=1:MM){ Subband differential CQI, Subband PMI } PFBCH indicator | No short term report when q=0 |
|  |  |  | Long | Best_subbands_index, Subband avg CQI PFBCH indicator |  |
| 2 (MM =3) | SFBCH | 2 | Short | fFor (mm=1:MM){ Subband differential CQI, Subband PMI } | No short term report when q=0 |
|  |  |  | Long | Best_subbands_index, Subband avg CQI, PFBCH indicator |  |
| 3 (MM =4) | SFBCH | 2 | Short | fFor (mm=1:MM){ Subband differential CQI, Subband PMI } | No short term report when q=0 |
|  |  |  | Long | Best_subbands_index, Subband avg CQI, PFBCH indicator |  |

If the ABS has 4 transmit antennas and assigns MFM 6 with feedback format 3, then it shall set Codebook_subset = 0b1 except if Codebook_mode indicates the differential codebook-based feedback mode.

The detailed format for MIMO feedback mode 6 with differential codebook is in Table 918. Short term report happens in every reporting period as defined in feedback allocation A-MAP IE . For M M >1, two long term reports will puncture two short term reports continuously according to long term feedback period in Feedback Allocation A-MAP IE. For M M =1, the long term report will puncture one short term report according to long term feedback period in Feedback Allocation A-MAP IE.

Table 918 Feedback formats for MFM 6 for differential codebook

| Feedback Format | FBCH | Number of Reports | Report Period | Feedback Fields | Description/Notes |
| --- | --- | --- | --- | --- | --- |
| 0 (M=1) | SFBCH | 2 | Short | Subband CQI, Differential PMI, Padding PFBCH indicator | Padding 1 bit: if Nt = 2 0 bit: otherwise |
|  |  |  | Long | Subband index Best_subbands_index, Base PMI, PFBCH indicator | PMI for reset. Base PMI is a PMI from the base codebook or the base codebook subset. |
| 1 (M=2) | SFBCH | 3 | Short | fFor (m=1:M){ Subband differential CQI, Subband differential PMI } PFBCH indicator |  |
|  |  |  | Long | Best_subbands_ index, Subband avg_CQI, PFBCH indicator |  |
|  |  |  | Long | fFor (m=1:M){ Base PMI } | PMI for reset. Base PMI is a PMI from the base codebook or the base codebook subset. |
| 2(M=3) | SFBCH | 3 | Short | fFor (m=1:M){ Subband differential CQI, Subband differential PMI } PFBCH indicator |  |
|  |  |  | Long | Best_subbands_ index, Subband avg_CQI, PFBCH indicator |  |
|  |  |  | Long | fFor (m=1:M){ Base PMI } | PMI for reset. Base PMI is a PMI from the base codebook or the base codebook subset. |
| 3 (M=4) | SFBCH | 3 | Short | fFor (m=1:M){ Subband differential CQI, Subband differential PMI } PFBCH indicator |  |
|  |  |  | Long | Best_subbands_ index, Subband avg_CQI, PFBCH indicator |  |
|  |  |  | Long | fFor (m=1:M){ Base PMI } | PMI for reset. PMI is a PMI from the base codebook or the base codebook subset. |

-----------------------------------------------   Text End  -----------------------------------------------------------