| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Modification on the error handling for lost AAI_TRF-IND message (section 16.2.16.23.1) |  |
| Date Submitted | 2010-03-05 |  |
| Source(s) | Yeongmon Son, Hyunjeong Kang, Jungje Son, Rakesh Taori, Mingxia Xu Samsung Electronics Co., Ltd. | ym1004.son@samsung.com +82-31-279-5845 |
| Re: | “IEEE 802.16 Working Group Letter Ballot#31” |  |
| Abstract | The contribution proposes modification on SCH and SCEH to cover no UL transmission in case of both the system information change and the lost AAI_TRF-IND message. |  |
| Purpose | To be discussed and adopted by TGm for 802.16m. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Modification on the error handling for lost AAI_TRF-IND message

Yeongmoon Son, Hyunjeong Kang, Jungje Son, Rakesh Taori, Mingxia Xu

Samsung Electronics Co., Ltd.

- Introduction

In 16m/D4, if the AMS in Sleep Mode detects that the information it has is not up-to-date, then it shall not transmit in the Listening Window until it receives the up-to-date system information. Therefore, when the AMS lost AAI_TRF-IND message and detects 'out of date' of system information (i.e. change of S-SFH info.), the AMS cannot transmit AAI_TRF_IND-REQ message. As you know, it takes a little long time for the AMS to get entire information of s-SFH. On the contrary, the current Sleep Cycle can be reset. As a result, the AMS may miss the Listening Windows several times without transmitting an AAI_TRF_IND-REQ message. In that sense, it may be meaningless for the AMS to inquire the traffic indication in the frame in which the first lost traffic was transmitted.

We need to modify the SCH/SCEH as well as AAI_TRF_IND-REQ/RSP message.

- Suggested proposal

I summarize the suggested proposal as follows.

|  | In D4 | New proposal |
| --- | --- | --- |
| Parameters in AAI_TRF_IND-REQ | ‘Frame_Number’ | Not required |
| Parameters in AAI_TRF_IND-RSP | ‘Frame_Number’ and ‘Traffic Indication’ | The starting frame number of Next Scheduled Listening Window (10 bits) The Sleep Cycle Length (10bits) |

- If the AMS receives neither the traffic indication message nor any unicast data in the Listening Window, the AMS shall transmit AAI_TRF_IND-REQ after it has up-to-date system information.

- The ABS shall provide the following two parameters through AAI_TRF_IND-RSP message

  - Frame Number : the least significant 10 bits of the frame number in which incoming LW will start

  - Sleep Cycle Length : the length of Sleep Cycle which contains the incoming LW.

- Text to change in D4

-------------------------------  Text Start  ---------------------------------------------------

[Modify the paragraph on the page 301, line 31 as follows:]

16.2.16.2.3.1 Traffic Indication

……..

If the traffic indication message is lost or otherwise not detected by the AMS, the AMS shall stay awake for the rest of the Listening Window. If the AMS receives any unicast data during the listening window, then it shall assume that the traffic indication was positive. If the AMS receives neither the traffic indication message nor any unicast data in the Listening Window, the AMS shall remain awake until it receives its own traffic indication using AAI_TRF_IND-REQ/RSPSCH or SCEH exchange. The AMS shall then send an AAI_TRF_IND-REQ message,SCH, or SCEH to ask the ABS what was the traffic indication for the AMSthe location of next scheduled Listening Window. The ABS shall respond to the AMS by unicasting an AAI_TRF_IND-RSP message,SCH, or SCEH containing the traffic indication for that AMS the starting frame number and the length of next Sleep Cycle. On receiving the traffic indication SCH, or SCEH, the AMS shall behave in accordance with the traffic indication be synchronized with the next Sleep Cycle.

[Delete the section 16.2.3.26 on the page 112 as follows:]

16.2.3.26 AAI_TRF_IND-REQ

If the TIMF is set to 1, the AMS shall receive AAI_TRF-IND message in the first frame during its own Listening Window. However, if the traffic indication message is lost or otherwise not detected by the AMS, the AMS shall stay awake for the rest of the Listening Window. If the AMS receives any unicast data during the listening window, then it shall assume that the traffic indication was positive. If the AMS receives neither the traffic indication message nor any unicast data in the Listening Window, the AMS shall send an AAI_TRF_IND-REQ message to the ABS in order to ask which kind of traffic indication (i.e., positive or negative traffic indication) the ABS sent to it. The ABS shall respond to the AMS by sending an AAI_TRF_IND-RSP with traffic indication for the AMS.

| Table 704—AAI_TRF_IND-REQ message format |  |  |
| --- | --- | --- |
| Name | Value | Usage |
| Frame_Number | 0-15 | This indicates the least significant 4 bits of frame number in which the AMS expected to receive the AAI_TRF-IND |

Parameters shall be as follows

Frame_Number

This indicates the least significant 4 bits of frame number in which the AMS expected to receive the AAI_TRF-IND.

[Delete the section 16.2.3.27 on the page 113, as follows:]

16.2.3.27 AAI_TRF_IND-RSP

When the ABS receives AAI_TRF_IND-REQ message from an AMS, the ABS shall respond to the AMS by sending AAI_TRF_IND-RSP message with the traffic indication for the AMS. When the AMS receives AAI_TRF_IND-RSP from the ABS, the AMS shall update the current Sleep Cycle based on the Traffic_Indication in the AAI_TRF_IND-RSP.

| Table 705—AAI_TRF_IND-RSP message format |  |  |
| --- | --- | --- |
| Name | Value | Usage |
| Frame_Number | 0-15 | This indicates the least significant 4 bits of frame number in which the AMS expected to receive the AAI_TRF- |
| Traffic_Indication | 0: Negative Traffic Indication 1: Positive Traffic Indication | It indicates the traffic indication which the ABS has transmitted to the AMS |

Parameters shall be as follows

Frame_Number

This indicates the least significant 4 bits of frame number in which the ABS sent the AAI_TRF-IND message to the AMS.

Traffic_Indication

This indicates the traffic indication of which the ABS informed the AMS in the specified frame number.

0 = the ABS sent the negative traffic indication to the AMS in the frame

1 = the ABS sent the positive traffic indication to the AMS in the frame

[Modify the table 660 on the page 43, line 50 as follows:]

| Table 660—Sleep Control header format |  |  |
| --- | --- | --- |
| Name | Value | Usage |
| … | … | … |
| SCH sub-type | 3 | 0b000 = Listening Window control 0b001 = Resume Sleep Cycle Indication 0b010 = Sleep cycle configuration switch 0b011 = Sleep Cycle control 0b100 = Multi-Carrier Listening Window control 0b101= Error Handling for lost AAI_TRF-IND message 0b1010b110~0b111 = reserved |
| Response Indication | 1 | 0: This indicates the request 1: This indicates the response (i.e., acknowledgement) to the request |
| … | … | …. |
| } |  |  |
| } |  |  |
| else if (SCH sub-type = 0b101 && Response Indication == 1) { |  | It implies the ABS’s response to the AMS’s request for error handling of lost AAI_TRF-IND message- |
| Frame Number | 10 | The least significant 10 bits of the frame number in which incoming LW will start |
| Sleep Cycle Length | 10 | The length of Sleep Cycle which contains the next scheduled Listening Window. If the AMS receives the negative traffic indication during the next scheduled Listening Window, the (current) Sleep Cycle shall be set to this value at that time, |
| } |  |  |
| Padding | variable | For byte alignment |
| } |  |  |

[Modify the table 669 on the page 53, line 48 as follows:]

| Table 669—SCEH format |  |  |
| --- | --- | --- |
| Name | Value | Usage |
| … | … | … |
| SCEH sub-type | 3 | 0b000 = Listening Window control 0b001 = Resume Sleep Cycle Indication 0b010 = Sleep cycle configuration switch 0b011 = Sleep Cycle control 0b100 = Multi-Carrier Listening Window control 0b101 = Error Handling for lost AAI_TRF-IND message 0b1010b110~0b111 = reserved |
| Response Indication | 1 | 0: This indicates the request 1: This indicates the response (i.e., acknowledgement) to the request |
| … | … | …. |
| } | — | — |
| } | — | — |
| else if (SCH sub-type = 0b100 && Response Indication == 1) { |  | It implies the ABS’s response to the AMS’s request for error handling of lost AAI_TRF-IND message- |
| Frame Number | 10 | The least significant 10 bits of the frame number in which incoming LW will start |
| Sleep Cycle Length | 10 | The length of Sleep Cycle which contains the next scheduled Listening Window. If the AMS receives the negative traffic indication during the next scheduled Listening Window, the (current) Sleep Cycle shall be set to this value at that time, |
| } |  |  |
| Padding | variable | For byte alignment |
| } |  |  |

-------------------------------  Text End  ---------------------------------------------------