| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text on Concurrent Transmission of Data and Control Channels (16.3.9.4.6) |  |
| Date Submitted | 2010-03-05 |  |
| Source(s) | Dongcheol Kim, Wookbong Lee, Jin Sam Kwak, HanGyu Cho LG Electronics, Rongzhen Yang Intel Corp, Jeongho Park Samsung | Voice : +82-31-450-7919 E-mail: dongcheol.kim@lge.com,wookbong.lee@lge.com Rongzhen.yang@intel.com jeongho.jh.park@samsung.com |
| Re: | Call for LB #31 on “ P802.16m/D4” Proposed text on P802.16m/D4 16.3.9.4.6 |  |
| Abstract | Text Proposal for concurrent transmission rule |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m D5 Draft |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text on Concurrent Transmission of Data and Control Channels (16.3.9.4.6)

Dongcheol Kim, Wookbong Lee, Jin Sam Kwak, HanGyu Cho

LG Electronics

Rongzhen Yang 
Intel Corp

Jeongho Park

Samsung

# Introduction

This contribution includes the proposed text on the concurrent transmission of data and control channels(16.3.9.4.6) for the IEEE 802.16m/D4 Amendment.

# References

[1] IEEE P802.16m/D4, “Draft Amendment to IEEE Standard for Local and metropolitan area network: Air Interface for Fixed and Mobile Broadband Wireless Access Systems”.

# Text proposal

Instruction to editor;

Black text: existing D4 text

Red and strike out text: for deletion

Blue and underline text: for addition

============================== Start of Proposed Text =================================

Line 63, page 660, include the following texts;

16.3.9.4.6 Concurrent transmission of uplink control channel and data

In case of simultaneous transmission among control channels, uplink transmission power of assigned channels is determined based on the channel list in Table 920 in descending order until the total transmission power reaches maximum power limitation of AMS.

If AMS has not sufficient power to transmit an assigned channel, compared to the power level determined by equation (287), then the AMS transmit this channel with the remaining insufficient power.

Table 920-The priority of uplink transmit channels

Channel Type

HARQ feedback

PFBCH/SFBCH

Synchronized Ranging

Sounding

DATA

Bandwidth Request

============================== End of Proposed Text =================================