| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text on The Configuration of UL Power Control Information for Handover (16.3.9.4) |  |
| Date Submitted | 2010-03-05 |  |
| Source(s) | Dongcheol Kim, Wookbong Lee, Jin Sam Kwak, HanGyu Cho LG Electronics | Voice : +82-31-450-7919 E-mail: dongcheol.kim@lge.com,wookbong.lee@lge.com |
| Re: | Call for LB #31 on “ P802.16m/D4” Proposed text changes to P802.16m/D4 |  |
| Abstract |  |  |
| Purpose | For member’s review and adoption into P802.16m D5 Draft |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text on The Configuration of Power Control Information for Handover (16.3.9.4)

Dongcheol Kim, Wookbong Lee, Jin Sam Kwak, HanGyu Cho

LG Electronics

# Introduction

In the current IEEE 802.16m/D4, there are no details on UL power control just after handover. More precisely, there is no description on how to obtain parameters required for UL power control during handover procedure.

Since power control-related parameters are transmitted in AAI-SCD message, AAI-UL Noise and Interference Level Broadcast Message, and S-SFH SP3 IE, it may not be guaranteed that a handover MS obtains all the parameters within the required handover latency.

# References

[1] IEEE P802.16m/D4, “Draft Amendment to IEEE Standard for Local and metropolitan area network: Air Interface for Fixed and Mobile Broadband Wireless Access Systems”.

# Text proposal

Instruction to editor;

Black text: existing D4 text

Red and strike out text: for deletion

Blue and underline text: for addition

Remedy #1. Line 54, page 659, include following texts;

============================== Start of Proposed Text #1 =============================

16.3.9.4.x Power Control for Handover

When AMS operates Handover, the AMS shall calculate transmit power level by equation (287) using serving ABS’s power control parameters and OffsetData and OffsetControl value shall be determined by OffsetData and OffsetControl which is transmitted via AAI-HO-CMD message.

After AMS decodes AAI-SCD and AAI-NI message of target ABS successfully, then the transmit power level shall be determined by equation (287) using target ABS’s power control parameters, and OffsetData and OffsetControl shall be 0.

============================== End of Proposed Text #1 ===============================

Remedy #2. Line 51, page 83, include following texts;

============================== Start of Proposed Text  #2 =============================

Table 686-Parameters for AAI_HO-CMD message

| Name | Value | Usage |
| --- | --- | --- |
| … | … | … |
| OffsetControl | 6 bits | OffsetControl for target ABS. |
| OffsetData | 6 bits | OffsetData for target ABS |

============================== End of Proposed Text #2 ===============================