| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text for E-MBS configuration indicator (16.9.3) |  |
| Date Submitted | 2010-03-05 |  |
| Source(s) | Chiu-Wen Chen, Chun-Yen Hsu, Kanchei(Ken) Loa, Youn-Tai Lee, Yi-Ting Lin, Tsung-Yu Tsai Institute for Information Industry (III) Shiann-Tsong Sheu National Central University (NCU) Whai-En Chen, Shih-Yuan Cheng National Ilan University (NIU) | hcy@iii.org.tw |
| Re: | In response to IEEE 802.16 Working Group Working Group Letter Ballot #31: Announcement |  |
| Abstract | This contribution proposes text for E-MBS configuration indicator |  |
| Purpose | For discussion and approval by IEEE 802.16 TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text for E-MBS configuration indicator (16.9.3)

Chiu-Wen Chen, Chun-Yen Hsu, Kanchei(Ken) Loa, Youn-Tai Lee, Yi-Ting Lin, Tsung-Yu Tsai
Institute for Information Industry (III)

Shiann-Tsong Sheu
National Central University (NCU)

Whai-En Chen, Shih-Yuan Cheng
National Ilan University (NIU)

Introduction

Neighbor ABS’s E-MBS zone affiliations are carried in both AAI_NBR-ADV and AAI_E-MBS-CFG for providing service continuity. Including the same information in these two messages is not necessary. It is suggested to remove neighbor ABS’s E-MBS zone affiliations from AAI_NBR-ADV. Another reason of removing this information from AAI_NBR-ADV is that idle AMSs may miss AAI_NBR-ADV.

Proposed text for the P80216m_D4:

----------------------------------------- Start of the Text Proposal # 1----------------------------------------------

[Make the following changes to Section 16.9.3.1 in Page 753 beginning Line 30 in P802.16m/D4.]

#### 16.9.3 E-MBS Protocol Features and Functions

##### 16.9.3.1 E-MBS Configuration Indicators

For each neighbor E-MBS_Zone, the MSTID and FID Mappings List between serving and neighbor E-MBS Zones shall be included if the serving and neighbor ABSs belong to different E-MBS Zones which provide service continuity.

The mappings list includes the followings:

Num_Neighbor_E-MBS_Zones (3 bits)

AAI_NBR-ADV Change Count (3 bits)

for (i = 0; i< Num_Neighbor_E-MBS_Zones; i++) {

Neighbor_E-MBS_Zone_ID (7 bits)

Physical Carrier Index (6 bits)

Number_Neighbor_ABS (8 bits)

for (j = 0; j< Number_Neighbor_ABS; j++) {

Index of BS (8 bits)

}

Num_MSTID_FID Mappings (4 bits)

for (k = 0; k< Num_MSTID_FID Mappings; k++) {

Current_MSTID and FID (k),

New_MSTID and FID (k),

}

}

----------------------------------------- End of the Text Proposal # 1----------------------------------------------

----------------------------------------- Start of the Text Proposal # 2----------------------------------------------

[Make the following changes to Section 16.9.3.2 in Page 754 beginning Line 3 in P802.16m/D4.]

##### 16.9.3.2 E-MBS Zone Configuration

Different MSTIDs and FIDs may be used in different E-MBS Zones for the same multicast and broadcast service flow. A multicast and broadcast zone identifier (E-MBS_ZONE_ID) is used to indicate a service area in which a MSTID and FID for a broadcast and multicast service flow are valid. An ABS that supports E-MBS shall include the E-MBS Zone identifier(s) to which it belongs in the AAI-E-MBS_CFG message. The E-MBS Zone identifier shall not be '0'.

When the E-MBS Zone identifier list appears in AAI_NBR_ADV message with only one value of '0' for some of ABS's, it indicates that the corresponding neighbor ABSs are not affiliated with any E-MBS zone.

----------------------------------------- End of the Text Proposal # 2----------------------------------------------

[1] IEEE P80216m/D4, “P802.16m/D4: Advanced Air Interface”