| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Editorial Text modification on Power Control Section (16.3.9.4) |  |
| Date Submitted | 2010-03-05 |  |
| Source(s) | Dongcheol Kim, Wookbong Lee, Jin Sam Kwak, HanGyu Cho LG Electronics Rongzhen Yang, Hujun Yin, Yang-seok Choi Intel Corp. | Voice : +82-31-450-7919 E-mail: dongcheol.kim@lge.com,wookbong.lee@lge.com Email : Rongzhen.yang@intel.com |
| Re: | Call for LB #31 on “ P802.16m/D4” Proposed text changes to P802.16m/D4 16.3.9.4 |  |
| Abstract | This contribution propose the editorial text modification on power control section for the IEEE8023.16m/D4 |  |
| Purpose | For member’s review and adoption into P802.16m D5 Draft |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Editorial Text modification on Power Control Section (16.3.9.4)

Dongcheol Kim, Wookbong Lee, Jin Sam Kwak, HanGyu Cho

LG Electronics

Rongzhen Yang, Hujun Yin, Yang-seok Choi
Intel Corp.

# Introduction

This contribution includes the editorial text modification on the power control section for the IEEE 802.16m/D4.

# References

[1] IEEE P802.16m/D4, “Draft Amendment to IEEE Standard for Local and metropolitan area network: Air Interface for Fixed and Mobile Broadband Wireless Access Systems”.

# Text proposal

Instruction to editor;

Black text: existing D4 text

Red and strike out text: for deletion

Blue and underline text: for addition

Line 63, page 660, include following texts;

============================== Start of Proposed Text =================================

16.3.9.4 Uplink Power Control

Uplink power control is supported for both an initial calibration and periodic adjustment on transmit power without loss of data. The uplink power control algorithm determines the transmission power of an OFDM symbol to compensate for the pathloss, shadowing and fast fading. Uplink power control shall intend to control inter-cell interference level.

A transmitting AMS shall maintain the transmitted power density, unless the maximum power level is reached. In other words, when the number of active LRU allocated to a user is reduced, the total transmitted power shall be reduced proportionally by the AMS, as long as there is no additional change of parameters for power control. When the number of LRU is increased, the total transmitted power shall also be increased proportionally. However, the transmitted power level shall not exceed the maximum levels dictated by signal integrity considerations and regulatory requirements.

For interference level control, current interface level of each ABS may be shared among ABSs.

The power per subcarrier and per stream shall be calculated according to Equation (287).

Where

SINRTarget is the target uplink SINR received by the ABS.

P is the TX power level (dBm) per stream and per subcarrier for the current transmission.

L is the estimated average current DL propagation loss calculated by AMS. It shall include AMS’s Tx

antenna gain and path loss.

NI is the estimated average power level (dBm) of the noise and interference per subcarrier at the ABS,

as indicated in the AAI_ULPC_NI message.

Offset is a correction term for AMS-specific power offset. It is controlled by the ABS through power control messages. There are two kinds of Offset values (Offsetdata and OffsetControl) that are used for data and control, respectively. Further details are given in following sections (16.3.9.4.1, 16.3.9.4.2 and 16.3.9.4.3).

The estimated average current DL propagation loss, L, shall be calculated based on the total power received on the active subcarriers of the frame preamble.

16.3.9.4.1 Power Control for Data Channel

For data channel transmission, the SINRTarget term in Equation (287) is calculated according to Equation (288):

SINRTarget=10log10max10(SINRMIN(dB)10), γIoT×SIRDL−α−β×10log10(TNS)                      (288)

where Equation (288) is the target SINR value for IoT control and tradeoff between overall system throughput and cell edge performance, decided by the control parameter  and SINRMIN(dB). The parameters used in Equation (288) are broadcasted in AAI_SCD message unless otherwise noted. The explanation for the parameters is as follows:

SINRMIN(dB) is the SINR requirement (SINRMIN_Data dataSinrMin) for the minimum rate expected by ABS.

is the fairness and IoT control factor broadcasted.

SIRDL is the linear ratio of the downlink signal to. interference power, measured by the AMS.

is the factor according to the number of receive antennas at the ABS.

is set to be zero or one to determine the influence of TNS on SINRTarget.

TNS is the Total Number of Streams in the LRU indicated by UL A-MAP IE. In case of SU-MIMO,

this value shall be set to Mt where Mt is the number of streams for AMS. In case of CSM, TNS is

the aggregated number of streams.

When calculated data channel SINRTarget is higher than SINRmax_Data dataSinrMax defined in AAI_SCD message, SINRTarget shall be set to SINRmax_Data dataSinrMax.

For a data channel transmission, the Offset in Equation (287) shall be set to the value OffsetData conveyed in an AAI UL_POWER_ADJUST message. The value used shall be taken from the most recent message which preceded the first uplink subframe of the frame by more than TprocTDL_Rx_Processing. The default value of OffsetData shall be initialized to 0 if AMS never get AAI_UL_POWER_ADJUST message.

16.3.9.4.2 Power Control for Control Channels

In the case of control channel transmission, except for initial ranging and sounding transmission, SINRTarget in Equation (287) is set according to Table 919. The parameters in Table 919 are conveyed in the AAI_SCD message.

Table 911 SINRtarget Parameters for Control Channel

| Control Channel Type | SINRtarget Parameters |
| --- | --- |
| HARQ Feedback | SINR_Target_HARQ targetHarqSinr |
| Synchronized Ranging | SINR_Target_SyncRanging targetSyncRangingSinr |
| P-FBCH | SINR_Target_PFBCH targetPfbchSinr |
| S-FBCH | SINR_Target_SFBCH_Base targetSfbchBaseSinr |
|  | SINR_Target_SFBCH_Delta targetSfbchDeltaSinr |
| Bandwidth Request | SINR_Target_BWRequest targetBwRequestSinr |

For HARQ Feedback, Synchronized Ranging, P-FBCH and Bandwidth Request, the SINRTarget values are indicated directly in Table 911. For S-FBCH channel, the SINRTarget value is defined in Table equation 289:

SINRTarget(SFBCH)=SFBCHBase+(l−lmin)×SFBCHDelta			(289)

Where

SFBCHBase is base SINRtarget value signaled by SINR_Target_SFBCH_BASE targetSfbchBaseSinr;

SFBCHDelta is differential SINRtarget value signaled by SINR_Target_SFBCH_Delta targetSfbchDeltaSinr;

lis S-FBCH payload information bits number defined in 16.3.9.2.1.2;

lminis minimum S-FBCH payload information bits number (lmin= 7 ) defined in 16.3.9.2.1.2;

For a control channel transmission, the Offset in Equation (287) shall be set to the value OffsetControl conveyed in an AAI UL_POWER_ADJUST message. The value used shall be taken from the most recent message which preceded the first uplink subframe of the frame by more than TprocTDL_Rx_Processing. The default value of OffsetControl shall be initialized to 0 if AMS never get AAI_UL_POWER_ADJUST message.

16.3.9.4.3 Power Correction using PC-A-MAP

The ABS may change the AMS’s TX power through direct power adjustment by PC-A-MAP. When AMS receives its PC-A-MAP IE from the ABS, it shall modify its OffsetControl value according to Equation (290).

OffsetControl = OffsetControl + ΔPowerAdjust                                                (290)

where  is the power correction value indicated by ABS through PC-A-MAP.

Note that OffsetControl is initially set to zero until AMS receives the first AAI_UL_POWER_ADJUST message. And if AMS receives AAI_UL_Power_ADJUST AAI_UL_POWER_ADJUST message at i-th frame, the AMS’s OffsetControl shall be replaced by the value indicated by the message from the (i+1)th frame. Also PC-A-MAP IE from (i+1)th frame is effective on this new value.

16.3.9.4.4 Initial Ranging Channel Power Control

For initial ranging, AMS sends initial ranging code at a randomly selected ranging channel. The initial transmission power is decided according to measured RSS. If AMS does not receive a response, it may increase its power level by PIR,Step and may send a new initial ranging code, where PIR,Step is the step size to ramp up, which is 2 dB. AMS could further increase the power until maximum transmit power is reached.

The initial transmission power of AMS is calculated as:

(xx1)

where EIRxPIR,min is the minimum targeting receiving power and BS_EIRP is the transmission power of the BS, which are obtained from S-SFH SP2 and SP1, respectively, and RSS is the received signal strength measured by the AMS.

In the case that the Rx and Tx gain of the AMS antenna are different, the AMS shall use Equation (273 291):

(291)

where GRx_MS is the antenna gain of AMS RX, and GTx_MS is the antenna gain of AMS TX.

16.3.9.4.5 Sounding Channel Power Control

Power control for the UL sounding channel is supported to manage the sounding quality. AMS’s transmit power for UL sounding channel is controlled separately according to its sounding channel target SINR value. The power per subcarrier shall be maintained for the UL sounding transmission as shown in Equation (287) of 16.3.9.4.1.

In Equation (287), SINRTarget is the sounding channel target SINR, which is set according to the DL SIR of the AMS defined by parameter SIRDL. In order to maintain the UL sounding quality, the different target SINR values are assigned according to the DL SIR of each AMS; the AMS with high DL SIR applies relatively high target SINR and the AMS with low DL SIR applies relatively low target SINR.

The SINRTarget for sounding channel shall be calculated from Equation (287) using the following parameter settings: SINRMIN is the minimum SINR requirement expected by ABS which is set to SINRmin_SOUNDING soundingSinrMin,is set to the IoT control factor for sounding channel GAMMA_IOT_SOUNDING gammaIotSounding, SIRDL is the ratio of the downlink signal vs. noise and interference power, measured by the AMS, and  parameters shall be set to 0.

When calculated sounding SINRTarget is higher than soundingSinrMax SINRmax_SOUNDING, the SINRTarget shall be set to soundingSinrMax SINRmax_SOUNDING.

All parameters necessary for sounding channel power control are transmitted through AAI_SCD message in 16.2.3.29.

16.3.9.4.6 Concurrent transmission of uplink control channel and data

In case of simultaneous transmission among control channels, uplink transmission power of assigned channels is determined based on the channel list in Table 920 in descending order until the total transmission power reaches maximum power limitation of AMS.

Table 920—The priority of uplink transmit channels

Channel Type

HARQ feedback

PFBCH/SFBCH

Synchronized Ranging

Sounding

DATA

Bandwidth Request

============================== End of Proposed Text =================================