| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification on Response Indication in SCH and SCEH (16.2.2.1.3.4) |  |
| Date Submitted | 2010-03-16 |  |
| Source(s) | Yeongmon Son, Hyunjeong Kang, Jungje Son, Rakesh Taori Samsung Electronics Co., Ltd. | ym1004.son@samsung.com +82-31-279-5845 |
| Re: | “IEEE 802.16 Working Group Letter Ballot#31” |  |
| Abstract | The contribution provides the clarification on Response Indication in SCH and SCEH. |  |
| Purpose | To be discussed and adopted by TGm for 802.16m. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarification on Response Indication in SCH and SCEH

Yeongmoon Son, Hyunjeong Kang, Jungje Son, Rakesh Taori

Samsung Electronics Co., Ltd.

Kiseon Ryu

LG Electronics

- Introduction

There is ‘Response Indication’ in SCH and SCEH. However, there is no description about it. Therefore, we need to clarify it.

- Text to change in D4

-------------------------------  Text Start  ---------------------------------------------------

[Modify the section 16.2.2.1.3.4 on the page 43, line 31 as follows:]

16.2.2.1.3.4 Sleep Control header (SCH)

The Sleep Control header shall be used to convey control signaling related to sleep cycle operation by an ABS or AMS. The specific control provided by SCH is given in Table 660.

When an AMS or an ABS begins to transmit the SCH, they shall set the Response Indication in the SCH to 0 in order to indicate the request.. On the other hand, when the AMS or the ABS receives the SCH with Response Indication = 0, they shall reply with the SCH with Response Indication = 1 as a response to the request. In any case, the SCH sub-type in both the SCHs shall be the same as each other. The AMS and the ABS or the AMS may reply with the SCEH with Response Indication = 1, if possible, in response to the SCH with Response Indication = 0 sent by counterpart.

In the SCH with Response Indication = 1, the ABS can change the remaining parameters as the ABS wishes. On the contrary, the AMS cannot do that (i.e. only acknowledgement to request is allowed)

In the event that the ABS-initiated request and an AMS-initiated request for Sleep Mode entry is being handled concurrently, the ABS-initiated request shall take precedence over the AMS-initiated Request. In this case, even though the AMS receives the SCH with Response Indication = 0 while it is waiting for the SCH with Response Indication =1 from the ABS, the AMS shall stop the remaining procedure of the AMS-initiated request and continue with the ABS-initiated request. The ABS shall ignore an AMS's request if the ABS has already initiated a request.

In case that a SCH is used for the response to the SCH or SCEH, an AMS and an ABS shall set the Response indication in the SCH to 1 (i.e. indicating a response) and the same sub-type upon reception of SCH/SCEH with Response indication = 0 (i.e. indicating a request). If both an ABS initiated request and an AMS-initiated request concurrently occur, the ABS-initiated request takes precedence over the AMS-initiated request.

When the AMS transmits SCH/SCEH with Response Indication = 1 to the ABS, the parameters in the SCH/SCEH shall be the same as the SCH/SCEH which the ABS sent.

[Modify the section 16.2.2.2.5 on the page 53, line 29 as follows:]

16.2.2.2.5 Sleep Control extended header (SCEH)

The Sleep Control extended header shall be used to convey control signaling related to sleep cycle operation by an ABS or AMS. The specific control provided by SCEH is given in Table 669.

When an AMS or an ABS begins to transmit the SCEH, they shall set the Response Indication in the SCEH to 0 in order to indicate the request.. On the other hand, when the AMS or the ABS receives the SCEH with Response Indication = 0, they shall reply with the SCEH with Response Indication = 1 as a response to the request. In any case, the SCEH sub-type in both the SCEHs shall be the same as each other. The AMS and the ABS or the AMS may reply with the SCH with Response Indication = 1, if possible, in response to the SCEH with Response Indication = 0 sent by counterpart.

In the SCEH with Response Indication = 1, the ABS can change the remaining parameters as the ABS wishes. On the contrary, the AMS cannot do that (i.e. only acknowledgement to request is allowed)

In the event that the ABS-initiated request and an AMS-initiated request for Sleep Mode entry is being handled concurrently, the ABS-initiated request shall take precedence over the AMS-initiated Request. In this case, even though the AMS receives the SCEH with Response Indication = 0 while it is waiting for the SCEH with Response Indication =1 from the ABS, the AMS shall stop the remaining procedure of the AMS-initiated request and continue with the ABS-initiated request. The ABS shall ignore an AMS's request if the ABS has already initiated a request.

In the case SCEH is used for the response to the SCH or SCEH, AMS and ABS shall set the Response indication in the SCEH to 1 (i.e. indicating a response) and the same sub-type upon reception of SCH/SCEH with Response indication = 0 (i.e. indicating a request). If both an ABS-initiated request and an AMS-initiated request concurrently occur, ABS initiated request takes precedence over the AMS-initiated request.

When the AMS transmits SCH/SCEH with Response Indication = 1 to the ABS, the parameters in the SCH/SCEH shall be the same as the SCH/SCEH which the ABS sent.

-------------------------------  Text End  ---------------------------------------------------