| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Text cleanup on Idle Mode in Multicarrier (16.2.8.2.10.2) |  |
| Date Submitted | 2010-03-05 |  |
| Source(s) | Yuqin Chen ZTE Corporation | chen.yuqin@zte.com.cn |
| Re: | Call for Letter Ballot #31. |  |
| Abstract | This contribution proposes text cleanup on idle mode in multicarrier. |  |
| Purpose | To discuss and adopt the proposed text in TGm. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Text cleanup on Idle Mode in Multicarrier (16.2.8.2.10.2)

Yuqin Chen

ZTE Corporation

Proposed Text Changes

------------------------------------------------------------Start of the Text------------------------------------------------------

[Modify the following text as following. ]

P241  L33

16.2.8.2.10.2 Idle Mode

A multicarrier supporting AMS in idle state follows the same procedures defined in 15.2.17.

In a multi-carrier system the PGID-Info message is transmitted in all the fully configured carriers. The AAI_PAG-ADV message for an AMS shall be transmitted in only one of the carriers. An idle mode AMS determines the carrier index for monitoring the paging message within AMS's paging-listening interval where its paging message is transmitted using the following equations:

Paging carrier index = DID modulo N  ----Eq (x)

The value of N is the number of carriers per PGID or a set of PGIDs in a ABS that are used for transmitting paging message for idle mode AMS(s). This can be all the carriers or a subset of carriers used by a multi-carrier ABS. The value of N is transmitted in AAI_MC-ADV message. Paging carrier indication bit is used to specify if a carrier is a paging carrier or not. When the Paging carrier indication =1, then the corresponding carrier is a paging carrier. The Paging carrier indication of current carrier is included in the PGID-Info message, and the paging carrier indications of other carriers are included in the AAI_MC-ADV message and different carriers is included in the PGID-Info message and may be included in the AAI_NBR-ADV message or the AAI_MC-ADV message.

When an idle mode AMS moves to a new ABS and determines that the SFH change count of the new ABS is same as the information it received through the AAI_NBR-ADV message, then the AMS may use the paging carrier indication received through AAI_NBR-ADV message if this paging carrier indication is included in the AAI_NBR-ADV message.

The AAI_PAG-ADV message is transmitted in one or more of the frames starting from the second sub-frame in the super-frame.

For an E-MBS AMS the AAI_PAG-ADV message may be transmitted in the same carrier as the dedicated carrier for E-MBS. In this case, the AMS does not use Eq. (x). The paging carrier for such AMS is indicated by DREG-CMD message.

An AMS may perform Location Update process to acquire its preferred carrier for the idle mode support when AMS can’t find the paging carrier. In this case, the new paging carrier for AMS is indicated by RNG-RSP message.

------------------------------------------------------------End of the Text --------------------------------------------------------