| Project IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16>. Title; Text Proposal for MultiBS MIMO operation. Date Submitted; 2010-03-05. Source(s); Shuangfeng Han, Sangwoo Lee, Taeyong Kim, Hokyu Choi, Heewon Kang Samsung Electronics Co., Ltd. Wookbong Lee LG Electronics Luciano Sarperi Fujitsu; shuangf.han@samsung.com wookbong.lee@lge.com luciano.sarperi@uk.fujitsu.com. Re:; Call for comments for Letter Ballot #31. Abstract; The contribution proposes some text change to multiBS MIMO section of 802.16m D4.. Purpose; To be discussed and adopted by TGm for the 802.16m D5.. Notice; This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein.. Release; The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16.. Patent Policy; The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>.. D4 Text Proposal for MultiBS MIMO operation Shuangfeng Han, Sangwoo Lee, Taeyong Kim, Hokyu Choi, Heewon Kang Samsung Electronics - Introduction This contribution includes proposed text to Multi-BS MIMO in the 802.16m D4 document. - Motivation of modifications To perform Downlink Multi-BS MIMO, an AMS is required to know certain parameters, such as temporary BSID of ABSs that take part in downlink multi-BS operation. In the current D4 document, Diversity set NUM, change count and BS index are broadcast in the AAI_DL_IM message. 4 bits are allocated to diversity set NUM, indicating at most 16 adjacent ABS for multi-BS MIMO operation. The current operation procedure for multi-BS joint processing in D4 is as following - ABS broadcast Temp_BSID through AAI_DL_IM Message - AMS measures NIP for each base station and see if it meets triggering mechanism - If it meets triggering mechanism, AMS sends AAI_MULTI_BS_MIMO-REQ with Temp_BSID and NIP value for base stations which has higher NIP than threshold. - ABS indicates which base station to coordinate in case of multi-BS joint processing (Co-MIMO and CL-MD) - ABS allocates feedback with Feedback Polling A-MAP IE - AMS feedback multi-BS MIMO CQI, PMI, CPMI through AAI_MultiBS_MIMO_FBK_Message - For multi-BS joint processing, NIP of each collaborative ABS may not be very high. For example, with 4 collaborative ABSs, NIP of each involved adjacent ABS may be less than 1/3. The most important requirement for collaborative ABSs selection is that all involved adjacent collaborative ABSs shall contribute a significant portion of total interference to AMS, if multiBS coordination is not implemented (which means the sum NIP of adjacent collaborative ABSs shall be high enough). - One problem in the trigger mechanism is that NIP_th_2 does not work when collaborative ABS number is larger than 2. Also, the overhead in AAI_MULTI_BS_MIMO-REQ can be reduced by using different BSID indication method for Single-BS precoding with multi-BS coordination and multi-BS joint MIMO processing. Therefore, we propose following text changes on NIP_th_2 and AAI_MULTI_BS_MIMO-REQ. - Proposed texts: Remedy #1: To modify section 16.5.1.4 -------------------------------------------------- Text Start --------------------------------------------------- 16.5.1.4 Multi-BS MIMO trigger mechanism ABS broadcasts normalized interference power (NIP) thresholds for 2 types of multi-BS MIMO schemes and one common CINR threshold. The NIP is defined as ratio of average interference power (with or without transmitter precoder) from one dominant interference BS to the total interference power plus noise received by AMS. The AMS may accordingly request the preferred multi-BS MIMO scheme. The operation procedure of the multi-BS MIMO trigger follows the steps below. - AMS in normal process of Single-Cell CL MIMO feeds back information for CL MIMO operation. - ABS selects a NIP threshold NIP_th_1 for single-BS precoding with multi-BS coordination, a threshold NIP_th_2 for multi-BS joint MIMO processing and a common CINR_th based on network measurements. NIP_th_1 is threshold for per-ABS NIP of one adjacent ABS and NIP_th_2 is threshold for sum NIP of several adjacent ABSs whose per-ABS NIP is larger than NIP_th_1. Then the ABS broadcasts the NIP thresholds NIP_th_1, NIP_th_2 and CINR_th in AAI_DL_IM message. - AMS computes its average per-ABS NIP of adjacent ABSs listed in AAI-DL-IM message, calculates sum NIP when per-ABS NIP is higher than NIP_th_1, and checks the trigger conditions. If per-ABS NIP≥NIP_th_1 and sum NIP<NIP_th_2 and CINR<CINR_th AMS may request operation with single-BS precoding with multi-BS coordination using the AAI_MULTI_BS_MIMO-REQ message. - AMS feedback its event-driven request to operate single-BS precoding with multi-BS coordination to its serving BS through AAI_MULTI_BS_MIMO-REQ which includes a NIP difference measurement with respect to NIP_th_1. - Once an AMS receives a Feedback Polling A-MAP IE, the procedure outlined in 16.5.1.2.1will be followed. - If per-ABS NIP≥NIP_th_1 and Sum NIP≥NIP_th2 and CINR < CINR_th, AMS may request operation with multi-BS joint MIMO processing using the AAI_MULTI_BS_MIMO-REQ message. - AMS feedback its event-driven request to operate with multi-BS joint MIMO processing to its serving BS through AAI_MULTI_BS_MIMO-REQ which includes a NIP difference measurement with respect to NIP_th_2. Also, an 8 bit bitmap is included indicating which adjacent ABSs are involved in multi-BS joint MIMO processing. This 8 bit bitmap represents strongest 8 adjacent ABS in AAI-DL-IM message based on measurement metric such as RSSI or CINR which is reported by AAI-SCN-REP message. - Once an AMS receives a Feedback Polling A-MAP IE, the procedure outlined in 16.5.1.3.1will be followed. -------------------------------------------------- Text End--------------------------------------------------- Remedy #2: To modify section 16.2.3.39 -------------------------------------------------- Text Start --------------------------------------------------- 16.2.3.39 AAI_MULTI_BS_MIMO-REQ The AAI_MULTI_BS_MIMO-REQ message shall be transmitted by the AMS to report its preference on single BS precoding with Multi-BS MIMO coordination or multi-BS joint MIMO processing. ---ASN.1START AAIMULTIBSMIMOREQ Message::SEQUENCE{ Multi-BS MIMO Request Boolean, NumBS INTEGER (1..8) -- NIPParam shall be transmitted from strongest to weakest NIPList::=SEQUENCE (SIZE (1..NumBS)) OF NIPParam } NIPParam::=SEQUENCE{ Temp_BSID INTEGER (0..15), NIP Value INTEGER (0..15) } --ASN1STOP Table 715—AAI_MULTI_BS_MIMO-REQ message format Name; Value; Usage. Multi-BS MIMO Request; 0: Single-BS precoding with Multi- BS Coordination 1: Multi-BS Joint MIMO Processing. NIP Value; Absolute NIP value encoded as difference to NIP_th_1 when Multi- BS MIMO Request = 0. The value shall be interpreted as unsigned 2bits with units of 0.5 dB, 0b00 is interpreted as NIP_th_1 and 0b11 is interpreted as NIP_th_1+1.5dB or higher. Absolute NIP value encoded as difference to NIP_th_2 when Multi- BS MIMO Request = 1. The value shall be interpreted as unsigned 2bits with units of 0.5 dB, 0b00 is interpreted as NIP_th_2 and 0b11 is interpreted as NIP_th_2+1.5dB or higher.. NumBS; 3 bits; Number of base station which exceeds threshold. Temp_BSID; 4 bits; Temp_BSID is broadcasted through AAI_DL_IM message. Table 715: Parameters for AAI_MULTI_BS_MIMO-REQ message M/O; Attributes / Array of attributes; Size (bits); Value / Note; Conditions. M; Control Message Type; 8; Type of AAI_MULTI_BS_MIMO-REQ; NA. M; multiBsMimoRequest; 1; 0b0: Single-BS precoding with Multi-BS Coordination 0b1: Multi-BS Joint MIMO Processing; NA. O; nipValueForSingleBS; 2; NIP value encoded as difference to NIP_th_1 when multiBsMimoRequest = 0b0. The value shall be interpreted as unsigned 2bits with units of 0.5dB, 0b00 is interpreted as NIP_th1 and 0b11 is interpreted as NIP_th1+1.5dB or higher; Present when multiBsMimoRequest = 0b0. O; numbs; 3; Number of base station which exceeds threshold; Present when multiBsMimoRequest = 0b0. O; tempBsid; 4; This represents Temp_BSID. Temp_BSID is broadcasted through AAI_DL_IM message; Present when multiBsMimoRequest = 0b0. O; bitmapForRequestedAdjABSs; 8; 0b0: ABS is not involved in multiBS MIMO 0b1: ABS is involved in multiBS MIMO Each bit in this bitmap represents one ABS and the i-th bit (i=1,…,8) represents the neighboring ABS (listed in AAI-DL-IM message) with the i-th strongest channel to the AMS. Please refer 16.5.1.4.; Present when multiBsMimoRequest = 0b1. O; nipValueForJoint; 2; Sum NIP value encoded as difference to NIP_th_2 when multiBsMimoRequest = 0b1. The value shall be interpreted as unsigned 2bits with units of 0.5dB, 0b00 is interpreted as NIP_th2 and 0b11 is interpreted as NIP_th2+1.5dB or higher; Present when multiBsMimoRequest = 0b1. ------------------------------------------------- Text End--------------------------------------------------------- Remedy #3: To modify section 16.2.3.40 AAI_MULTI_BS_MIMO-RSP Message -------------------------------------------------- Text Start --------------------------------------------------- 16.2.3.40 AAI_MULTI_BS_MIMO-RSP The AAI_MULTI_BS_MIMO-RSP message shall be transmitted by the ABS to AMS indicating which of the adjacent ABSs listed in previously broadcasted AAI-DL-IMAAI_NBR-Adv message are involved in multiBS MIMO joint processing operation. ---ASN.1START AAIMULTIBSMIMORSP Message::SEQUENCE{ BitmapforinvolvedadjacentABSsinmultiBSMIMO BIT STRING (SIZE (8)) } [Replace Table 716 with the following table] Table 716: AAI_MULTI_BS_MIMO-RSP M/O; Attributes / Array of attributes; Size (bits); Value / Note; Conditions. M; Control Message Type; 8; Type of AAI_MULTI_BS_MIMO-RSP; NA. M; adjAbsBitmapMultiBSMIMO; 8; 0: ABS is not involved in multiBS MIMO 1: ABS is involved in multiBS MIMO Each bit in this bitmap represents one ABS and the i-th bit (i=1,…,8) represents the neighboring ABS (listed in AAI-DL-IM message) with the i-th strongest channel to the AMS. Please refer 16.5.1.4.; NA. -------------------------------------------------- Text End --------------------------------------------------- Remedy #4: Annex P. (Normative) Definition of AAI MAC control messages Annex P.2 MAC Control Message Definitions -------------------------------------------------- Text Start --------------------------------------------------- [Add the following ASN.1 code to Annex P.2] -- Multiple BS MIMO Request AAI-MultiBS-MIMO-REQ ::= SEQUENCE { messageType OCTET STRING (SIZE(1)), multiBsMimoRequest ENUMERATED { singleBsPrecoding (0), jointMimoProcessing (1)}, numBs INTEGER (1..8) OPTIONAL, nIPList NIPList OPTIONAL, bitmapForRequestedAdjABSs BIT STRING (SIZE(8)) OPTIONAL, nipValueForJoint INTEGER (0..3) OPTIONAL } -- NIPParam shall be transmitted from strongest to weakest -- Number of NIPParam report is same as numBs NIPList ::= SEQUENCE { nipParamForBS1 NIPParam, nipParamForBS2 NIPParam OPTIONAL, nipParamForBS3 NIPParam OPTIONAL, nipParamForBS4 NIPParam OPTIONAL, nipParamForBS5 NIPParam OPTIONAL, nipParamForBS6 NIPParam OPTIONAL, nipParamForBS7 NIPParam OPTIONAL, nipParamForBS8 NIPParam OPTIONAL} NIPParam :: SEQUENCE { tempBsid INTEGER (0..15), nipValueForSingleBS INTEGER (0..3)} -- Multiple BS MIMO Response AAI-MultiBS-MIMO-RSP ::= SEQUENCE { messageType OCTET STRING (SIZE(1)), adjAbsBitmapMultiBSMIMO BIT STRING (SIZE(8)) } -------------------------------------------------- Text End --------------------------------------------------- |  |  |
| --- | --- | --- |

|  | Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Title | Text Proposal for MultiBS MIMO operation |  |  |  |  |
|  | Date Submitted | 2010-03-05 |  |  |  |  |
|  | Source(s) | Shuangfeng Han, Sangwoo Lee, Taeyong Kim, Hokyu Choi, Heewon Kang Samsung Electronics Co., Ltd. Wookbong Lee LG Electronics Luciano Sarperi Fujitsu |  |  | shuangf.han@samsung.com wookbong.lee@lge.com luciano.sarperi@uk.fujitsu.com |  |
|  | Re: | Call for comments for Letter Ballot #31 |  |  |  |  |
|  | Abstract | The contribution proposes some text change to multiBS MIMO section of 802.16m D4. |  |  |  |  |
|  | Purpose | To be discussed and adopted by TGm for the 802.16m D5. |  |  |  |  |
|  | Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |  |  |  |
|  | Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |  |  |  |
|  | Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |  |  |  |

D4 Text Proposal for MultiBS MIMO operation

Shuangfeng Han, Sangwoo Lee, Taeyong Kim, Hokyu Choi, Heewon Kang

Samsung Electronics

Wookbong Lee

LG Electronics

Luciano Sarperi

Fujitsu

- Introduction

This contribution includes proposed text to Multi-BS MIMO in the 802.16m D4 document.

- Motivation of modifications

To perform Downlink Multi-BS MIMO, an AMS is required to know certain parameters, such as temporary BSID of ABSs that take part in downlink multi-BS operation. In the current D4 document, Diversity set NUM, change count and BS index are broadcast in the AAI_DL_IM message. 4 bits are allocated to diversity set NUM, indicating at most 16 adjacent ABS for multi-BS MIMO operation.

The current operation procedure for multi-BS joint processing in D4 is as following

- ABS broadcast Temp_BSID through AAI_DL_IM Message

- AMS measures NIP for each base station and see if it meets triggering mechanism

- If it meets triggering mechanism, AMS sends AAI_MULTI_BS_MIMO-REQ with Temp_BSID and NIP value for base stations which has higher NIP than threshold.

- ABS indicates which base station to coordinate in case of multi-BS joint processing (Co-MIMO and CL-MD)

- ABS allocates feedback with Feedback Polling A-MAP IE

- AMS feedback multi-BS MIMO CQI, PMI, CPMI through AAI_MultiBS_MIMO_FBK_Message

- For multi-BS joint processing, NIP of each collaborative ABS may not be very high. For example, with 4 collaborative ABSs, NIP of each involved adjacent ABS may be less than 1/3. The most important requirement for collaborative ABSs selection is that all involved adjacent collaborative ABSs shall contribute a significant portion of total interference to AMS, if multiBS coordination is not implemented (which means the sum NIP of adjacent collaborative ABSs shall be high enough).

- One problem in the trigger mechanism is that NIP_th_2 does not work when collaborative ABS number is larger than 2. Also, the overhead in AAI_MULTI_BS_MIMO-REQ can be reduced by using different BSID indication method for Single-BS precoding with multi-BS coordination and multi-BS joint MIMO processing. Therefore, we propose following text changes on NIP_th_2 and AAI_MULTI_BS_MIMO-REQ.

- Proposed texts:

Remedy #1: To modify section 16.5.1.4

-------------------------------------------------- Text Start ---------------------------------------------------

16.5.1.4 Multi-BS MIMO trigger mechanism

ABS broadcasts normalized interference power (NIP) thresholds for 2 types of multi-BS MIMO schemes and one common CINR threshold. The NIP is defined as ratio of average interference power (with or without transmitter precoder) from one dominant interference BS to the total interference power plus noise received by AMS. The AMS may accordingly request the preferred multi-BS MIMO scheme. The operation procedure of the multi-BS MIMO trigger follows the steps below.

- AMS in normal process of Single-Cell CL MIMO feeds back information for CL MIMO operation.

- ABS selects a NIP threshold NIP_th_1 for single-BS precoding with multi-BS coordination, a threshold NIP_th_2 for multi-BS joint MIMO processing and a common CINR_th based on network measurements. NIP_th_1 is threshold for per-ABS NIP of one adjacent ABS and NIP_th_2 is threshold for sum NIP of several adjacent ABSs whose per-ABS NIP is larger than NIP_th_1. Then the ABS broadcasts the NIP thresholds NIP_th_1, NIP_th_2 and CINR_th in AAI_DL_IM message.

- AMS computes its average per-ABS NIP of adjacent ABSs listed in AAI-DL-IM message, calculates sum NIP when per-ABS NIP is higher than NIP_th_1, and checks the trigger conditions. If per-ABS NIP≥NIP_th_1 and sum NIP<NIP_th_2 and CINR<CINR_th AMS may request operation with single-BS precoding with multi-BS coordination using the AAI_MULTI_BS_MIMO-REQ message.

  - AMS feedback its event-driven request to operate single-BS precoding with multi-BS coordination to its serving BS through AAI_MULTI_BS_MIMO-REQ which includes a NIP difference measurement with respect to NIP_th_1.

  - Once an AMS receives a Feedback Polling A-MAP IE, the procedure outlined in 16.5.1.2.1will be followed.

- If per-ABS NIP≥NIP_th_1 and Sum NIP≥NIP_th2 and CINR < CINR_th, AMS may request operation with multi-BS joint MIMO processing using the AAI_MULTI_BS_MIMO-REQ message.

  - AMS feedback its event-driven request to operate with multi-BS joint MIMO processing to its serving BS through AAI_MULTI_BS_MIMO-REQ which includes a NIP difference measurement with respect to NIP_th_2. Also, an 8 bit bitmap is included indicating which adjacent ABSs are involved in multi-BS joint MIMO processing. This 8 bit bitmap represents strongest 8 adjacent ABS in AAI-DL-IM message based on measurement metric such as RSSI or CINR which is reported by AAI-SCN-REP message.

  - Once an AMS receives a Feedback Polling A-MAP IE, the procedure outlined in 16.5.1.3.1will be followed.

-------------------------------------------------- Text End---------------------------------------------------

Remedy #2: To modify section 16.2.3.39

-------------------------------------------------- Text Start ---------------------------------------------------

16.2.3.39 AAI_MULTI_BS_MIMO-REQ

The AAI_MULTI_BS_MIMO-REQ message shall be transmitted by the AMS to report its preference on single BS precoding with Multi-BS MIMO coordination or multi-BS joint MIMO processing.

---ASN.1START

AAIMULTIBSMIMOREQ Message::SEQUENCE{

Multi-BS MIMO Request    Boolean,

NumBS	INTEGER (1..8)

-- NIPParam shall be transmitted from strongest to weakest

NIPList::=SEQUENCE (SIZE (1..NumBS)) OF NIPParam

}

NIPParam::=SEQUENCE{

Temp_BSID   INTEGER (0..15),

NIP Value   INTEGER (0..15)

}

--ASN1STOP

Table 715—AAI_MULTI_BS_MIMO-REQ message format

| Name | Value | Usage |
| --- | --- | --- |
| Multi-BS MIMO Request |  | 0: Single-BS precoding with Multi- BS Coordination 1: Multi-BS Joint MIMO Processing |
| NIP Value |  | Absolute NIP value encoded as difference to NIP_th_1 when Multi- BS MIMO Request = 0. The value shall be interpreted as unsigned 2bits with units of 0.5 dB, 0b00 is interpreted as NIP_th_1 and 0b11 is interpreted as NIP_th_1+1.5dB or higher. Absolute NIP value encoded as difference to NIP_th_2 when Multi- BS MIMO Request = 1. The value shall be interpreted as unsigned 2bits with units of 0.5 dB, 0b00 is interpreted as NIP_th_2 and 0b11 is interpreted as NIP_th_2+1.5dB or higher. |
| NumBS | 3 bits | Number of base station which exceeds threshold |
| Temp_BSID | 4 bits | Temp_BSID is broadcasted through AAI_DL_IM message |

Table 715: Parameters for AAI_MULTI_BS_MIMO-REQ message

| M/O | Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| M | Control Message Type | 8 | Type of AAI_MULTI_BS_MIMO-REQ | NA |
| M | multiBsMimoRequest | 1 | 0b0: Single-BS precoding with Multi-BS Coordination 0b1: Multi-BS Joint MIMO Processing | NA |
| O | nipValueForSingleBS | 2 | NIP value encoded as difference to NIP_th_1 when multiBsMimoRequest = 0b0. The value shall be interpreted as unsigned 2bits with units of 0.5dB, 0b00 is interpreted as NIP_th1 and 0b11 is interpreted as NIP_th1+1.5dB or higher | Present when multiBsMimoRequest = 0b0 |
| O | numbs | 3 | Number of base station which exceeds threshold | Present when multiBsMimoRequest = 0b0 |
| O | tempBsid | 4 | This represents Temp_BSID. Temp_BSID is broadcasted through AAI_DL_IM message | Present when multiBsMimoRequest = 0b0 |
| O | bitmapForRequestedAdjABSs | 8 | 0b0: ABS is not involved in multiBS MIMO 0b1: ABS is involved in multiBS MIMO Each bit in this bitmap represents one ABS and the i-th bit (i=1,…,8) represents the neighboring ABS (listed in AAI-DL-IM message) with the i-th strongest channel to the AMS. Please refer 16.5.1.4. | Present when multiBsMimoRequest = 0b1 |
| O | nipValueForJoint | 2 | Sum NIP value encoded as difference to NIP_th_2 when multiBsMimoRequest = 0b1. The value shall be interpreted as unsigned 2bits with units of 0.5dB, 0b00 is interpreted as NIP_th2 and 0b11 is interpreted as NIP_th2+1.5dB or higher | Present when multiBsMimoRequest = 0b1 |

------------------------------------------------- Text End---------------------------------------------------------

Remedy #3: To modify section 16.2.3.40 AAI_MULTI_BS_MIMO-RSP Message

-------------------------------------------------- Text Start ---------------------------------------------------

16.2.3.40 AAI_MULTI_BS_MIMO-RSP

The AAI_MULTI_BS_MIMO-RSP message shall be transmitted by the ABS to AMS indicating which of the adjacent ABSs listed in previously broadcasted AAI-DL-IMAAI_NBR-Adv message are involved in multiBS MIMO joint processing operation.

---ASN.1START

AAIMULTIBSMIMORSP Message::SEQUENCE{

BitmapforinvolvedadjacentABSsinmultiBSMIMO BIT STRING (SIZE (8))

}

[Replace Table 716 with the following table]

Table 716: AAI_MULTI_BS_MIMO-RSP

| M/O | Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| M | Control Message Type | 8 | Type of AAI_MULTI_BS_MIMO-RSP | NA |
| M | adjAbsBitmapMultiBSMIMO | 8 | 0: ABS is not involved in multiBS MIMO 1: ABS is involved in multiBS MIMO Each bit in this bitmap represents one ABS and the i-th bit (i=1,…,8) represents the neighboring ABS (listed in AAI-DL-IM message) with the i-th strongest channel to the AMS. Please refer 16.5.1.4. | NA |

-------------------------------------------------- Text End ---------------------------------------------------

Remedy #4:

Annex P.

(Normative)

Definition of AAI MAC control messages

Annex P.2 MAC Control Message Definitions

-------------------------------------------------- Text Start ---------------------------------------------------

[Add the following ASN.1 code to Annex P.2]

-- Multiple BS MIMO Request

AAI-MultiBS-MIMO-REQ ::= SEQUENCE {

messageType                    OCTET STRING (SIZE(1)),

multiBsMimoRequest             ENUMERATED {

singleBsPrecoding   (0),

jointMimoProcessing (1)},

numBs                          INTEGER (1..8) OPTIONAL,

nIPList                        NIPList OPTIONAL,

bitmapForRequestedAdjABSs  BIT STRING (SIZE(8)) OPTIONAL,

nipValueForJoint             INTEGER (0..3) OPTIONAL

}

-- NIPParam shall be transmitted from strongest to weakest

-- Number of NIPParam report is same as numBs

NIPList ::= SEQUENCE {

nipParamForBS1                      NIPParam,

nipParamForBS2                      NIPParam OPTIONAL,

nipParamForBS3                      NIPParam OPTIONAL,

nipParamForBS4                      NIPParam OPTIONAL,

nipParamForBS5                      NIPParam OPTIONAL,

nipParamForBS6                      NIPParam OPTIONAL,

nipParamForBS7                      NIPParam OPTIONAL,

nipParamForBS8                      NIPParam OPTIONAL}

NIPParam :: SEQUENCE {

tempBsid                       INTEGER (0..15),

nipValueForSingleBS          INTEGER (0..3)}

-- Multiple BS MIMO Response

AAI-MultiBS-MIMO-RSP ::= SEQUENCE {

messageType                    OCTET STRING (SIZE(1)),

adjAbsBitmapMultiBSMIMO      BIT STRING (SIZE(8))

}

-------------------------------------------------- Text End ---------------------------------------------------