| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Inclusion of the essential parameters for MC operation in the AAI_MC-ADV message (section 16.2.8.2.3.1.1) |  |
| Date Submitted | 2010-03-05 |  |
| Source(s) | Kiseon Ryu, Eunjong Lee, Youngsoo Yuk LG Electronics, Inc. | +82-31-450-1856 inuk.jung@lge.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Call for comments |  |
| Abstract | Text proposal for Multi-carrier section 16.2.8.2.3.1.1 |  |
| Purpose | For member’s review and adoption into P802.16m_D4 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Inclusion of the essential parameters for MC operation in the AAI_MC-ADV message (section 16.2.8.2.3.1.1)

Kiseon Ryu, Eunjong Lee, Youngsoo Yuk

LG Electronics, Inc.

- Introduction

AAI_MC-ADV should contain the essential parameters (e.g. periodic ranging parameters) of the available carriers for the AMS to activate the secondary carrier or change the primary carrier properly.

- References

[1] IEEE P802.16m/D4, “Air Interface for Broadband Wireless Access Systems - Advanced Air Interface”

- Proposed Text Changes

Remedy #1

On the 80216m_D4, page 232, line 44, replace current text in 16.2.8.2.3.1.1 with the proposed text as follows:

--------------------------------------------------------Text Start -----------------------------------------------------

16.2.8.2.3.1.1 AAI_MC-ADV (multicarrier Advertisement) Message

The MC ABS shall periodically broadcast AAI_MC-ADV message for the reception by all AMSs.

Table 748—AAI_MC-ADV MAC Control Message Format

| Field | Size (bit) | Descrpition |
| --- | --- | --- |
| MAC Control Message Type | 8 |  |
| Multi-carrier configuration change count | 4 | Incremented by 1 upon each update. |
| Serving BS Carrier Number | 3 |  |
| Serving BS Uniformity Flag | 1 | 0: All Carriers supported by serving ABS have the same Protocol Version, SFH_Info 1: otherwise |
| Physical Carrier Index of current carrier | 6 | The carrier that ABS broadcast this message; the physical carrier index refers to AAI_Global-Config message |
| MAC Protocol version | 8 | Consistent with REV.2 definition, with new MAC protocol version 9 defined for 16m |
| For(i=1;i<=Serving BS Carrier Number-1; i++){ |  |  |
| Physical carrier index | 6 | //physical carrier index in AAI_Global-Config message |
| SA-Preamble Index | 10 |  |
| if (Serving BS Uniformity Flag==1) { |  |  |
| MAC Protocol Version | 8 | Consistent with REV.2 definition, with new MAC protocol version 9 defined for 16m |
| SFH_encoding_format | 1 | 0b0: full Subpkt information 0b1: delta encoding, w.r.t. the current carrier |
| Control_bitmap | 4 | Each bit maps to one SFH subpacket or extended broadcast information |
| If (SFH_encoding_format==00) { |  |  |
| If (Control_bitmap[0]==1){ |  |  |
| SFH Subpkt 1 | 88 |  |
| } |  |  |
| If (Control_bitmap[1]==1){ |  |  |
| SFH Subpkt 2 | 88 |  |
| If (Control_bitmap[2]==1){ |  |  |
| SFH Subpkt 3 | 88 |  |
| } |  |  |
| If (SFH_encoding_format==01) { |  |  |
| Delta information | variables | Delta encoding, w.r.t. the current carrier |
| } |  |  |
| If (Control_bitmap[3]==1){ |  |  |
| Allocation periodicity of ranging channel for synchronized AMSs | 2 | Indicates the periodicity of ranging channel allocation according to the Table 900. |
| Start code information of ranging channel for synchronized AMSs | 4 | Indicates the subframe offset (OSF) of ranging channel allocation related to the Table 900. The range of values is 0≤OSF≤15 |
| Ranging preamble code partition information | 4 | Indicates the ks which is the parameter controlling the start root index of ranging preamble codes (rs0). rs0 = 6×ks+1 The range of values is 0≤ks≤15 |
| } |  |  |
| } |  |  |
| } |  |  |

--------------------------------------------------------Text End -----------------------------------------------------