| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clean-up Text for HF-A-MAP (Section 16.3.6.3) |  |
| Date Submitted | 2010-03-05 |  |
| Source(s) | Hyunkyu Yu, Jeongho Park Samsung Electronics Co., Ltd. | hk.yu@samsung.com |
| Re: | IEEE 802.16-10/0011, “IEEE 802.16 Working Group Letter Ballot#31” Target topic: “IEEE P802.16m/D4 section 16.3.6.5”. |  |
| Abstract | The contribution proposes physical processing for HF-A-MAP. |  |
| Purpose | To be discussed and adopted by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clean-up Text for HF-A-MAP (Section 16.3.6.3)

Hyunkyu Yu, Jeongho Park

Samsung Electronics

- Introduction

This contribution clarifies unclear and undefined procedures in the physical processing block for HARQ feedback A-MAP.

- Bit value mapping

- Scrambling method

- Overall structure

- Resource index

- Proposed Text Changes

[Remedy#1: Change the following text, at line 30 in page 441, section 16.3.6.3.2.2]

------------------------------------------------Start proposed text----------------------------------------------------------------

16.3.6.3.2.2 HARQ Feedback A-MAP

…

Figure 521 shows the construction procedure of HF-A-MAP symbols from HF-A-MAP-IE for an HF-A-MAP group. An HF-A-MAP region has NHF-A-MAP/4 HF-A-MAP groups and each group of HF-A-MAP consists of 4 HF-A-MAPs numbered as 4c, 4c+1, 4c+2, 4c+3 where NHF-A-MAP is the total number of HF-A-MAPs per HF-A-MAP region, and c (0≤c<NHF-A-MAP/4) is the HF-A-MAP group index. The HF-A-MAP resource index is then denoted as 4∙c+k , where 0≤k<4.

Each HF-A-MAP IE carries 1 bit information. Firstly, 0 is mapped to +1 and 1 is mapped to -1, and it is repeated NRep, HF-A-MAP times, where NRep, HF-A-MAP is 8. Then, the NRep, HF-A-MAP LSBs of the STID or the RAID for the associated AMS are applied to the Rrepeated HF-A-MAP IE bits by bitwise XOR operation. are scrambled by the NRep, HF-A-MAP LSBs of the STID of the associated AMS. Depending on the channel conditions, power scaling can be applied to each scrambled sequence. Before MIMO encoding, each scrambled sequence is mapped to either real part or imaginary part in the signal constellation and multiplexed with other scrambled sequence, if exist.

After scrambling, power scaling by sqrt(Pi) (0≤i<NHF-A-MAP) is applied to the ith HF-A-MAP where sqrt(Pi) is the value determined by the management entity to satisfy the link performance. Note that if the jth HF-A-MAP is not allocated to any AMSs, Pj is set to 0.

Each power-scaled sequence is mapped to either real part or imaginary part in the signal constellation and multiplexed with other sequence, if exist. As shown in Figure 521, HF-A-MAP IE 4c occupies the real part of the first symbol in each tone pair, and HF-A-MAP IE 4c+1 occupies the imaginary part of the first symbol in each tone pair. In the same way, HF-A-MAP IE 4c+2 occupies the real part of the second symbol in each tone pair, and HF-A-MAP IE 4c+3 occupies the imaginary part of the second symbol in each tone pair.

Figure 521 shows a cluster of HF-A-MAP channels, which consists of 4 HF-A-MAP channels numbered as 4c, 4c+1, 4c+2, 4c+3 where c is the HF-A-MAP cluster index in a HF-A-MAP region. Channel 4c in the cluster occupies the real part of the first symbol in each tone pair before the SFBC encoder. Channel 4c+1 in the cluster occupies the imaginary part of the first symbol in each tone pair before the SFBC encoder. Channel 4c+2 in the cluster occupies the real part of the second symbol in each tone pair before the SFBC encoder. Channel 4c+3 in the cluster occupies the imaginary part of the second symbol in each tone pair before the SFBC encoder.

For each Tx antenna, symbols at the output of MIMO encoder, denoted by SHF[0] to SHF[LHF-1], are directly mapped to tone-pairs from RMP[0] to RMP[LHF/2 -1] denoted by RMP[p] = {SHF[2q], SHF[2q+1]}, where RMP refers to the renumbered A-MAP tone-pairs described in 16.3.5.3.4 and LHF is the number of tones required to transmit the entire HARQ feedback one HF-A-MAP region. p and q are integer values from 0 to LHF/2-1. Clusters of the HF-A-MAP are indexed sequentially from index 0 within an HF-A-MAP region in the mapping process.

There is one HF-A-MAP region in each DL AAI subframe. Within each HF-A-MAP region, the index for HF-A-MAP channel is calculated as follows.

Within each HF-A-MAP region, the ABS allocates the HF-A-MAP to the AMS using following rules.

For the deallocation of a persistent allocation, the HF-A-MAP resource index is specified in the HFA the of UL Persistent Allocation A-MAP IE.

For group resource allocation, the HF-A-MAP resource index for the lth AMS in the GRA allocation is , where istart is the ACK Channel HFA Offset in the UL group resource allocation A-MAP IE for UL allocation, NHF-A-MAP is the total number of HF-A-MAP configured per HF-A-MAP region, and NGRA is the User Bit Map Size user bitmap size in the UL group resource allocation A-MAP IE for UL allocation.

…

For resource allocation using the UL Basic Assignment A-MAP IE, UL Subband Assignment A-MAP IE, CDMA Allocation A-MAP IE, Feedback Polling A-MAP IE and the UL Persistent Allocation A-MAP IE, the HF-A-MAP resource index is (M(j)+n) mod NHF-A-MAP, where j is HF-A-MAP Index Parameter in the Non-user specific A-MAP IE, n is a 3 bit HFA value in each assignment A-MAP IE, NHF-A-MAP is the total number of HF-A-MAP channels configured per HF-A-MAP region. M(j) is STID for the UL Basic Assignment A-MAP IE and UL Subband Assignment A-MAP IE and RA-ID for CDMA Allocation A-MAP IE when j=0 and M(j) is the lowest LRU index of the corresponding UL transmission when j=1. For the UL persistent allocation A-MAP IE and the Feedback Polling A-MAP IE, M(j) is always STID regardless of value j.

-------------------------------------------------End proposed text----------------------------------------------------------------