| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text to UL Persistent Allocation and UL Group Resource Allocation (Section 16.3.6.5) |  |
| Date Submitted | 2010-03-05 |  |
| Source(s) | Hyunkyu Yu, Mihyun Lee Samsung Electronics Co., Ltd. | hk.yu@samsung.com |
| Re: | IEEE 802.16-10/0011, “IEEE 802.16 Working Group Letter Ballot#31” Target topic: “IEEE P802.16m/D4 section 16.3.6.5”. |  |
| Abstract | This contribution proposes to support default TTI for group resource allocation and persistent allocation in case of D<U. |  |
| Purpose | To be discussed and adopted by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text to UL Persistent Allocation and UL Group Resource Allocation (Section 16.3.6.5)

Hyunkyu Yu, Mihyun Lee

Samsung Electronics

- Introduction

In D4, only long TTI transmission is allowed in case of D<U, where D is the number of DL subframes and U is the number of UL subframes. This contribution proposes to support default TTI as well for group resource allocation and persistent allocation.

The followings are the rationale for proposal.

- GRA and PA are mainly designed to support VoIP which is delay-sensitive service. If we only support long TTI transmission, the number of allowable retransmissions will be reduced, and it may decrease the outage performance.

- There’re many cases where D<U

. 5/10/20MHz - 3:5, 3:4

. 8.75MHz - 3:4 2:4

. 7MHz - 2:3

- When ranging channel (one subband) is present in a UL subframe, the resource in the corresponding subband cannot be allocated during whole UL subframes in a frame if we only support long TTI transmission.

- Proposed text with markups

Blue/Underline: Text Added

Red/Strikeout: Text Deleted

- References

[1] IEEE P802.16m/D4, “P802.16m DRAFT Amendment to IEEE Standard for Local and metropolitan area networks Part 16: Air Interface for Broadband Wireless Access Systems”

- Proposed Text Changes

[Remedy#1: Modify the text, line 16 in page 509, section 16.3.6.5.2.4.9]

--------------------------------------------------Start Proposed Text-------------------------------------------------------------

- 16.3.6.5.2.4.9 UL PA A-MAP IE

The UL persistent allocation A-MAP IE is specified in Table 834.

Table 834 - UL Persistent Allocation A-MAP IE*

| Syntax | Size in bits | Description/Notes |
| --- | --- | --- |
| UL Persistent Allocation A-MAP_IE(){ | - | - |
| A-MAP IE Type | 4 | UL Persistent Allocation A-MAP IE |
| … | … | … |
| Initial_ACID | 4 | Initial value of HARQ channel identifier for implicit cycling of HARQ channel identifiers. |
| if (D<U){ |  | D: number of DL AAI subframes, U: number of UL AAI subframes |
| Allocation Relevance | 1 | 0b0: Allocation in the first UL subframe relevant to an A-MAP region 0b1: Allocation in the second UL subframe relevant to an A-MAP region |
| } else { |  |  |
| Reserved | 1 | Reserved bits |
| } |  |  |
| Reserved | 21 | Reserved bits |
| } | - | - |
| } | - | - |

---------------------------------------------------End Proposed Text-------------------------------------------------------------

[Remedy#2: Modify the text, line 65 in page 509, section 16.3.6.5.2.4.10]

--------------------------------------------------Start Proposed Text-------------------------------------------------------------

Table 835 – Group Resource Allocation A-MAP IE*

| Syntax | Size in bits | Description/Notes |
| --- | --- | --- |
| Group_Resource_Allocation A-MAP_IE(){ | - | - |
| A-MAP IE Type | 4 | UL Persistent Allocation A-MAP IE |
| if (UL allocation && D<U){ |  | If group corresponds to UL allocations and the number of DL AAI subframes (D) is less than the number of UL AAI subframes (U), |
| Allocation Relevance | 1 | 0b0: Allocation in the first UL subframe relevant to an A-MAP region 0b1: Allocation in the second UL subframe relevant to an A-MAP region |
| } |  |  |
| User Bitmap | Variable | … |
| … | … | … |
| } | - | - |

---------------------------------------------------End Proposed Text-------------------------------------------------------------

[Remedy#3: Modify the text, in page 442, line 1, section 16.3.6.3.2.2]

--------------------------------------------------Start Proposed Text-------------------------------------------------------------

There is one HF-A-MAP region in each DL AAI subframe. Within each HF-A-MAP region, the index for HF-A-MAP channel is calculated as follows. In FDD, there is one HF-A-MAP region in each DL AAI subframe. In TDD, with notation in Table 764 in 16.2.14.2.2.2.2, for HF-A-MAP region in DL AAI subframe l, the associated UL subframes can start from a set of UL AAI subframe indices denoted by M ={m0, m1, ..., mK-1}, where m0 < m1 < ... < mK-1. The number of HF-A-MAP regions in an DL AAI subframe is equal to K according to the number of the associated UL subframes, i.e. the size of set M.

For the deallocation of …

---------------------------------------------------End Proposed Text-------------------------------------------------------------