| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text for Multicarrier Operation for Femtocell ABS (16.2.8.2.3.1.1, 16.4) |  |
| Date Submitted | 2010-03-05 |  |
| Source(s) | Motoki Morita, Yasuhiko Matsunaga, Linghang Fan, Nader Zein, Hassan Al-Kanani, Tetsu Ikeda NEC | m-morita@bx.jp.nec.com |
| Re: | LB comment to 802.16m Amendment Working Document D4 |  |
| Abstract | This contribution provides a scheme allowing multi-carrier operation for Femtocell ABS. |  |
| Purpose | For discussion and approval by IEEE 802.16m TG |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text for Multicarrier Operation for Femtocell ABS (16.2.8.2.3.1.1, 16.4)

Motoki Morita, Yasuhiko Matsunaga, Linghang Fan, Nader Zein, Hassan Al-Kanani, Tetsu Ikeda

NEC

# Introduction

This contribution proposes a scheme that allows multicarrier operation for Femtocell ABS without increasing interference to macro/micro cells.

# Multicarrier Operation

## Multicarrier Operation for Femtocell ABS

Multicarrier operation may be supported by Femtocell ABS as depicted in Fig.1. All operational principles for multicarrier operation outlined in section 16.2.8 of D4 are applied to the multicarrier operation for Femtocell. In addition, the following principles shall also be applied to multicarrier operation for Femtocell in order to mitigate interference.

Fig.1  Illustration of the concept of multicarrier operation for Femtocell ABS

## Interference in Multicarrier Operation for Femtocells

If Femtocell ABS happens to select a secondary carrier which is the same carrier as the one used by neighbor ABS, the interference from Femtocell to neighbor cells may subsequently increase. This may occasionally happen when  multicarrier operation scheme is enabled for Femtocell since  secondary carrier management does not consider the interference impact to the neighbor cells.

A scenario in which interference will increase in multicarrier operation for Femtocell ABS is described in Fig.2. RF1 is assigned to a MS connected to the Femtocell ABS as a primary carrier. RF2 is to be assigned as a secondary carrier for the MS connected to the Femtocell ABS meanwhile RF2 is also assigned to a MS connected to the macro ABS (macro MS) as a primary carrier.  So the interference to the macro MS will increase and performance of the macro cell may subsequently be degraded.

Fig.2 Illustration of the scenario that interference increase in multicarrier operation for Femtocell ABS.

## Proposed Scheme for Interference Mitigation in Multicarrier Operation

In case of multicarrier operation for Femtocell ABS, the Femtocell ABS should consider interference avoidance or mitigation when assigning the secondary carrier.

In assigning the secondary carrier, the Femtocell ABS should decide activating or deactivating of secondary carrier(s) based on  factors such as load condition of each carrier of the neighbor ABS.

We propose to use DL and UL available resources for each carrier of the neighbor ABS as the co-channel interference indicator for the Femtocell ABS. The ABS should transmit the DL and UL available resource condition for each carrier (“Available Radio Resource DL” and “Available Radio Resource UL”) in AAI_MC-ADV messages as shown in Fig.3. “Available Radio Resource DL” and “Available Radio Resource UL” are represented by the percentage of non assigned DL resources to the total usable DL radio resources. Femtocell ABSs monitor the load condition at the initial setting and then monitor it periodically. If there is limited DL or UL available radio resource for a carrier from the neighbor ABS, the Femtocell ABSs should avoid using this carrier as the secondary carrier.

The ABS transmits the DL and UL load

condition for each carrier in AAI_MC

-

ADV

messages to

Femtocell

ABSs

.

Load condition

The

Femtocell

ABS receives the

ABS

’

s

load condition and avoids using the

RF2 as the secondary carrier

RF1

RF2

Primary

carrier

Not to be used as

secondary carrier

Carrier assignment for a MS in

Femtocell

The ABS transmits the DL and UL load

condition for each carrier in AAI_MC

-

ADV

messages to

Femtocell

ABSs

.

Load condition

The

Femtocell

ABS receives the

ABS

’

s

load condition and avoids using the

RF2 as the secondary carrier

RF1

RF2

Primary

carrier

Not to be used as

secondary carrier

Carrier assignment for a MS in

Femtocell

Fig.3 Proposed Scheme for interference mitigation in multi-carrier operation

# Proposed Text

-------------------------------------------------------Start of Text 1-------------------------------------------------------

[Modify the Table 748 as follows]

16.2.8.2.3.1.1 AAI_MC-ADV (multicarrier Advertisement) Message

Table 748   AAI_MC-ADV MAC Control Message Format

| Field | Size (bit) | Description |
| --- | --- | --- |
| MAC Control Message Type | 8 |  |
| Multi-carrier configuration change count | 4 | Incremented by 1 upon each update. |
| Serving BS Carrier Number | 3 |  |
| Serving BS Uniformity Flag | 1 | 0: All Carriers supported by serving ABS have the same Protocol Version, SFH_Info 1: otherwise |
| Physical Carrier Index of current carrier | 6 | The carrier that ABS broadcast this message; the physical carrier index refers to AAI_Global-Config message |
| MAC Protocol version | 8 | Consistent with REV.2 definition, with new MAC protocol version 9 defined for 16m |
| For(i=1;i<=Serving BS Carrier Number-1; i++){ |  |  |
| Physical carrier index | 6 | //physical carrier index in AAI_Global-Config message |
| SA-Preamble Index | 10 |  |
| Available Radio Resource DL | 8 | 8-bit unsigned integer 0x00 = 0% 0x01 = 1% … 0x64 = 100% All other values are Reserved. Available Radio Resource indicator DL SHALL indicate the average ratios of non assigned DL resources to the total usable DL radio resources. |
| Available Radio Resource UL | 8 | 8-bit unsigned integer 0x00 = 0% 0x01 = 1% … 0x64 = 100% All other values are Reserved. Available Radio Resource indicator DL SHALL indicate the average ratios of non assigned UL resources to the total usable UL radio resources. |
| if (Serving BS Uniformity Flag==1) { |  |  |
| MAC Protocol Version | 8 | Consistent with REV.2 definition, with new MAC protocol version 9 defined for 16m |
| SFH_encoding_format | 1 | 0b0: full Subpkt information 0b1: delta encoding, w.r.t. the current carrier |
| Control_bitmap | 4 | Each bit maps to one SFH subpacket or extended broadcast information |
| If (SFH_encoding_format==00) { |  |  |
| If (Control_bitmap[0]==1){ |  |  |
| SFH Subpkt 1 | 88 |  |
| } |  |  |
| If (Control_bitmap[1]==1){ |  |  |
| SFH Subpkt 2 | 88 |  |
| } |  |  |
| If (Control_bitmap[2]==1) { |  |  |
| SFH Subpkt 3 | 88 |  |
| } |  |  |
| } |  |  |
| If (SFH_encoding_format==01) { |  |  |
| Delta information | variables | Delta encoding, w.r.t. the current carrier |
| } |  |  |
| } |  |  |
| } |  |  |

-------------------------------------------------------Start of Text 2-------------------------------------------------------

[Insert text in subclause 16.4 as follows]

16.4 Support for Femto ABS

16.4.x Multicarrier Operation

Multicarrier operation may be supported by Femtocell ABS. All operational principles for multicarrier operation set out in section 16.2.8 are applied to the multicarrier operation for Femtocell ABS. In addition, the following principles shall also be adopted  for multicarrier operation for Femtocell in order to mitigate interference.

In assigning the secondary carrier, the Femtocell ABS should decide activating or deactivating the secondary carrier(s) based on  factors such as load condition of each carrier of the neighbor ABS.

The DL and UL available radio resource for each carrier shall be utilized as the load condition. The load condition for each carrier should be included in the AAI_MC-ADV MAC control message. The ABS should transmit the available radio resource for each carrier (“Available Radio Resource DL” and “Available Radio Resource UL”)in AAI_MC-ADV messages. “Available Radio Resource DL” and “Available Radio Resource UL” are represented by the percentage of non assigned DL resources to the total usable DL radio resources. Femtocell ABSs monitor the load condition at the initial setting and then monitor it periodically. If there is limited DL or UL available radio resource for a carrier from  a neighbor ABS,  the Femtocell ABSs should avoid using this carrier as the secondary carrier.

--------------------------------------------------------End of the Text-------------------------------------------------------