| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed changes to MCEH for the efficiency of UL bandwidth allocation |  |
| Date Submitted | 2010-03-06 |  |
| Source(s) | Yongseouk Choi, Namsuk Lee, Jisoo Park, Sookjin Lee ETRI 138 Gajeongno, Yuseong-gu, Daejeon, 305-700, KOREA | +82-42-860-5677 [choiys,namsuk, jsp, sjlee] @etri.re.kr *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE 802.16 Working Group Letter Ballot 31 Section 16.2.2.2.2 MAC Control extended header(MCEH) |  |
| Abstract | This contribution proposes the text for unsolicited UL bandwidth allocation indicator in MCEH |  |
| Purpose | Discussion and adoption by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed changes to MCEH for the efficiency of UL bandwidth allocation

Yongseouk Choi, Namsuk Lee, Jisoo Park, Sookjin Lee

ETRI

- Introduction

The MCEH shall be used when all control messages are transferred between AMS and ABS. In cases that the polling bit in the MCEH is set, the receiver should transmit an ACK message. Both ABS and AMS can know the control message, which may be transferred the next time in the signaling procedure.

This contribution proposes to insert an UBIC bit and an UBIA bit into the MCEH, then AMS and ABS can skip the BR procedure after checking those bits in the MCEH.

The ABS will assign UL bandwidth to the AMS for the next control message in an unsolicited manner and notify the AMS of the UL bandwidth assignment using the UBIC or UBIA in the MCEH. Then AMS can check with the UBIC or UBIA in the MCEH as to whether the UL bandwidth will be assigned or not.

If the UL bandwidth is assigned, the AMS can transmit a control message without a BR procedure. The transferring of control message after skipping BR procedure can reduce signaling latency.

Using unsolicited bandwidth indicator in RNG-RSP message, the UL assignment without a bandwidth request from AMS can be accomplished. But this proposal has the following advantages comparing unsolicited bandwidth indicator in RNG-RSP.

- Can assign UL bandwidth for all ACK message by using the polling bit and UBIA bit in MCEH

- This proposal can cover all control messages, but unsolicited bandwidth indicator in RNG-RSP message covers only network entry messages.

- More precisely can control the predefined time duration for possible bandwidth allocation without performing any bandwidth request, because the time interval for UL bandwidth allocation using MCEH is shorter than using RNG-RSP

The addition of two bits (UBIC and UBIA) will use the reserved bits for byte alignment, so this proposal does not require more extra bits than the original MCEH.

- Proposed Text

Modify the Table 666 – MCEH Format as follows

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| MCEH () { |  |  |
| EC | 1 | Encryption Control indicator 0 = Payload is not encrypted 1 = Payload is encrypted |
| Control Connection Channel ID (CCC ID) | 1 | Channel ID to identify separate fragmentation / reassembly state machines 0: channel 1 1: channel 2 |
| UBIC | 1 | Unsolicited UL bandwidth allocation indicator for control message 0 = ABS will not allocate UL bandwidth for transmission of control message 1 = ABS will allocate UL bandwidth for transmission of control message |
| SN Indicator | 1 | 0 = no FC and sequence number 1= FC and sequence number are followed |
| If (SN Indicator = 0) { |  |  |
| Reserved | 54 | For byte alignment |
| else{ |  |  |
| Polling | 1 | 0 = no acknowledgement required 1 = acknowledge required upon receiving the MAC message |
| UBIA | 1 | Unsolicited UL bandwidth allocation indicator for ACK message 0 = ABS will not allocate UL bandwidth for transmission of ACK message 1 = ABS will allocate UL bandwidth for transmission of ACK message |
| FC | 2 | Fragmentation control (see Table 665) |
| SN | 8 | Payload sequence number |
| Reserved | 2 | For byte alignment |
| } |  |  |
| } |  |  |

- Reference

[1] IEEE P802.16m/D3, “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks Part 16: Air Interface for Broadband Wireless Access Systems”