| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text for FDM-Based UL PUSC Zone Support (16.3.8) |  |
| Date Submitted | 2010-03-05 |  |
| Source(s) | Jinsoo Choi, Jinyoung Chun, HanGyu Cho, Jin Sam Kwak LG Electronics Amir Rubin, Jong-Kae (JK) Fwu Intel | {js.choi, jiny.chun, hg.cho, jinsam.kwak }@lge.com {amir.rubin, jong-kae.fwu}@intel.com |
| Re: | Comments on P802.16m/D4 for LB #31 |  |
| Abstract | The contribution provides the proposed text for FDM-based UL PUSC zone support to be included in IEEE 802.16m amendment document. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text for FDM-Based UL PUSC Zone Support (16.3.8)

Jinsoo Choi, Jinyoung Chun, HanGyu Cho, Jin Sam Kwak

LG Electronics

Amir Rubin, Jong-Kae (JK) Fwu

Intel

- Introduction

This contribution provides the proposed text for FDM-based UL PUSC zone support. When subchannel rotation is applied to WirelessMAN-OFDMA system, it is difficult to support dynamic load balancing with current FDM-based UL PUSC zone structure because the total number of subchannel which is used for subchannel rotation must be depending on the subchannel information by UL subchannel bitmap transmitted in UCD.

In this contribution, we propose a method to support load balancing in frame-level based on usage of safety zone. Details are addressed in [1].

- References

[1] IEEE S802.16m-10/0256, “Proposed Text for FDM-based UL PUSC Zone Support (16.3.8)”

[2] IEEE C802.16m-07/0034r2, “802.16m System Requirements”

[3] IEEE C802.16m/D4, “Advanced Air Interface (Draft)”

[4] IEEE C802.16m-08/043, “Style guide for writing the IEEE 802.16m amendment”

- Text proposal for inclusion in the 802.16m amendment

Modify the texts in 16.3.8.3.5.3 section of page 600 in P802.16m/D4 as following

-------------------------------------------------------  Text Start  --------------------------------------------------------------

16.3.8.5.3 Subchannelization for FDM based UL PUSC Zone Support

When supporting FDM based UL PUSC zone, UL subchannelization shall conform the following rules:

1) For the WirelessMAN-OFDMA system bandwidth, all usable subcarriers given in Table 887, Table 888, and Table 889 are divided into PUSC tiles.

2) UL PUSC subchannelization is performed as described in 8.4.6.2.2

3) All PUSC tiles of specified subchannels from step 2 are extended in time domain from 3 OFDM symbols to Nsym OFDM symbols, where Nsym is dependent of AAI subframe type.

4) Based on specified subchannels of step 23 with symbol extension tiles of step 3, DRUs for Advanced Air Interface are made up. When subchannel rotation defined in 8.4.6.2.6 is applied to WirelessMAN-OFDMA system, DRUs for Advanced Air Interface system shall be allocated within the subchannels indicated as safety zone by table 378 of 8.4.5.4.2 section i.e. “PAPR Reduction/Safety Zone/Sounding Zone Allocation IE format” or within subchannels indicated as not used by TLV 157 of the UCD i.e. “UL allocated subchannels bitmap” and these subchannels shall not be allocated for the WirelessMAN-OFDMA system. Available subchannels for WirelessMAN-OFDMA MSs shall start with lowest PUSC subchannel index and be logically contiguous within the subchannels.

5) Repeat step 3 and step 4 for remained OFDMA symbols of every uplink AAI subframe.

6) Renumber the DLRU index in reverse order of PUSC subchannel index.

-------------------------------------------------------  Text End  ----------------------------------------------------------------