| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | The MAC PDU header format for E-MBS service data (16.2.2.1) |  |
| Date Submitted | 2010-03-15 |  |
| Source(s) | Kaiying Lv, Li Nan, Bo Sun, Yang Liu ZTE Corporation Kaushik Josiam, Jaejeong Shim Samsung | E-mail: {lv.kaiying}@zte.com.cn |
| Re: | Call for LB #30b on “ P802.16m/D4”: Target topic: “16.2.2.1” |  |
| Abstract | This contribution proposes the MAC header format for E-MBS data (16.2.2.1) |  |
| Purpose | To be discussed and adopted |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

The MAC PDU header format for E-MBS service data (16.2.2.1)

Kaiying Lv, Nan Li, Bo Sun, Yang Liu

ZTE Corporation

Kaushik Josiam,  Jaejeong Shim

Samsung

# Proposed text with change marks

[-------------------------------------------------Start of Text Proposal---------------------------------------------------]

On the 80216m_D4, page 38, line 1, modify the text in 16.2.2.1.1 with the proposed text as follows:

16.2.2.1 .1 Advanced Generic MAC Header（AGMH）

The AGMH format is defined in Table 653. For E-MBS services, the FID field in the AGMH can be ignored.

Table 653 - AGMH Format

| Syntax | Size (bits) | notes |
| --- | --- | --- |
| Advanced Generic MAC Header(){ |  |  |
| Flow ID | 4 | Flow identifier. |
| EH | 1 | Extended header presence indicator; When set to ‘1’, this field indicates that an Extended Header is present following this AGMH. |
| Length | 11 | This field indicates the length in bytes of MAC PDU including the AGMH and extended header if present. If the MLEH is present in a MPDU, Length field indicates 11 LSB of length in byte of MAC PDU. |
| } |  |  |

[-------------------------------------------------End of Text Proposal----------------------------------------------------]