| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | BR ACK timer for BR ACK A-MAP IE (16.2.11.1.1) |  |
| Date Submitted | 2010-03-05 |  |
| Source(s) | Jaehyuk Jang, Hyunkyu Yu, Jung Je Son, Rakesh Taori Samsung Electronics | E-mail: jack.jang@samsung.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE 802.16-10/0011: I EEE 802.16 Working Group Letter Ballot #31 Section 16.2.11.1.1 Contention-based random access bandwidth request |  |
| Abstract | This contribution introduces BR ACK timer for BR ACK A-MAP IE reception. |  |
| Purpose | To be discussed and adopted by WG LB |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

BR ACK timer for BR ACK A-MAP IE (16.2.11.1.1)

Jaehyuk Jang, Hyunkyu Yu, Jung Je Son, Rakesh Taori
Samsung Electronics

# Introduction

This contribution introduces BR ACK timer for BR ACK A-MAP IE reception.

D4 text requires for an ABS to send BR ACK A-MAP IE at the *next* DL frame after receiving BR preamble. This is too strict and rigid condition. An ABS may not be able to generate the BR ACK A-MAP IE at the next DL frame according to its capability.

In order to liberalize the condition, we propose to introduce the BR ACK timer. Using this timer, an ABS can send the BR ACK A-MAP IE after receiving BR preamble at the expiration of the BR ACK timer instead of the next DL frame. And the AMS which sent the BR preamble shall wait the BR ACK A-MAP IE until expiration of the BR ACK timer.

If the BR ACK timer is set to 1 frame, it becomes same as current D4 text. So, the proposal is superset of current text.

# Proposed Text

Update the following section in line 33, page 254 in D4.

----------------------------------------------------- Start of Proposed Text -----------------------------------------------------

16.2 Medium access control

16.2.11 Bandwidth Request and Allocation Mechanism

16.2.11.1.1 Contention-based random access bandwidth request

…

{ Note: line 59, page 254}

The 3-step random access based BR procedure is illustrated in Figure 429. At step 1, the AMS shall transmit a BR preamble sequence and a quick access message on a randomly selected opportunity. The AMS shall start the BR ACK timer after sending BR preamble sequence and a quick access message. The ABS shall start the BR ACK timer after receiving BR preamble sequence. The BR ACK timer is defined in section 10.1 Global values. At lease one BR-ACK A-MAP IE shall be sent in the next DL frameat the expiration of the BR ACK timer if the ABS detects at least one BR preamble sequence in the BR opportunities in the previous frame. The ABS may send multiple BR-ACK A-MAP IEs, with each BR-ACK A-MAP IE containing its own bitmap relating to the preamble sequences being acknowledged/granted in this message alone. Each AMS should try to decode all BR-ACK MAP-IEs in the next DL frame at the expiration of the BR ACK timer after it transmitted a BR preamble sequence. In this case if no BR-ACK A-MAP IEs are not sent in the next DL frame at the expiration of the BR ACK timer, the AMS considers it as an implicit-NACK and may restart BR procedure.

…

{ Note: line 26, page 255}

The AMS shall start the BR timer at the DL frame which is right after the UL frame in which the AMS sent the BR after receiving Positive if the AMS receives neither any UL grant nor Negative-ACK. If the BR-ACK A-MAP IE indicates successful reception of BR preamble sequence and quick access message, the BR Timer value shall be set to the Differentiated BR timer acquired during the DSx transaction. For all other cases, the BR Timer value shall be fixed.

…

{ Note: line 61, page 256}

In the regular 5-step random access BR procedure, an AMS shall send a BR preamble sequence only. The AMS shall start the BR ACK timer after sending BR preamble sequence. If the AMS receives neither any UL grant nor Negative-ACK at the next DL frame after it has transmitted the BR preamble sequence after expiration of the BR ACK timer, a fixed value of BR timer shall be activated. The rest of the BR procedure shall be the same as the fallback 5-step procedure of Figure 430.

------------------------------------------------------ End of Proposed Text ------------------------------------------------------