| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarifications on the BR ACK A-MAP IE format (16.3.6.5.2.4.12) |  |
| Date Submitted | 2010-03-14 |  |
| Source(s) | Jaehyuk Jang, Hyunkyu Yu, Jung Je Son, Rakesh Taori Samsung Electronics | E-mail: jack.jang@samsung.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE 802.16-10/0011: I EEE 802.16 Working Group Letter Ballot #31 Section 16.3.6.5.2.4.12 BR ACK A-MAP IE |  |
| Abstract | This contribution provides several clarifications on the BR ACK A-MAP IE format. |  |
| Purpose | To be discussed and adopted by WG LB |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarifications on the BR ACK A-MAP IE format (16.3.6.5.2.4.12)

Jaehyuk Jang, Hyunkyu Yu, Jung Je Son, Rakesh Taori
Samsung Electronics

# Introduction

This contribution provides several clarifications on the BR ACK A-MAP IE format.

D4 text allows multiple BR ACK A-MAP IE transmission in a frame. In order to prevent unnecessary decoding A-MAP IE, BS needs to indicate whether this BR ACK A-MAP IE is the last one or not. We propose to add 1 bit ‘More BR ACK A-MAP IE’ field for that purpose.

Besides, some text clarifications are provided in this contribution.

# Proposed Text

Update the following section in line 15, page 522 in D4.

----------------------------------------------------- Start of Proposed Text -----------------------------------------------------

16.3 Physical layer

16.3.6 Downlink control structure

16.3.6.5.2.4.12 BR ACK A-MAP IE

…

{ Note: line 41, page 522}

| Table 838— BR-ACK A-MAP IE Format |  |  |
| --- | --- | --- |
| Syntax | Size (bits) | Notes |
| BR-ACK_A-MAP_ IE() { | — | — |
| A-MAP IE Type | 4 | BR-ACK A-MAP IE |
| BR-ACK Bitmap | N_BR_Opportunities | Each bit indicates the decoding status of BR preamble sequence in the corresponding BR opportunity. The length of bitmap determines based on the number of BR opportunities in a frame. 0b0: No BR preamble sequence is detected, 0b1: At least one preamble sequence is detected |
| MSB of resource start offset | 2 | 0b00, 0b01, 0b10: 2bit-MSB of the start offset of the resource allocation (LRU) 0b11: No grant exist in this BR ACK A-MAP IE. |
| if (MSB of resource start offset != 0b11) { | — | — |
| LSB of resource start offset | 5 | This field is the LSB of the start offset of the Resource allocation (LRU) for BR Header |
| HFA start offset | 6 | This field is start offset of HARQ Feedback Allocation. |
| Allocation size | 1 | Resource size in a subframe which is allocated to a BR preamble sequence index for each BR header 0b0: 1 LRU, 0b1: 2 LRUs |
| Long TTI Indicator | 1 | Indicates number of subframes spanned by the allocated resource for BR header. 0b0: 1 subframe (default) 0b1: 4 UL subframes for FDD or all UL subframes for TDD If number of DL subframes, D, is less than number of UL subframes, U, Long TTI Indicator= 0b1 |
| } | — | — |
| for ( i = 0; i < N_BR_Opportunities ; i++) { | — | — |
| if (BR-ACK Bitmap[i] == 1) { | — | — |
| Number of received preamble sequences (L) | TBD | The number of BR preamble sequence indices included in this ACK A-MAP IE. |
| for (j = 0; j < L; j++) { | — | — |
| Preamble sequence index | 5 | Preamble sequence index received in the BR opportunity |
| MSG decoding indicator | 1 | To indicate the decoding status of quick access message 0b0: MSG not decoded 0b1: MSG decoded relevant to Preamble sequence index |
| If (MSB of resource start offset != 0b11) && (MSG decoding indicator == 0b0) { | — | — |
| Grant indicator | 1 | To indicate whether grant of BR Header for the BR preamble sequence index is included or not If this bit is set, the UL resource is allocated with fixed size and MCS. |
| } | — | — |
| } | — | — |
| } | — | — |
| } | — | — |
| Reserved | variable | For byte alignment |
| } | — | — |

------------------------------------------------------ End of Proposed Text ------------------------------------------------------