| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification on the CDMA_RNG_FLAG field in the AAI-HO-CMD message (16.2.3.11) |  |
| Date Submitted | 2010-03-05 |  |
| Source(s) | Jaehyuk Jang, Jung Je Son, Rakesh Taori Samsung Electronics | E-mail: jack.jang@samsung.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE 802.16-10/0011: I EEE 802.16 Working Group Letter Ballot #31 Section 16.2.3.11 AAI_HO-CMD |  |
| Abstract | This contribution provides a clarification on the CDMA_RNG_FLAG field in the AAI-HO-CMD message. |  |
| Purpose | To be discussed and adopted by WG LB |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarification on the CDMA_RNG_FLAG field in the AAI-HO-CMD message (16.2.3.11)

Jaehyuk Jang, Jung Je Son, Rakesh Taori
Samsung Electronics

# Introduction

If CDMA_RNG_FLG is set to 0 which means CDMA ranging procedure is omitted, the serving ABS must pre-allocate STID at the target ABS through AAI_HO-CMD message, so that the target ABS can allocate uplink resources for the AAI_RNG-REQ message to the AMS. (Unlike 16e, 16m does not have the Fast_Ranging_IE which provides unicast allocation using MAC address)

# Proposed Text

Update the following section in line 13, page 82 in D4.

----------------------------------------------------- Start of Proposed Text -----------------------------------------------------

16.2 Medium access control

16.2.3 MAC Control messages

16.2.3.11 AAI_HO-CMD

…

{ Note: line 13, page 82}

| Table 686—Parameters for AAI_HO-CMD message |  |  |
| --- | --- | --- |
| Name | Value | Usage |
| … |  |  |
| CDMA_RNG_FLAG |  | If CDMA_RNG_FLAG=0, CDMA based ranging shall be skipped. Otherwise, if set to 1, it indicates that ranging shall be done. If CDMA_RNG_FLAG is set to 0, STID at the target BS shall be assigned through the AAI_HO-CMD message. This parameter shall be included for each target ABS when Mode=0b00.; It shall be included when Mode = 0b01. |
| … |  |  |

------------------------------------------------------ End of Proposed Text ------------------------------------------------------