| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Preassigned STID field in the AAI-HO-CMD message (16.2.3.11) |  |
| Date Submitted | 2010-03-05 |  |
| Source(s) | Jaehyuk Jang, Jung Je Son, Rakesh Taori Samsung Electronics | E-mail: jack.jang@samsung.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE 802.16-10/0011: I EEE 802.16 Working Group Letter Ballot #31 Section 16.2.3.11 AAI_HO-CMD |  |
| Abstract | This contribution provides missing field in the AAI_HO-CMD message. |  |
| Purpose | To be discussed and adopted by WG LB |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Preassigned STID field in the AAI-HO-CMD message (16.2.3.11)

Jaehyuk Jang, Jung Je Son, Rakesh Taori
Samsung Electronics

# Introduction

In the handover procedure in D4, the target ABS can assign STID for the AMS through the serving ABS using the AAI_HO-CMD message, but it is missing in the AAI_HO-CMD message.

# Proposed Text

Update the following section in line 51, page 83 in D4.

----------------------------------------------------- Start of Proposed Text -----------------------------------------------------

16.2 Medium access control

16.2.3 MAC Control messages

16.2.3.11 AAI_HO-CMD

…

{ Note: line 51, page 83}

| Table 686—Parameters for AAI_HO-CMD message |  |  |
| --- | --- | --- |
| Name | Value | Usage |
| … |  |  |
| Preassigned STID | 12 | Preassigned STID which will be used at the target ABS. |

------------------------------------------------------ End of Proposed Text ------------------------------------------------------