| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | HO message tables and ASN.1 format (16.2.3) |  |
| Date Submitted | 2010-03-05 |  |
| Source(s) | Jaehyuk Jang Samsung Electronics | E-mail: jack.jang@samsung.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE 802.16-10/0011: I EEE 802.16 Working Group Letter Ballot #31 Section 16.2.3 MAC Control messages |  |
| Abstract | This contribution provides message tables and each ASN.1 format for handover messages. |  |
| Purpose | To be discussed and adopted by WG LB |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

HO message tables and ASN.1 format (16.2.3)

Jaehyuk Jang 
Samsung Electronics

# Introduction

This contribution provides message tables and each ASN.1 format for the following handover messages.

- AAI_SCN-REQ, AAI_SCN-RSP, AAI_SCN-REP

- AAI_HO-REQ, AAI_HO-CMD, AAI_HO-IND

- AAI_NBR-ADV

.

# Proposed Text

Update the following section in D4.

----------------------------------------------------- Start of Proposed Text -----------------------------------------------------

16.2 Medium access control

16.2.3 MAC Control messages

{Remedy #1}

16.2.3.9 AAI HO-IND

{Note: Replace the Table 684 to following table and ASN.1 encoded format}

Table 684—Parameters for AAI_HO-IND message

| M/O | Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| M | HO Event Code | 8 | Used to distinguish AAI_HO-IND among different scenarios 0b00: Target ABS selection in case of multiple candidate T-ABSs. 0b01: All target ABSs in AAI_HO-CMD are unreachable. In this case, the AMS shall include a new target ABS that was not included in AAI_HO-CMD. 0b10: AMS unable to stay connected to serving ABS until expiration of disconnect time 0b11: HO cancel. | NA |
| O | Target ABS ID | 48 | NA | Shall be included when HO Event Code = 0b00 or 0b01 |
| O | Physical_Carrier_Index | 8 | Physical carrier index of the Target ABS where an AMS will perform network reentry procedure | May be included when the target ABS is a multi-carrier ABS and HO Event Code = 0b00 |

-- HandOver Indication Message

AAI-HO-IND ::= SEQUENCE {

-- HO Event Code

evtCode             ENUMERATED {

evtCode0(0), -- Target ABS selection

evtCode1(1), -- All target ABS unreachable

evtCode2(2), -- AMS unable to stay connected

evtCode3(3) -- HO cancel

},

-- Target ABS ID

tgtABSId             OCTET STRING SIZE(6) OPTIONAL,

-- Physical Carrier Index

phyCxrIdx            INTEGER (0 .. 255) OPTIONAL

}

{Remedy #2}

16.2.3.10 AAI_HO-REQ

{Note: Replace the Table 685 to following table and ASN.1 encoded format}

Table 685—Parameters for AAI_HO-REQ message

| M/O | Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| M | Report Metric | 8 | Bitmap indicator of trigger metrics that the AMS reports in this message Bit #0: BS CINR mean Bit #1: BS RSSI mean Bits #3-7: Reserved; shall be set to zero | NA |
| M | N_New_BS_Index | 8 | Number of neighboring ABSs to be considered for HO, which are included in AAI_NBR-ADV message. | NA |
| O | Neighbor_ABS_ID | 48 | - | Shall be included for each recommended neighbor BS |
| O | ABS CINR mean | 8 | - | Shall be included if Report Metric Bit #0 is set to 0 |
| O | ABS RSSI mean | 8 | - | Shall be included if Report Metric Bit #1 is set to 1 |
| O | Relative delay | 8 | - | Shall be included if Report Metric Bit #2 is set to 1 |
| O | Carrier Preassignment Indication | 1 | Indicates whether AMS needs pre-assignment of secondary carriers at the Target ABS. | May be included when Multi-Carrier Support = 1 |

-- HO Request Message

AAI-HO-REQ ::= SEQUENCE {

-- Report Metric

reportMetric        ENUMERATED {

bit0(0), -- BS CINR mean

bit1(1), -- BS RSSI mean

bit2(2) -- Relative delay

},

-- Number of ABS in NBR-ADV msg to be considered for HO

numNewBSIndex       NumBS,

-- Neighbor ABS HO Info Lst

nbrAbsHoInfoLst     NbrAbsHoInfoLst OPTIONAL

}

-- Number of ABS

NumBS ::= INTEGER (0 .. 255)

-- Neighbor ABS HO Info Lst

NbrAbsHoInfoLst ::= SEQUENCE SIZE(NumBS) OF NbrAbsHoInfo

-- Neighbor ABS HO Info

NbrAbsHoInfo ::= SEQUENCE {

nbrBSID             OCTET STRING SIZE(6),

cinrMean            INTEGER (0 .. 255) OPTIONAL,

rssiMean            INTEGER (0 .. 255) OPTIONAL,

relativeDelay       INTEGER (0 .. 255) OPTIONAL,

carrierPreassignInd BOOLEAN            OPTIONAL

}

{Remedy #3}

16.2.3.11 AAI_HO-CMD

{Note: Replace the Table 686 to following table and ASN.1 encoded format}

Table 686—Parameters for AAI_HO-CMD message

| M/O | Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| M | Mode | 2 | 0b00: HO command; 0b01: Zone switch command from MZone to LZone; 0b10: AMS HO request rejected (ABS in list unavailable). In this case, AAI_HO-CMD message shall not include any target ABS. | NA |
| O | HO Reentry Mode | 1 | 1: the AMS maintains communication with Serving ABS while performing network reentry with the Target ABS (Mode=0b00); or the AMS maintains communication with MZone while performing network reentry with LZone in the same frame or on another carrier (Mode=0b01). 0: the AMS disconnects from the Serving ABS before performing network reentry with the Target ABS (Mode=0b00); or the AMS disconnects from the MZone before performing network reentry with the LZone (Mode=0b01). | Should be included when Mode = 0b00. (Note: This should be changed to SHALL) Shall be included when Mode = 0b01. |
| O | HO Reentry Interleaving Interval | 8 (TBD) | If HO Reentry Interleaving Interval > 0, the AMS performs network reentry to the target ABS within the HO Reentry Interleaving Interval and continues data transmission with the Serving ABS in the remaining time. If HO Reentry Interleaving Interval =0, the AMS performs multi-carrier EBB (Established Before Break) HO procedure per 16.2.8.2.9.2.2 | Shall be included when HO Reentry Mode is set to 1 |
| O | HO Reentry Iteration | 3 (TBD) | The requested number of iterating HO Reentry Intervals by an AMS. | Shall be included when HO Reentry Interleaving Interval > 0 |
| O | Disconnect time offset | 8 (TBD) | Difference between Disconnect time and Action time in units of frames. The value of disconnect time shall be calculated for each T-ABS by adding/ subtracting this value with the value of Action time specified for this T-ABS. For HO_Reentry_Mode = 0, Disconnect time will be (Action time - Disconnect time offset); For HO_Reentry_Mode = 1, Disconnect time will be (Action time + Disconnect time offset). | Should be included when Mode = 0b00. |
| O | Resource_Retain_Time | 8 (TBD) | The time when the serving BS discards MS’s context. | Should be included when Mode = 0b00. Shall be included when Mode = 0b01. |
| O | targetBSID | 48 | - | Shall be included for each target ABS when Mode=0b00 |
| O | Preamble Index | 10 (TBD) |  | Shall be included for each target ABS when Mode=0b00 |
| O | FA index | 8 (TBD) |  | Shall be included for each target ABS when Mode=0b00 |
| O | S-SFH change count | 4 |  | Shall be included for each target ABS when Mode=0b00 |
| O | Action Time | 8 (TBD) | (Mode=0b00) Action Time included in this message is the absolute frame number at the serving ABS (Mode=0b01) This value is defined as the frame number that AMS starts zone switch. Action Time included in this message is indicated by frame number | Shall be included for each target ABS when Mode=0b00. Shall be included when Mode = 0b01. |
| O | CDMA_RNG_FLAG | 1 | If CDMA_RNG_FLAG=0, CDMA based ranging shall be skipped. Otherwise, if set to 1, it indicates that ranging shall be done. This parameter shall be included for each target ABS when Mode=0b00.; It shall be included when Mode = 0b01. | Shall be included for each target ABS when Mode=0b00 |
| O | Dedicated Ranging Code Flag | 1 | If set to 0, it indicates that no dedicated ranging code is provided. If set to 1, it indicates that no dedicated ranging code is provided. | May be included for each target ABS when CDMA_RNG_FLAG=1 |
| O | Dedicated Ranging Opportunity Flag | 1 | If set to 0, it indicates that no dedicated ranging opprotunity is provided. If set to 1, it indicates that no dedicated ranging opportunity is provided. | May be included for each target ABS when CDMA_RNG_FLAG=1 |
| O | Dedicated CDMA ranging code | 5 | Indicates the dedicated ranging code. | Shall be included when Dedicated Ranging Code Flag =1 |
| O | Ranging opportunity index | 3 | Indicates the index of the allocated ranging opportunity. | Shall be included when Dedicated Opportunity Code Flag=1 |
| O | Subframe Index | 3 | Indicates the subframe index of the allocated ranging opportunity. | Shall be included when Dedicated Opportunity Code Flag=1 |
| O | HO process optimization | 4 | For each bit location, a value of 0 indicates the associated reentry MAC control messages shall be required, a value of 1 indicates the reentry MAC control message may be omitted. Bit #0: Omit AAI_SBC-REQ/RSP MAC control messages during reentry processing Bit #1: Omit PKM Authentication phase Bit #2: Omit AAI_REG-REQ/RSP message and higher layer protocol triggering (for IP address refresh) during reentry processing. Bit #3: If Bit #3 = 1, Full service and operational state transfer or sharing between Serving BS and Target BS (All static and dynamic context, e.g., ARQ windows, state machines | Shall be included for each target ABS when Mode=0b00 When Bit #2 is set to 0 and IP address refresh is needed, the AAI_REG-REQ/RSP includes Host configuration capabilities and its parameters to provide IP address information. |
| O | Seamless HO | 1 | Indicates whether seamless HO is supported | Shall be included for each target ABS when Mode=0b00 |
| O | Ranging initiation deadline | 8 (TBD) | An AMS shall send the AAI_RNG-REQ message during HO until Ranging initiation deadline | Shall be included for each target ABS when Mode=0b00 |
| O | Service level prediction | 2 | Indicates the level of service the AMS can expect from this ABS. The following encodings apply: 0 = No service possible for this AMS 1 = Some service is available for one or several service flows authorized for the AMS 2 = For each authorized service flow, a MAC connection can be established with QoS specified by the AuthorizedQoSParamSet. 3 = No service level prediction available. | Shall be included for each target ABS when Mode=0b00 |
| O | Physical_Carrier_Index | 8 (TBD) | Physical carrier index of the Target ABS where AMS will perform network reentry procedure. | Should be included for each target ABS which is multi-carrier ABS when Mode = 0b00 |
| O | Carrier Preassignment Indication | 1 | 1: information of pre-assigned secondary carriers by the target ABS is included in this message; 0: no pre-assigned secondary carrier | Shall be included for an AMS with multi-carrier capability when Mode = 0b00 |
| O | N_Preassigned_Carriers | 3 (TBD) | Number of pre-assigned secondary carriers at the target ABS. A value of 0 means no secondary is pre-assigned. | Shall be included when Carrier Preassignment Indication = 1 |
| O | Pre-assigned secondary carrier information | TBD | Carrier information of each pre-assigned secondary carrier, which includes: Carrier Status Bitmap: indicating whether this pre-assigned carrier will be activated immediately after HO procedure is done; Physical carrier index of pre-assigned secondary carriers; Logical carrier index, which is assigned implicitly in the order of Carrier information of each pre-assigned secondary carriers. | Shall be included when N_Preassigned_Carriers > 0 |
| O | LZone Presambile Index | 7 | LZone Preamble Index for AMS to switch from MZone to LZone | Shall be included when Mode = 0b01. |
| O | Pre allocated Basic CID | 16 | Used by the AMS to derive its own primary CID and transport CID in target YBS. | May be included when HO to a LZone or a YBS |

-- HO Command Message

AAI-HO-CMD ::= SEQUENCE {

-- Mode

mode          ENUMERATED {

hoCmd(0),

zoneSwitchCmd(1),

hoReqRej(2)

},

-- HO Re Entry Mode Indication

hoReEntryInd  BIT STRING SIZE(1) OPTIONAL,

-- HO Re Entry Interleaving Interval

hoReEntryInterleavingInt        BIT STRING SIZE(8) OPTIONAL,

-- HO Re Entry Iteration

hoReEntryIteration              BIT STRING SIZE(3) OPTIONAL,

-- Disconnect Time Offset

discTimeOffset                  BIT STRING SIZE(8) OPTIONAL,

-- Resource Retain Time SIZE TBD

rsrcRetainTime                 BIT STRING SIZE(8) OPTIONAL,

-- Target BS ID List

tgtBsIdHoInfoLst               BSHoInfoLst OPTIONAL,

-- LZone Presambile Index

lzonePreambleIdx     	    INTEGER (0 .. 127),

-- Pre allocated Basic CID

preallocatedBasicCID 	    INTEGER (0 .. 65535) OPTIONAL

}

-- Number of Target Base Stations

NumTBS     ::= INTEGER (0..15)

-- BS HandOver Info List

BSHoInfoLst ::= SEQUENCE SIZE(NumTBS) OF BSHoInfo

-- BS HandOver Info

BSHoInfo ::= SEQUENCE {

bsId         BIT STRING (SIZE (48)),

preambleIdx  INTEGER (0 .. 511),

faIdx        INTEGER (0 .. 255),

sSFHChngCnt  INTEGER (0 .. 15),

actionTime   INTEGER (0 .. 255),

-- cdma rng flag, 0: no rng 1: rng shall be done

cdmaRNGflag  BIT STRING SIZE(1),

-- dedicated rng code, 0: no dedicated code

dedicatedCDMARngflg    BIT STRING SIZE(1) OPTIONAL,

-- Dedicated Rng Code

dedicatedCDMRngCode    INTEGER (0 .. 31) OPTIONAL,

-- Dedicated rng opp flag

rngOppFlg    BIT STRING SIZE(1) OPTIONAL,

-- Dedicated Rng Opp Index

rngOppIdx    BIT STRING SIZE(3) OPTIONAL,

rngSubframeIdx          INTEGER (0 .. 7)    OPTIONAL,

hoPrcxOpt    BIT STRING (SIZE (4)),

seamlessHO   BOOLEAN,

rngInitDeadline          INTEGER (0 .. 255),

svcLvl       INTEGER (0 .. 3),

phyCxrIdx    INTEGER (0 .. 255)  OPTIONAL,

cxrPreAssignInd          BOOLEAN OPTIONAL,

numPreAssignCxrs         INTEGER (0 .. 7),

preAssigned2ndCxrInfo    SEQUENCE {

...

} OPTIONAL

}

{Remedy #4}

16.2.3.12 AAI_NBR-ADV

{Note: Replace the Table 687 to following table and ASN.1 encoded format}

Table 687—AAI_NBR-ADV message format

| M/O | Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| M | Change Count | 3 | NBR-ADV Change Count | NA |
| M | Cell type | 3 | Cell type in this message 0b000 macro 0b001 micro 0b010 macro hotzone 0b011 femto 0b100 relay 0b101-0b111 reserved | NA |
| M | Total Number of AAI_NBR-ADV Segments | 4 | Total number of segments of AAI_NBR-ADV for this cell type | NA |
| M | AAI_NBR-ADV Segment Index | 4 | Indicates current segment index of this message in the specific cell type | NA |
| M | BS number M | 8 | Total number of BSs to be included in this AAI_NBR-ADV segment | NA |
| M | Starting ABS Index | 8 | Starting ABS Index is the index offset from the last ABS of the previous AAI_NBR-ADV segment. If this is the first AAI_NBR-ADV segment, the Starting ABS Index will be 0. Hence, each AAI_NBR-ADV segment has one Index which corresponds to the first ABS in that AAI_NBR-ADV segment. | NA |
| M | BSID | 48 | - | Shall be included for each neighbor BS |
| M | Number of carriers | 3 | Number of carriers of the BS | Shall be included for each neighbor BS |
| M | SA-PREAMBLE index | 10 | - | Shall be included for each carrier of each neighbor BS |
| M | A-PREAMBLE transmit power | 8 | - | Shall be included for each carrier of each neighbor BS |
| M | Physical carrier index | 6 | Refer to the physical carrier index in AAI_Global-Config message | Shall be included for each carrier of each neighbor BS |
| M | MAC protocol versions | 8 | MAC protocol version of the BS Consistent with REV.2 definition, with new MAC protocol version 9 defined for 16m. | Shall be included for each carrier of each neighbor BS |
| M | SFH_encoding_format | 2 | 0b00: full Subpkt information 0b01: delta encoding (the 1st BS in this cell type shall use full Subpkt encoding ) 0b10: no SFH included 0b11: reserved For macrocell ABS, the bitmap shall be either 0b00 or 0b01 | Shall be included for each neighbor BS |
| M | Control_bitmap | 3 | Each bit maps to one SFH subpacket or extended broadcast information. For macrocell ABS, the bitmap shall be 111 | Shall be included for each neighbor BS |
| O | SFH Subpkt 1 | 88 | - | Shall be included for each neighbor BS if SFH_encoding_format is set to 0b00, and Control_bitmap Bit #0 is set to 1 |
| O | SFH Subpkt 2 | 88 | - | Shall be included for each neighbor BS if SFH_encoding_format is set to 0b00, and Control_bitmap Bit #1 is set to 1 |
| O | SFH Subpkt 3 | 88 | - | Shall be included for each neighbor BS if SFH_encoding_format is set to 0b00, and Control_bitmap Bit #2 is set to 1 |
| O | SFH delta information | Var. (TBD) | Delta encoding, w.r.t. the reference BS | Shall be included for each neighbor BS if SFH_encoding_format is set to 0b01 |
| O | Neighbor-specific trigger TLVs | Var. (TBD) | Optional neighbor-specific triggers with encoding defined in Table 744 | May be included for each neighbor BS |

-- Neighbor Advertisement Message

AAI-NBR-ADV ::= SEQUENCE {

messageType         OCTET STRING(SIZE(1)),

-- Change Count

chngCount           INTEGER (0 .. 7),

-- Cell Type

cellType            ENUMERATED {

macro(0),

micro(1),

macroHotZone(2),

femto(3),

relay(4)

},

-- Total Number of AAI_NBR-ADV Segments

numOfAdvSegments    INTEGER (0 .. 15),

-- Current Segment Idx in specific cell type

advSegmentIdx       INTEGER (0 .. 15),

-- Total number of BS to be included

numBS               NumNbrBS,

-- Starting ABS Index

startingABSIdx      INTEGER (0 .. 255),

-- Neighbor BS Info List

nbrBSInfoList       NbrBSInfoList

}

-- Neighbor BS Info List

NbrBSInfoList ::= SEQUENCE (SIZE(0 .. 15)) OF NbrBSInfo

-- Neighbor BS Info

NbrBSInfo ::= SEQUENCE  {

bsIdx         INTEGER (0 .. 1024), -- BS Index

bsId          OCTET STRING SIZE(6),

numOfCxrs     NumOfCxrs, -- NC

bsCxrInfoLst  BsCxrInfoLst,

sfhEncFmt     ENUMERATED {

fullSubPktInfo(0),

deltaEnc(1),

noSFHIncl(2),

reserved(3)

},

ctrlBmp       BIT STRING SIZE(4),

subPktType    SEQUENCE {

type1 [1] INTEGER (0 .. 88) OPTIONAL,

type2 [2] INTEGER (0 .. 88) OPTIONAL,

type3 [3] INTEGER (0 .. 88) OPTIONAL

} OPTIONAL,

sfhDeltaInfo  SEQUENCE { ... } OPTIONAL, -- TBD

trigTLV       SEQUENCE { ... } OPTIONAL -- TBD

}

-- BS Carrier Info List

BsCxrInfoLst ::= SEQUENCE SIZE(NumOfCxrs) OF BsCxrInfo

-- BS CxrInfo

BsCxrInfo ::= SEQUENCE {

preamble-A-Idx     INTEGER (0 .. 15),

preamble-A-txPwr   INTEGER (0 .. 255),

phyCxrIdx          INTEGER (0 .. 255),

macVer             MacVerLst

}

MacVerLst ::= SEQUENCE (SIZE ( 0 .. 15 )) OF MacVer

MacVer ::= SEQUENCE {

verNumber          INTEGER ( 0 .. 15 ),

preambleIdx        INTEGER ( 0 .. 63 )

}

-- Number of Carriers (NC)

NumOfCxrs ::= INTEGER (0 .. 31) -- RANGE:TBD

--Number of Neighboring BS

NumNbrBS ::= INTEGER (1 .. 256)

{Remedy #5}

16.2.3.13 AAI_SCN-REQ

{Note: Replace the Table 688 to following table and ASN.1 encoded format}

Table 688—Parameters for AAI_SCN-REQ message

| M/O | Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| M | Scan Purpose | 1 | 0b0: scan BS(s) which is in the list of the AAI_NBR-ADV message 0b1: scan BS(s) which is not in the list of the AAI_NBR-ADV message | NA |
| M | Scan duration | 8 | Duration (in units of AAI subframes) of the requested scanning period. | NA |
| M | Interleaving Interval | 8 | The period of AMS’s normal operation which is interleaved between Scan Durations. | NA |
| M | Scan Iteration | 8 | The requested number of iterating scanning interval by an AMS. | NA |
| O | Recommended start super frame number / frame number | 8 | Bit#0-#5: Represents the 6 least significant bits of the absolute super frame number for which the AMS recommends the first Scanning Interval to start. This field is set to 0 if an AMS has no preferred value. Bit#6-#7Represents recommended start frame number within the super frame number which is indicated by Recommended start super frame number field. The value indicates: 0b00: The 1st frame in the super frame 0b01: The 2nd frame in the super frame 0b10: The 3rd frame in the super frame 0b11: The 4th frame in the super frame |  |
| O | N_Recommended_BS_Index | 8 | This is the number of neighboring BS to be scanned, which are included in AAI_NBR-ADV message. When an AMS receives AAI_SCN-RSP message from BS in response to AAI_SCN-REQ message, the AMS shall check whether Configuration Change Count stored by the AMS is the same as one included in AAI_SCN-RSP message sent by the ABS. If an AMS detects mismatch of Configuration Change Counts, it may retransmit AAI_SCN-REQ message to the ABS. | Depending on the scenario |
| O | Configuration Change Count for AAI_NBR-ADV | 4 | The value of Configuration Change Count in AAI_NBR-ADV message referred in order to compress neighbor BSID | Shall be presented if N_Recommended_BS_Index is presented. |
| O | Neighbor_BS_Index | 8 | Based on the value of N_Recommended_BS_Index, BS index corresponds to position of BS in AAI_NBR-ADV message is included. | Shall be included when N_Recommended_BS_Index > 0 |
| O | N_Recommended_BS_Full | 8 | Number of neighboring BS to be scanned, which are using full 48-bit BSID. | Depending on the scenario |
| O | Recommended BS ID | 48 | Based on the value of N_Recommended_BS_Full, identifiers of the BSs the MS plans to scan is included. | Shall be included when N_Recommended_BS_Full > 0 |
| O | N_Recommended_ Preambe_Index | 8 | Number of preambles to be scanned | Depending on the scenario |
| O | Preamble Index | 10 | Based on the value of N_Recommended_BS_Full, identifiers of the BSs the MS plans to scan is included. | Shall be included when N_Recommended_Preamble_Index> 0 |
| O | N_Recommended_Carrier_Index | 8 | Number of carriers to be scanned at each neighboring ABS. | May be included when N_Recommended_BS_Index > 0 or N_Recommended_BS_Full > 0 |
| O | Recommended_Carrier_Index | 8 | Recommended physical carrier index for scan at each neighboring ABS. | Shall be included when N_Recommended_Carrier_Index > 0 |
| O | N_Recommended_Carrier_Index_at_Serving_ABS | 8 | Number of physical carrier index of the serving ABS to be scanned | Depending on the scenario |
| O | Carrier index at serving ABS | 8 | Recommended physical carrier index to be used for scanning | Shall be included when N_Recommended_Carrier_Index_at_Serving_ABS > 0 |
| O | CSGID(s) | 8 | One or more identifiers for one or more CSG(s) | Sent by AMS to aid ABS reselection |

-- Scanning Interval Allocation Request Message

AAI-SCN-REQ ::= SEQUENCE {

messageType         OCTET STRING(SIZE(1)),

--Duration of the requested scanning period

scnDuration         INTEGER,

--Purpose of scanning

scnPurpose          ScanPurpose,

--Interleaving Interval

interleavePeriod    INTEGER,

--Requested number of iterating

--scanning interval by an AMS.

scnIteration        INTEGER,

--6 lsb of the absolute super frame number for which

--the AMS recommends the first Scanning Interval.

--Default 0

recStartSuperFrameNum BIT STRING SIZE(12) OPTIONAL , --6lsb of super frame number

--Recommended start frame number within the super frame number

recStartFrameNum    FrameNum OPTIONAL,

--Recommended Scanning Info

--Includes BsInfo, PreambleInfo & CxrInfo

recScnInfo          RecScnInfo,

--One or more identifiers for one or more CSG(s)

csgId               CsgId

}

MetricInfo ::= SEQUENCE {

--BS CINR Mean - 1 signed byte with units of 0.5 db

bsCinrMean          OCTET STRING SIZE(1) OPTIONAL,

--BS RSSI Mean - 1 unsigned byte with units of 0.25db

--eg:0x00 is interpreted as -103.75 dBm

bsRssiMean          OCTET STRING SIZE(1) OPTIONAL,

--Relative Delay

relativeDelay       INTEGER ( 0 .. MAX ) OPTIONAL,

-- BS Round Trip Delay - unsigned byte

bsRtd               OCTET STRING SIZE(1) OPTIONAL

}

ReportInfo ::= SEQUENCE {

--Report Mode

reportMode          ENUMERATED {

none(0),

periodic(1),

evtTrig(2),

oneTime(3)

},

--Report Metric

reportMetric        BIT STRING {

bsCinrMean(0),

bsRssiMean(1),

relativeDelay(2),

bsRtd(3)

} SIZE(5),

--Report Period

reportPeriod        INTEGER OPTIONAL

}

ScnInfo ::= SEQUENCE {

--neighboring BSID Index Info

bsIdxInfo           BSIdxInfo OPTIONAL,

--BS Info

bsInfo              BSInfo OPTIONAL,

--Preamble Info

preambleInfo        PreambleInfo OPTIONAL,

--Carrier Info

cxrInfo             CxrInfo OPTIONAL,

--Serving ABS Carrier Info

servAbsCxrInfo      ServABSCxrInfo OPTIONAL,

-- CSG ID

csgIdLst            CsgIdLst OPTIONAL

}

-- CSG Id List

CsgIdLst ::= SEQUENCE (SIZE(0 .. 10)) OF CsgId

-- CSG ID: CsgID Assumed as an integer for compilation puposes

CsgId ::= INTEGER (0 .. 255)

BSIdxInfo ::= SEQUENCE {

--Number of Neighboring BS to be scanned

nBsIdx              INTEGER ,

--Configuration Change Count for NBR-ADV

nbrAdvCCnt          INTEGER ,

--Recommended BS idx corresponds to position of BS

--in AAI-NBR-ADV message

nbrBsIdx            INTEGER

}

BSInfo ::= SEQUENCE {

--Number of Neighboring BS to be scanned

nBsFull             INTEGER OPTIONAL,

--AMS BS ID - shall be included if N_BS_Full > 0

bsId                OCTET STRING SIZE(6) OPTIONAL

}

PreambleInfo ::= SEQUENCE {

--Number of preambles to be scanned

nPreambleIdx        INTEGER ,

--Preamble IDs the MS plans to scan

--Shall be included if nRecPreambleIdx > 0

preambleIdx         INTEGER

}

CxrInfo ::= SEQUENCE {

--Number of cxrs to be scanned at each neighboring ABS.

--Included if(nRecBsIdx >0 || nRecBsFull >0)

nCxrIdx             INTEGER OPTIONAL,

--Cxr Idx for scan at each neigboring ABS

cxrIdx              INTEGER OPTIONAL

}

ServABSCxrInfo ::= SEQUENCE {

--Number of phy cxr index of the serving ABS to be scanned

nServAbsCxrIdx      INTEGER OPTIONAL,

--Recommended Phy Cxr idx to be used for scanning

servAbsCxrIdx       INTEGER OPTIONAL

}

--SDC: Indication Bit is assumed to be 1

BsIdxInd ::= BIT STRING SIZE (1)

BsFullInd ::= BIT STRING SIZE(1)

PreambleIdxInd ::= BIT STRING SIZE(1)

NBsIdxInd ::= BIT STRING SIZE (1)

NbrScnInfo ::= ScnInfo

RecScnInfo ::= ScnInfo

NbrBSInfo ::= BSInfo

RecBSInfo ::= BSInfo

NbrPreambleInfo ::= PreambleInfo

RecPreambleInfo ::= PreambleInfo

NbrServABSCxrInfo ::= ServABSCxrInfo

RecServABSCxrInfo ::= ServABSCxrInfo

--Frame number within the super frame number

FrameNum ::= ENUMERATED {

firstFrame(0),

secondFrame(1),

thirdFrame(2),

fourthFrame(3)

}

ScanPurpose ::= ENUMERATED {

bsList0(0),

bsList1(1)

}

{Remedy #6}

16.2.3.13 AAI_SCN-RSP

{Note: Replace the Table 689 to following table and ASN.1 encoded format}

Table 689—Parameters for AAI_SCN-RSP message

| M/O | Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| M | Scan Purpose | 1 | 0b0: scan BS(s) which is in the list of the AAI_NBR-ADV message 0b1: scan BS(s) which is not in the list of the AAI_NBR-ADV message | NA |
| M | Scan duration | 8 | Duration (in units of AAI subframes) of the requested scanning period. | NA |
| M | Report mode | 2 | 0b00:No report 0b01:Periodic report 0b10:Event-triggered report 0b11:One-time scan report | NA |
| M | Report metric | 6 | Bitmap indicator of trigger metrics that the serving ABS requests the AMS to report. The serving ABS shall indicate only the trigger metrics agreed during AAI_SBC-REQ/RSP negotiation. Each bit indicates whether reports will be initiated by trigger based on the corresponding metric: Bit 0: BS CINR mean Bit 1: BS RSSI mean Bit 2: Relative delay Bit 3: BS RTD Bits 4-5: Reserved; shall be set to zero | NA |
| O | Report period | 8 | The period of MS's report of scanning result when the MS is required to report the value periodically or one-time. The period is calculated from the start of the first scan duration. If the BS sends an unsolicited AAI_SCN-RSP message without assignment of a scanning interval and the scan duration is set to zero, the period is calculated from the frame the MS receives the AAI_SCN-RSP message. For MS scanning request denied by AAI_SCN-RSP with Scan Duration set to zero, Report period is the number of frames that BS suggests to MS before transmitting next AAI_SCN-REQ | when the MS is required to report the value periodically or one-time |
| O | Start super frame number/ frame number | 8 | Bits#0-#5: Represents the 6 least significant bits of the absolute frame number in which the first Scanning interval starts Bits#6-7: Represents start frame number in the Start super frame number. 0b00: The 1st frame in the super frame 0b01: The 2nd frame in the super frame 0b10: The 3rd frame in the super frame 0b11: The 4th frame in the super frame | Included only when Scan Duration > 0 |
| O | Interleaving Interval | 8 | The period of AMS's normal operation which is interleaved between Scan Durations. | Included only when Scan Duration > 0 |
| O | Scan Iteration | 8 | The number of iterating scanning interval. | Included only when Scan Duration > 0 |
| O | N_Recommended_BS_Index | 8 | Number of neighboring BS to be scanned, which are included in AAI_NBR-ADV message. | Included only when Scan Duration > 0 |
| O | Configuration Change Count for AAI_NBR-ADV | 4 | The value of Configuration Change Count in AAI_NBR-ADV message referred in order to compress neighbor BSID | Included when N_Recommended_BS_Index > 0 |
| O | Neighbor_BS_Index | 8 | Based on the value of N_Recommended_BS_Index, BS index corresponds to position of BS in AAI_NBR-ADV message is included. | Included when N_Recommended_BS_Index > 0 |
| O | N_Recommended_BS_Full | 8 | Number of neighboring BS to be scanned, which are using full 48-bit BS ID. | Included when Scan Duration > 0 |
| O | Recommended BS ID | 48 | Based on the value of N_Recommended_BS_Full, Recommended BS ID list for scan is included | Included when N_Recommended_BS_Full > 0 |
| O | Preamble Index of Recommended BS ID | 10 | Preamble Index of Recommended BS ID | Shall be included when N_Recommended_BS_Full > 0 |
| O | N_Recommended_Carrier_Index | 8 | Number of physical carriers to be scanned at each neighboring ABS. | May be included for a multi-carrier target ABS |
| O | Recommended_Carrier_Index | 8 | Recommended physical carrier index for scan at each neighboring ABS. | Included when N_Recommended_Carrier_Index > 0 |
| O | Carrier index at serving ABS | 8 | Recommended physical carrier index to be used for scanning | May be included when the serving ABS is a multi-carrier ABS |

-- Scanning Interval Allocation Response Message

AAI-SCN-RSP ::= SEQUENCE {

messageType         OCTET STRING(SIZE(1)),

--Duration of the requested scanning period

scnDuration         INTEGER,

--Purpose of scanning

scnPurpose          ScanPurpose,

--Report Info

reportInfo          ReportInfo,

--6 lsb of the absolute super frame number for which

--the AMS recommends the first Scanning Interval.

--Default 0

recStartSuperFrameNum BIT STRING SIZE(12) OPTIONAL , --6lsb of super frame number

--Recommended start frame number within the super frame number

recStartFrameNum    FrameNum OPTIONAL,

--Interleaving Interval

interleavePeriod    INTEGER,

--Requested number of iterating

--scanning interval by an AMS.

scnIteration        INTEGER,

--Recommended Scanning Info

--Includes BsInfo, PreambleInfo & CxrInfo

recScnInfo          RecScnInfo,

--One or more identifiers for one or more CSG(s)

csgId               CsgId

}

MetricInfo ::= SEQUENCE {

--BS CINR Mean - 1 signed byte with units of 0.5 db

bsCinrMean          OCTET STRING SIZE(1) OPTIONAL,

--BS RSSI Mean - 1 unsigned byte with units of 0.25db

--eg:0x00 is interpreted as -103.75 dBm

bsRssiMean          OCTET STRING SIZE(1) OPTIONAL,

--Relative Delay

-- SDC: Range TBD

relativeDelay       INTEGER ( 0 .. MAX ) OPTIONAL,

-- BS Round Trip Delay - unsigned byte

bsRtd               OCTET STRING SIZE(1) OPTIONAL

}

ReportInfo ::= SEQUENCE {

--Report Mode

reportMode          ENUMERATED {

none(0),

periodic(1),

evtTrig(2),

oneTime(3)

},

--Report Metric

reportMetric        BIT STRING {

bsCinrMean(0),

bsRssiMean(1),

relativeDelay(2),

bsRtd(3)

} SIZE(5),

--Report Period

reportPeriod        INTEGER OPTIONAL

}

ScnInfo ::= SEQUENCE {

--neighboring BSID Index Info

bsIdxInfo           BSIdxInfo OPTIONAL,

--BS Info

bsInfo              BSInfo OPTIONAL,

--Preamble Info

preambleInfo        PreambleInfo OPTIONAL,

--Carrier Info

cxrInfo             CxrInfo OPTIONAL,

--Serving ABS Carrier Info

servAbsCxrInfo      ServABSCxrInfo OPTIONAL

}

BSIdxInfo ::= SEQUENCE {

--Number of Neighboring BS to be scanned

nBsIdx              INTEGER OPTIONAL,

--Configuration Change Count for NBR-ADV

nbrAdvCCnt          INTEGER OPTIONAL,

--Recommended BS idx corresponds to position of BS

--in AAI-NBR-ADV message

nbrBsIdx            INTEGER OPTIONAL

}

BSInfo ::= SEQUENCE {

--Number of Neighboring BS to be scanned

nBsFull             INTEGER OPTIONAL,

--AMS BS ID - shall be included if N_BS_Full > 0

bsId                OCTET STRING SIZE(6) OPTIONAL

}

PreambleInfo ::= SEQUENCE {

--Number of preambles to be scanned

nPreambleIdx        INTEGER OPTIONAL,

--Preamble IDs the MS plans to scan

--Shall be included if nRecPreambleIdx > 0

preambleIdx         INTEGER OPTIONAL

}

CxrInfo ::= SEQUENCE {

--Number of cxrs to be scanned at each neighboring ABS.

--Included if(nRecBsIdx >0 || nRecBsFull >0)

nCxrIdx             INTEGER OPTIONAL,

--Cxr Idx for scan at each neigboring ABS

cxrIdx              INTEGER OPTIONAL

}

ServABSCxrInfo ::= SEQUENCE {

--Number of phy cxr index of the serving ABS to be scanned

nServAbsCxrIdx      INTEGER OPTIONAL,

--Recommended Phy Cxr idx to be used for scanning

servAbsCxrIdx       INTEGER OPTIONAL

}

--SDC: Indication Bit is assumed to be 1

BsIdxInd ::= BIT STRING SIZE (1)

BsFullInd ::= BIT STRING SIZE(1)

PreambleIdxInd ::= BIT STRING SIZE(1)

NBsIdxInd ::= BIT STRING SIZE (1)

NbrScnInfo ::= ScnInfo

RecScnInfo ::= ScnInfo

NbrBSInfo ::= BSInfo

RecBSInfo ::= BSInfo

NbrPreambleInfo ::= PreambleInfo

RecPreambleInfo ::= PreambleInfo

NbrServABSCxrInfo ::= ServABSCxrInfo

RecServABSCxrInfo ::= ServABSCxrInfo

--SDC:CSGID assumed to be an integere for compilation purposes

CsgId ::= INTEGER

--Frame number within the super frame number

FrameNum ::= ENUMERATED {

firstFrame(0),

secondFrame(1),

thirdFrame(2),

fourthFrame(3)

}

ScanPurpose ::= ENUMERATED {

bsList0(0),

bsList1(1)

}

{Remedy #7}

16.2.3.13 AAI_SCN-REP

{Note: Replace the Table 690 to following table and ASN.1 encoded format}

Table 690—Parameters for AAI_SCN-REP message

| M/O | Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| M | Report mode | 1 | Action code for an AMS's scan report of its measurement 0: Event-triggered report 1: Periodic report according to Scan report period of AAI_SCN-RSP | NA |
| M | Report metric | 6 | Bitmap indicator of trigger metrics that the serving ABS requests the AMS to report. The serving ABS shall indicate only the trigger metrics agreed during AAI_SBC-REQ/RSP negotiation. For each bit location, a value of 0 indicates the trigger metric is not included, while a value of '1' indicates the trigger metric is included in the message. The bitmap interpretation for the metrics shall be: Bit 0: BS CINR mean Bit 1: BS RSSI mean Bit 2: Relative delay Bit 3: BS RTD Bits 4-5: Reserved; shall be set to zero |  |
| O | BS CINR mean | 8 | The BS CINR Mean parameter indicates the CINR measured by the AMS from the particular ABS. The value shall be interpreted as a signed byte with units of 0.5 dB. The measurement shall be performed on the subcarriers of the frame preamble that are active in the serving ABS's segment and averaged over the measurement period. | Included when Report metric Bit 0 = 1 |
| O | BS RSSI mean | 8 | The BS RSSI Mean parameter indicates the Received Signal Strength measured by the AMS from the particular ABS. The value shall be interpreted as an unsigned byte with units of 0.25 dB, e.g., 0x00 is interpreted as -103.75 dBm. An MS shall be able to report values in the range -103.75 dBm to -40 dBm. The measurement shall be performed on the frame preamble and averaged over the measurement period. | Included when Report metric Bit 1 = 1 |
| O | Relative delay | 8 | This parameter indicates the delay of neighbor DL signals relative to the serving BS, as measured by the MS for the particular BS. The value shall be interpreted as a signed integer in units of samples. | Included when Report metric Bit 2 = 1 |
| O | BS RTD | 8 | The BS RTD parameter indicates the round trip delay (RTD) measured by the AMS from the particular ABS. RTD can be given by the latest time advance taken by MS. The value shall be interpreted as an unsigned byte with units of 1/Fs. | Included when Report metric Bit 3 = 1 |
| O | N_Neighbor_BS_Index | 8 | Number of neighboring BS reported in this message and which are included in AAI_NBR-ADV message. | Depending on the scenario |
| O | Configuration Change Count for AAI_NBR-ADV | 4 | The value of Configuration Change Count in AAI_NBR-ADV message referred in order to compress neighbor BSID | Included when N_Neighbor_BS_Index_indication = 1 |
| O | Neighbor_BS_Index | 8 | Based on the value of N_Neighbor_BS_Index, BS index corresponds to position of BS in AAI_NBR-ADV message is included. | Included when N_Neighbor_BS_Index > 0 |
| O | N_Neighbor_BS_Full | 8 | Number of neighboring BS reported in this message that are using full 48 bits BS ID. | Depending on the scenario |
| O | Neighbor_BSID | 48 | Based on the value of N_Neighbor_BS_Full, full BSID (48-bit) is included. | Included when N_Neighbor_BS_Full > 0 |
| O | N_FA_Index | 8 | Number of scanned FAs | Depending on the scenario |
| O | FA_Index | 8 | Scanned FA Index | Included when N_FA_Index > 0 |
| O | N_Scanned_Preamble_Indices | 8 | Number of scanned preambles reported in this message | Included when N_FA_Index > 0 |
| O | Preamble_Index | 10 | Based on the value of N_Scanned_Preamble_Indices, preamble index (10-bit) is included | Included when N_Scanned_Preamble_Indices > 0 |
| O | N_Recommended_Carrier_Index | 8 | Recommended physical carrier index for scan at each neighboring ABS | May be included for a multi-carrier target ABS |
| O | Recommended_Carrier_Index | 8 | Recommended physical carrier index for scan at each neighboring ABS. | Included when N_Recommended_Carrier_Index > 0 |
| O | Carrier index at serving ABS | 8 | Recommended physical carrier index to be used for scanning | May be included when the serving ABS is a multi-carrier ABS |

-- Scanning Interval Allocation Report Message

AAI-SCN-REP ::= SEQUENCE {

messageType         OCTET STRING(SIZE(1)),

--Report Info

reportInfo          ReportInfo,

--Metric Info

metricInfo          MetricInfo,

--Scanning Info

--Includes BsInfo, PreambleInfo & CxrInfo

nbrScnInfo          NbrScnInfo OPTIONAL,

--N_FA_Index Number of Scanned FAs

nFaIdxNum           INTEGER OPTIONAL,

--Group FA Info

grpFAInfo           GrpFAInfo,

--One or more identifiers for one or more CSG(s)

csgId               CsgId

}

--for i=0 -> NumberOfFAIdx

GrpFAInfo ::= SEQUENCE (SIZE(0 .. 10 )) OF SEQUENCE {

faIdx             INTEGER,

nScnPreambleIdx   INTEGER,

preambleReportVal PreambleReportValue

}

--for i=0 -> Num_scanned_preambles reported in this message

PreambleReportValue ::= SEQUENCE (SIZE(0 .. 10)) OF SEQUENCE {

preambleIdx BIT STRING SIZE(10),

reportVal   INTEGER

}

MetricInfo ::= SEQUENCE {

--BS CINR Mean - 1 signed byte with units of 0.5 db

bsCinrMean          OCTET STRING SIZE(1) OPTIONAL,

--BS RSSI Mean - 1 unsigned byte with units of 0.25db

--eg:0x00 is interpreted as -103.75 dBm

bsRssiMean          OCTET STRING SIZE(1) OPTIONAL,

--Relative Delay

--SDC: Range TBD

relativeDelay       INTEGER ( 0 .. MAX ) OPTIONAL,

-- BS Round Trip Delay - unsigned byte

bsRtd               OCTET STRING SIZE(1) OPTIONAL

}

ReportInfo ::= SEQUENCE {

--Report Mode

reportMode          ENUMERATED {

none(0),

periodic(1),

evtTrig(2),

oneTime(3)

},

--Report Metric

reportMetric        BIT STRING {

bsCinrMean(0),

bsRssiMean(1),

relativeDelay(2),

bsRtd(3)

} SIZE(5),

--Report Period

reportPeriod        INTEGER OPTIONAL

}

ScnInfo ::= SEQUENCE {

--neighboring BSID Index Info

bsIdxInfo        BSIdxInfo OPTIONAL,

--BS Info

bsInfo           BSInfo OPTIONAL,

--Preamble Info

preambleInfo     PreambleInfo OPTIONAL,

--Carrier Info

cxrInfo          CxrInfo OPTIONAL,

--Serving ABS Carrier Info

servAbsCxrInfo   ServABSCxrInfo OPTIONAL

}

BSIdxInfo ::= SEQUENCE {

--Number of Neighboring BS to be scanned

nBsIdx           INTEGER OPTIONAL,

--Configuration Change Count for NBR-ADV

nbrAdvCCnt          INTEGER OPTIONAL,

--Recommended BS idx corresponds to position of BS

--in AAI-NBR-ADV message

nbrBsIdx         INTEGER OPTIONAL

}

BSInfo ::= SEQUENCE {

--Number of Neighboring BS to be scanned

nBsFull          INTEGER OPTIONAL,

--AMS BS ID - shall be included if N_BS_Full > 0

bsId             OCTET STRING SIZE(6) OPTIONAL

}

PreambleInfo ::= SEQUENCE {

--Number of preambles to be scanned

nPreambleIdx     INTEGER OPTIONAL,

--Preamble IDs the MS plans to scan

--Shall be included if nRecPreambleIdx > 0

preambleIdx      INTEGER OPTIONAL

}

CxrInfo ::= SEQUENCE {

--Number of cxrs to be scanned at each neighboring ABS.

--Included if(nRecBsIdx >0 || nRecBsFull >0)

nCxrIdx          INTEGER OPTIONAL,

--Cxr Idx for scan at each neigboring ABS

cxrIdx           INTEGER OPTIONAL

}

ServABSCxrInfo ::= SEQUENCE {

--Number of phy cxr index of the serving ABS to be scanned

nServAbsCxrIdx   INTEGER OPTIONAL,

--Recommended Phy Cxr idx to be used for scanning

servAbsCxrIdx    INTEGER OPTIONAL

}

--SDC: Indication Bit is assumed to be 1

BsIdxInd ::= BIT STRING SIZE (1)

BsFullInd ::= BIT STRING SIZE(1)

PreambleIdxInd ::= BIT STRING SIZE(1)

NBsIdxInd ::= BIT STRING SIZE (1)

NbrScnInfo ::= ScnInfo

RecScnInfo ::= ScnInfo

NbrBSInfo ::= BSInfo

RecBSInfo ::= BSInfo

NbrPreambleInfo ::= PreambleInfo

RecPreambleInfo ::= PreambleInfo

NbrServABSCxrInfo ::= ServABSCxrInfo

RecServABSCxrInfo ::= ServABSCxrInfo

CsgId ::= INTEGER

--Frame number within the super frame number

FrameNum ::= ENUMERATED {

firstFrame(0),

secondFrame(1),

thirdFrame(2),

fourthFrame(3)

}

ScanPurpose ::= ENUMERATED {

bsList0(0),

bsList1(1)

}

------------------------------------------------------ End of Proposed Text ------------------------------------------------------