| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Tables and ASN.1 format for Multicarrier Messages (16.2.3) |  |
| Date Submitted | 2010-03-05 |  |
| Source(s) | Youngsoo Yuk, Kiseon Ryu and Ronny Yongho Kim LG Electronics | E-mail: youngsoo.yuk@lge.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE 802.16-10/0011: I EEE 802.16 Working Group Letter Ballot #31 Section 16.2.3 MAC Control messages |  |
| Abstract | This contribution provides message tables and each ASN.1 format for multicarrier messages. |  |
| Purpose | To be discussed and adopted by WG LB |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Tables and ASN.1 format for Multicarrier Messages

Youngsoo Yuk, Kiseon Ryu and Ronny Yongho Kim

LG Electronics

- Introduction

This contribution provides message tables and each ASN.1 format for the following multicarrier messages.

- AAI_Global-Config

- AAI_MC-REQ, AAI_MC-RSP

- Proposed Text

----------------------------------------------------- Start of Proposed Text -----------------------------------------------------

16.2 Medium access control

16.2.3 MAC Control messages

[Remedy 1]

16.2.3.51 AAI_MC-REQ (multicarrier Request) Message

{Note: Replace the Table 733 to following table and ASN.1 encoded format}

Table 733—Parameters for AAI_MC-REQ message

| M/O | Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| M | Physical_Carrier_Index | 6 | Index of physical carrier which is uniquely defined in the network | NA |
| M | Support of data transmission over guard subcarrier | 1 | 0b0 = not supported 0b1 = support | NA |

-- Multicarrier Request Message

AAI-MC-REQ::= SEQUENCE {

messageType 		OCTET STRING (SIZE(1)),

-- Combinations of candidate assigned carrier

candidateAssignedCarrier 	SEQUENCE(SIZE(1..16)) OF CarrierCombination

-- Flag on the capability of data transmission over guard subcarrier

supportGuardCarrier		ENUMERATED{

notupport (0),

support (1)

}

}

-- Combination of assigned carrier

CarrierCombination::= 	SEQUENCE {

-- List of physical carrier index of the assigned carriers

phyCarrierIndex	SEQUENCE(SIZE(1..8)} OF INTEGER(1..64)

}

[Remedy 2]

16.2.3.52 AAI_MC-RSP (Multicarrier Response) Message

{Note: Replace the Table 734to following table and ASN.1 encoded format}

Tab le 734—Parameters for AAI_MC-RSP message

| M/O | Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| M | Physical_Carrier_Index | 6 | Index of physical carrier which is uniquely defined in the network | NA |
| M | Support of data transmission over guard subcarrier | 1 | 0b0 = not supported 0b1 = support | NA |

-- Multicarrier Request Message

AAI-MC-RSP::= SEQUENCE {

messageType 		OCTET STRING (SIZE(1)),

-- Information on assigned carriers

assignedCarrier 	SEQUENCE(SIZE(1..16) OF AssignedCarrierInfo

}

-- Combination of assigned carrier

AssignedCarrierInfo::= 	SEQUENCE {

-- Physical carrier index of the assigned carriers

phyCarrierIndex	INTEGER(1..64)

-- Flag on the capability of data transmission over guard subcarrier

supportGuardCarrier		ENUMERATED{

notupport (0),

support (1)

}

}

[Remedy 3]

16.2.3.53. AAI_Global-Config (global carrier configuration) Message

{Note: Replace the Table 735 to following table and ASN.1 encoded format}

Table XXX—Parameters for AAI_Global-Config message

| M/O | Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| M | Duplexing Mode | 1 | “0” for TDD “1” for FDD | NA |
| M | Multi-Carrier Configuration Index Across the Network | 6 | Index associated to Table 770 | NA |
| M | Physical_Carrier_Index | 6 | Index of physical carrier which is uniquely defined in the network | NA |
| M | Physical Carrier Index of current carrier | 6 | The carrier that broadcasts this message; the physical carrier index refers to AAI_Global-Config message | NA |

-- Global Carrier Configuration Message

AAI-Global-CFG::= SEQUENCE {

messageType 		OCTET STRING (SIZE(1)),

-- All possible carrier configuration in the Network

netWideCarrierGroupInfo 	SEQUENCE(SIZE(1..16)) OF CarrierGroupInfo

-- Physical carrier index of the carrier this message is sent on

currentCarrierIndex		INTEGER(1..64)

}

-- carrier group configuration information

CarrierGroupInfo::= 	SEQUENCE {

-- Multicarrier configuration index associated table 770

multiCarrierConfigurationIndex 	 INTEGER(1..64),

-- FA Index of the center frequency of first DL carrier in this carrier group.

-- Starting from 400MHz and increasing in unit of channel raster

startFAIndex				 INTEGER(1..16000),

-- Information of Component carriers in this group

compCarrierInfo 			SEQUENCE(SIZE(1..64)) OF CarrierInfo

}

CarrierInfo::=		SEQUENCE{

-- Physical carrier indices of the component carrier

phyCarrierIndex 			INTEGER(1..64)

-- duplexing mode of this component carrier

duplexingMode 				ENUMERATED{

tdd (0),

fdd (1)

}

}

----------------------------------------------------- End of Proposed Text -----------------------------------------------------