| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Tables and ASN.1 format for Multicarrier Messages (16.2.3) |  |
| Date Submitted | 2010-03-18 |  |
| Source(s) | Youngsoo Yuk, Kiseon Ryu and Ronny Yongho Kim LG Electronics | E-mail: youngsoo.yuk@lge.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE 802.16-10/0011: I EEE 802.16 Working Group Letter Ballot #31 Section 16.2.3 MAC Control messages |  |
| Abstract | This contribution provides message tables and each ASN.1 format for multicarrier messages. |  |
| Purpose | To be discussed and adopted by WG LB |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Tables and ASN.1 format for Multicarrier Messages

Youngsoo Yuk, Kiseon Ryu and Ronny Yongho Kim

LG Electronics

- Introduction

This contribution provides message tables and each ASN.1 format for the following multicarrier messages.

- AAI_MC-REQ, AAI_MC-RSP

- Proposed Text

----------------------------------------------------- Start of Proposed Text -----------------------------------------------------

[Remedy: Add the followings after line 21 of page 763: ]

Annex P.2.z AAI_MC-REQ message

MCREQ DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- MultiCarrier Request Message

AAI-MC-REQ ::= SEQUENCE {

messageType        OCTET STRING SIZE(1),

-- present when the AMS supports subset of all carriers supported in ABS. Subset of Cxrs which can be supported by AMS

cxrList            CxrList Optional

}

-- Number of Candidate Combinations

N ::= INTEGER (0 .. 15)

CxrList ::= SEQUENCE (SIZE (N) ) OF Cxrs

Cxrs ::= SEQUENCE {

-- Number of Candidate Assigned Cxrs(Nc)

numOfAssignedCxrs    INTEGER (0 .. 7) OPTIONAL,

-- Cxrs that AMS can simultaneously support

phyCxrLst           PhyCxrLst OPTIONAL,

-- Support for Data txn over guard Sub-Carrier

txnOvrSubCxrSupp   BIT STRING SIZE(1)

}

-- Physical Carrier List

PhyCxrLst ::= SEQUENCE (SIZE(numOfAssignedCxrs)) OF PhyCxrIdx

-- Physical Carrier Index

PhyCxrIdx ::= INTEGER (0 .. 63)

END

Annex P.2.(z+1) AAI_MC-RSP message

MCRSP DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- MultiCarrier Response Message

AAI-MC-RSP ::= SEQUENCE {

messageType         OCTET STRING SIZE(1),

-- present when the AMS supports subset of all carriers requested by AMS. List of assigned carriers

amsAssignedCxrList CxrList Optional,

-- Support for Data txn over guard Sub-Carrier

txnOvrSubCxrSupp   BIT STRING SIZE(1) Optional

}

CxrList ::= SEQUENCE {

-- Number of Assigned Cxrs

numOfAssignedCxrs    INTEGER (0 .. 7),

-- Cxrs that AMS can simultaneously support

phyCxrLst           PhyCxrLst

}

-- Physical Carrier List

PhyCxrLst ::= SEQUENCE (SIZE (numOfAssignedCxrs)) OF PhyCxrIdx

-- Physical Carrier Index

PhyCxrIdx ::= INTEGER (0 .. 63)

END

----------------------------------------------------- End of Proposed Text -----------------------------------------------------