| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Uplink 4TX antenna codebook for CSM mode (Section 16.3.10 of the IEEE 802.16m/D4) |  |
| Date Submitted | 2010-03-05 |  |
| Source(s) | Alexei Davydov Gregory Morozov Apostolos Papathanassiou Srinivasan Roshni Maxim Bovykin Intel Corporation | alexei.davydov@intel.com |
| Re: | Category: LB31 / Area: Chapter 16.3.10 |  |
| Abstract | The contribution proposes 4 bit codebook for UL CSM mode with beamforming |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m Advanced Air Interface (Draft 5) |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Uplink 4TX antenna codebook subset for CSM mode (Section 16.3.10 of the IEEE 802.16m/D4)

Alexei Davydov, Gregory Morozov, Apostolos Papathanassiou, Srinivasan Roshni, Maxim Bovykin

Intel Corporation

# Introduction

To support 4 bit ACID in all UL A-MAP IEs a new Assignment A-MAP IE for the special case of UL collaborative spatial multiplexing (CSM) with beamforming was proposed [1]. By defining a new Assignment A-MAP IE type for the special case of UL collaborative spatial multiplexing with beamforming, the number of bits required for signaling MIMO transmission formats was reduced by one and the additional bit was used for signaling the ACID. In this contribution we propose 4 bits codebook for UL CSM mode with beamforming (which is a subset of 6 bits 4TX codebook) that allows to union Assignment A-MAP and UL CSM beamforming A-MAP IE’s into single Assignment A-MAP IE type. In this case 4 bit ACID can be supported and one IE type can be saved for future needs.

# Simulation results

A system level performance analysis has been carried out for CSM mode with beamforming for 4TX antenna AMS. For CSM mode maximum rank of one per AMS was used. The simulation results are summarized in Table 1.

Table 1. System level simulation of CSM mode with beamforming

|  | IoT in dB {average, standard deviation} | Average sector throughput (Mbps) | Cell-edge user throughput (Kbps) |
| --- | --- | --- | --- |
| 6-bit SU/6-bit MU, low IoT | {2.64, 0.79} | 8.75 | 352 |
| 6-bit SU/4-bit MU, low IoT | {2.75, 0.72} | 8.75 | 344 |
| 6-bit SU/6-bit MU, high IoT | {8.53, 1.33} | 11.67 | 509 |
| 6-bit SU/4-bit MU, high IoT | {8.28, 1.34} | 11.74 | 491 |

It can be seen that usage of 4 bit codebook for CSM mode give performance loss of less than 2-4%.

A similar analysis was conducted for mITU Ped-B channel model on link level for 2 AMS in CSM mode. S-LRU allocation with codebook based precoding was used. The results are presented on Figure 1. The performance for 2x4 antenna configuration is conducted for reference.

Figure 1. Spectral efficiency vs. SNR for CSM mode

It can be seen that usage of 4 bit codebook for CSM mode provides performance loss of about 0.3-0.4dB comparing to 6 bit codebook.

# References

[1] “Updates to Support 4 bit HARQ Channel Indexing in the UL Assignment A-MAP IEs in IEEE P802.16m/D3 (16.3.6.5.2.4.2)”, C80216m_2876r2

# Text proposal for inclusion in the 802.16m/D5

--------------------------------------------------  Text Start #1  ------------------------------------------------

{Modify text on page 669, line 32, section 16.3.10.3.2.2 as follows}

16.3.10.3.2.2 MU-MIMO base codebook

The base codebook for UL collaborative spatial multiplexing MIMO is same as 4 bits subset of the base codebook for UL SU-MIMO, defined in 16.3.10.3.2.1. The codebook is defined in Table X for rank 1, 2 and 3.

Table X - CUL,CL,CSM(4,1,4), CUL,CL,CSM(4,2,4), CUL,CL,CSM(4,3,4)

| UL CSM codebook | Rank | Indexes of base codebook Cbase,UL |
| --- | --- | --- |
| CUL,CL,CSM(4,1,4,m) m=0 to 15 | 1 | 0, 5, 10, 15, 20, 25, 30, 35, 40, 44, 45, 49, 50, 54, 55 ,59 |
| CUL,CL,CSM(4,2,4,m) m=0 to 15 | 2 | 0 ,6, 9, 11, 14, 16, 19, 24, 33, 38, 41, 43, 46, 48, 51, 56 |
| CUL,CL,CSM(4,3,4,m) m=0 to 15 | 3 | 0, 5, 10, 15, 20, 25, 30, 35, 40, 44, 45, 49, 50, 54, 55, 59 |

--------------------------------------------------  Text End #1 ------------------------------------------------