| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Uplink 4TX antenna codebook for CSM mode (Section 16.3.10 of the IEEE 802.16m/D4) |  |
| Date Submitted | 2010-03-16 |  |
| Source(s) | Alexei Davydov Gregory Morozov Apostolos Papathanassiou Srinivasan Roshni Maxim Bovykin Intel Corporation | alexei.davydov@intel.com roshni.m.srinivasan@intel.com |
| Re: | Category: LB31 / Area: Chapter 16.3.10 |  |
| Abstract | The contribution proposes 4 bit codebook for UL CSM mode with beamforming |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m Advanced Air Interface (Draft 5) |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Uplink 4TX antenna codebook subset for CSM mode (Section 16.3.10 of the IEEE 802.16m/D4)

Alexei Davydov, Gregory Morozov, Apostolos Papathanassiou, Srinivasan Roshni, Maxim Bovykin

Intel Corporation

# Introduction

To support 4 bit ACID in all UL A-MAP IEs a new Assignment A-MAP IE for the special case of UL collaborative spatial multiplexing (CSM) with beamforming was proposed [1]. By defining a new Assignment A-MAP IE type for the special case of UL collaborative spatial multiplexing with beamforming, the number of bits required for signaling MIMO transmission formats was reduced by one and the additional bit was used for signaling the ACID. In this contribution we propose a 4 bit codebook for UL CSM mode with beamforming (which is a subset of 6 bits 4TX codebook) that facilitates the merging of the UL Basic Assignment A-MAP IE and the UL CSM beamforming A-MAP IE into a single Assignment A-MAP IE. In this case 4 bit ACID can be supported and one IE type can be saved for future needs.

# Simulation results

A system level performance analysis has been carried out for CSM mode with beamforming for 4TX antenna AMS. For CSM mode maximum rank of one per AMS was used. The simulation results are summarized in Table 1.

Table 1. System level simulation of CSM mode with beamforming

|  | IoT in dB {average, standard deviation} | Average sector throughput (Mbps) | Cell-edge user throughput (Kbps) |
| --- | --- | --- | --- |
| 6-bit SU/6-bit MU, low IoT | {2.64, 0.79} | 8.75 | 352 |
| 6-bit SU/4-bit MU, low IoT | {2.75, 0.72} | 8.75 | 344 |
| 6-bit SU/6-bit MU, high IoT | {8.53, 1.33} | 11.67 | 509 |
| 6-bit SU/4-bit MU, high IoT | {8.28, 1.34} | 11.74 | 491 |

It can be seen that usage of 4 bit codebook for CSM mode give performance loss of less than 2-4%.

A similar analysis was conducted for mITU Ped-B channel model on link level for 2 AMS in CSM mode. S-LRU allocation with codebook based precoding was used. The results are presented on Figure 1. The performance for 2x4 antenna configuration is conducted for reference.

Figure 1. Spectral efficiency vs. SNR for CSM mode

It can be seen that usage of 4 bit codebook for CSM mode provides performance loss of about 0.3-0.4dB comparing to 6 bit codebook.

# References

[1] “Updates to Support 4 bit HARQ Channel Indexing in the UL Assignment A-MAP IEs in IEEE P802.16m/D3 (16.3.6.5.2.4.2)”, C80216m_2876r2

[2] P802.16m/D4, “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks, Part 16: Air Interface for Fixed and Mobile Broadband Wireless Access Systems, Advanced Air Interface”, January 2010.

# Text proposal for inclusion in the 802.16m/D5

- Instructions to Editor

- Page 669, line 32, update the MU-MIMO base codebook with the 4 bit subset and Table X (to be renumbered accordingly).

- Page 460, starting on Line 17: Update table 818 in [2] on A-MAP IE types by changing ‘UL CSM Beamforming A-MAP IE’ to ‘Reserved’. Add the missing word ‘Allocation’ to the DL/UL Persistent A-MAP IEs

- Page 468, starting on Line 1: Replace Table 822 (UL Basic Assignment A-MAP IE in [2] with the table shown in the proposed text.

- Page 525, starting on Line 20: Delete the entire subclause 15.3.6.5.2.4.14 including Table 840 (UL CSM Beamforming A-MAP IE) in [2]

________________________________Begin proposed text change #1_________________________________

{Modify text on page 669, line 32, section 16.3.10.3.2.2 as follows}

16.3.10.3.2.2 MU-MIMO base codebook

The base codebook for UL collaborative spatial multiplexing MIMO is same as 4 bits subset of the base codebook for UL SU-MIMO, defined in 16.3.10.3.2.1. The codebook is defined in Table X for rank 1, 2 and 3.

Table X - CUL,CL,CSM(4,1,4), CUL,CL,CSM(4,2,4), CUL,CL,CSM(4,3,4)

| UL CSM codebook | Rank | Indexes of base codebook Cbase,UL |
| --- | --- | --- |
| CUL,CL,CSM(4,1,4,m) m=0 to 15 | 1 | 0, 5, 10, 15, 20, 25, 30, 35, 40, 44, 45, 49, 50, 54, 55 ,59 |
| CUL,CL,CSM(4,2,4,m) m=0 to 15 | 2 | 0 ,6, 9, 11, 14, 16, 19, 24, 33, 38, 41, 43, 46, 48, 51, 56 |
| CUL,CL,CSM(4,3,4,m) m=0 to 15 | 3 | 0, 5, 10, 15, 20, 25, 30, 35, 40, 44, 45, 49, 50, 54, 55, 59 |

________________________________End proposed text change #1_________________________________

________________________________Begin proposed text change #2_________________________________

Page 460, starting on Line 17

Table 818 -- A-MAP IE Types

| A-MAP IE Type | Usage |
| --- | --- |
| 0b0000 | DL Basic Assignment A-MAP IE |
| 0b0001 | UL Basic Assignment A-MAP IE |
| 0b0010 | DL Subband Assignment A-MAP |
| 0b0011 | UL Subband Assignment A-MAP IE |
| 0b0100 | Feedback Allocation A-MAP IE |
| 0b0101 | UL Sounding Command A-MAP IE |
| 0b0110 | CDMA Allocation A-MAP IE |
| 0b0111 | DL Persistent Allocation A-MAP IE |
| 0b1000 | UL Persistent Allocation A-MAP IE |
| 0b1001 | Group Resource Allocation A-MAP IE |
| 0b1010 | Feedback Polling A-MAP IE |
| 0b1011 | BR-ACK A-MAP IE |
| 0b1100 | Broadcast A-MAP IE |
| 0b1101 | UL CSM Beamforming A-MAP IE Reserved |
| 0b1110 | Reserved |
| 0b1111 | Extended Assignment A-MAP IE |

________________________________End proposed text change #2_________________________________

________________________________Begin proposed text change #3_________________________________

Page 468, starting on Line 1

# 16.3.6.5.2.4.2 UL basic assignment A-MAP IE

Table 822 describes the fields in a UL Basic Assignment A-MAP IE used for resource assignment in the UL.

Table 822 UL basic assignment A-MAP IE*

| Syntax | Size in bits | Description/Notes |
| --- | --- | --- |
| UL_Basic_Assignment_A-MAP_IE() { | - | - |
| A-MAP IE Type | 4 | UL Basic Assignment A-MAP IE |
| ISizeOffset | 5 | Offset used to compute burst size index |
| PMI Indicator | 1 | Flag to indicate if PMI is signaled explicitly 0b0: PMI not signaled explicitly 0b1: PMI signaled explicitly |
| CSM | 1 | Flag to indicate CSM 0b0: SU-MIMO 0b1: CSM |
| if (PMI Indicator == 0b0){ |  | PMI not signaled |
| if (CSM == 0b0){ |  | SU-MIMO (Adaptive without PMI or Non- adaptive) |
| MEF | 1 | MIMO encoder format - 0b0: SFBC - 0b1: Vertical encoding Non-adaptive precoding shall be used at the AMS with SFBC. Mt shall be ignored with SFBC. |
| if (MEF == 0b1){ |  |  |
| Mt | 2 | Number of streams in transmission (Mt <= Nt ), up to 4 streams per AMS supported. Nt is the number of transmit antennas at the AMS. 0b00: 1 stream 0b01: 2 streams 0b10: 3 streams 0b11: 4 streams |
| PF | 1 | Precoding flag for SU-MIMO when PMI is not signaled 0b0: Non-adaptive precoding 0b1: Adaptive precoding using the precoder of rank Mt of the AMS's choice |
| Reserved | 4 | Reserved bits |
| else{ |  |  |
| Reserved | 7 | Reserved bits |
| } |  |  |
| else{ |  | CSM (without PMI) (Adaptive without PMI or Non- adaptive) |
| CSM Format | 4 | For non CSM, modes, TNS = Mt. For CSM modes the following combinations apply for upto 4 streams 0b0000: TNS = 2, Mt=1, SI =1 0b0001: TNS = 2, Mt =1, SI =2 0b0010: TNS = 3, Mt =1, SI =1 0b0011: TNS = 3, Mt =1, SI =2 0b0100: TNS = 3, Mt =1, SI =3 0b0101: TNS = 3, Mt =2, SI =1 0b0110: TNS = 3, Mt =2, SI =2 0b0111: TNS = 4, Mt =1, SI =1 0b1000: TNS = 4, Mt =1, SI =2 0b1001: TNS = 4, Mt =1, SI =3 0b1010: TNS = 4, Mt =1, SI =4 0b1011: TNS = 4, Mt =2, SI =1 0b1100: TNS = 4, Mt =2, SI =2 0b1101: TNS = 4, Mt =2, SI =3 0b1110: TNS = 4, Mt =3, SI =1 0b1111: TNS = 4, Mt =3, SI =2 |
| PF | 1 | Precoding flag for SU-MIMO when PMI is not signaled 0b0: Non-adaptive precoding 0b1: Adaptive precoding using the precoder of rank Mt of the AMS's choice |
| Reserved | 3 |  |
| } |  |  |
| else { |  | PMI signaled explicitly |
| if (CSM == 0b0){ |  | SU-MIMO with PMI |
| Mt | 2 | Number of streams in transmission (Mt <= Nt ), up to 4 streams per AMS supported. Nt is the number of transmit antennas at the AMS. 0b00: 1 stream 0b01: 2 streams 0b10: 3 streams 0b11: 4 streams |
| PMI | 6 | 4 bits PMI for Nt = 2, first 2 MSB set to 0 6 bits PMI for Nt = 4 |
| } |  |  |
| else { |  | CSM with PMI |
| CSM Format | 4 | For non CSM, modes, TNS = Mt. For CSM modes the following combinations apply for upto 4 streams 0b0000: TNS = 2, Mt=1, SI =1 0b0001: TNS = 2, Mt =1, SI =2 0b0010: TNS = 3, Mt =1, SI =1 0b0011: TNS = 3, Mt =1, SI =2 0b0100: TNS = 3, Mt =1, SI =3 0b0101: TNS = 3, Mt =2, SI =1 0b0110: TNS = 3, Mt =2, SI =2 0b0111: TNS = 4, Mt =1, SI =1 0b1000: TNS = 4, Mt =1, SI =2 0b1001: TNS = 4, Mt =1, SI =3 0b1010: TNS = 4, Mt =1, SI =4 0b1011: TNS = 4, Mt =2, SI =1 0b1100: TNS = 4, Mt =2, SI =2 0b1101: TNS = 4, Mt =2, SI =3 0b1110: TNS = 4, Mt =3, SI =1 0b1111: TNS = 4, Mt =3, SI =2 |
| PMI | 4 | 4 bits PMI |
| } |  |  |
| } |  |  |
| Resource Index | 11 | 5 MHz: 0 in first 2 MSB bits + 9 bits for resource index 10 MHz: 11 bits for resource index 20 MHz: 11 bits for resource index Resource index includes location and allocation size |
| Long TTI Indicator | 1 | Indicates number of subframes spanned by the allocated resource. 0b0: 1 subframe (default) 0b1: 4 UL subframes for FDD or all UL subframes for TDD If number of DL subframes, Dis less than number of UL subframes, U, Long TTI Indicator= 0b1 |
| HFA | 3 | HARQ Feedback Allocation |
| AI_SN | 1 | HARQ identifier sequence number |
| ACID | 4 | HARQ channel identifier |
| Reserved | 1 | Reserved bit |
| } | - | - |

*A 16 bit CRC is generated based on the contents of the UL Basic Assignment A-MAP IE. The CRC is masked by the Station ID.

Table 822 UL basic assignment A-MAP IE*

| Syntax | Size in bits | Description/Notes |
| --- | --- | --- |
| UL_Basic_Assignment_A-MAP_IE() { | - | - |
| A-MAP IE Type | 4 | UL Basic Assignment A-MAP IE |
| ISizeOffset | 5 | Offset used to compute burst size index |
| Mt | 2 | Number of streams in transmission (Mt <= Nt ), up to 4 streams per AMS supported. Nt is the number of transmit antennas at the AMS. 0b00: 1 stream 0b01: 2 streams 0b10: 3 streams 0b11: 4 streams |
| SU-PMI Indicator | 1 | Flag to indicate if both SU-MIMO and PMI are signaled. 0b0: SU MIMO without PMI indication, or CSM without PMI indication 0b1: SU MIMO with PMI indication |
| if (SU-PMI Indicator == 0b0){ |  |  |
| CSM | 1 | Flag to indicate CSM 0b0: SU-MIMO 0b1: CSM |
| if (CSM == 0b0){ |  |  |
| MEF | 1 | MIMO encoder format - 0b0: SFBC - 0b1: Vertical encoding Non-adaptive precoding shall be used at the AMS with SFBC. Mt shall be ignored with SFBC. |
| if (MEF == 0b0){ |  |  |
| Reserved | 4 | Reserved bits |
| else if (MEF == 0b1){ |  |  |
| PF | 1 | Precoding flag for SU-MIMO when PMI is not signaled 0b0: Non-adaptive precoding 0b1: Adaptive precoding using the precoder of rank Mt of the AMS's choice |
| Reserved | 4 3 | Reserved bits |
| } |  |  |
| }else if (CSM == 0b1){ |  | Parameters for CSM |
| PF | 1 | Precoding flag for CSM when PMI is not signaled 0b0: Non-adaptive precoding 0b1: Adaptive precoding using the precoder of rank Mt of the AMS's choice |
| TNS | 2 | - Total number of streams in the LRU for CSM 0b00: reserved 0b01: 2 streams 0b10: 3 streams 0b11: 4 streams |
| SI | 2 | First pilot index for CSM with TNS = 2 streams: 0b00, 0b01 First pilot index for CSM with TNS = 3,4: 0b00, 0b01, 0b10, 0b11 |
| } |  |  |
| }else if (SU-PMI Indicator == 0b1){ |  | Parameters for SU-MIMO with PMI ( adaptive precoding) |
| PMI | 6 | 4 bits PMI for Nt = 2, first 2 MSB set to 0 6 bits PMI for Nt = 4 |
| } |  |  |
| Resource Index | 11 | 5 MHz: 0 in first 2 MSB bits + 9 bits for resource index 10 MHz: 11 bits for resource index 20 MHz: 11 bits for resource index Resource index includes location and allocation size |
| Long TTI Indicator | 1 | Indicates number of subframes spanned by the allocated resource. 0b0: 1 subframe (default) 0b1: 4 UL subframes for FDD or all UL subframes for TDD If number of DL subframes, Dis less than number of UL subframes, U, Long TTI Indicator= 0b1 |
| HFA | 3 | HARQ Feedback Allocation |
| AI_SN | 1 | HARQ identifier sequence number |
| ACID | 34 | HARQ channel identifier |
| Reserved | 12 | Reserved bit |
| } | - | - |

*A 16 bit CRC is generated based on the contents of the UL Basic Assignment A-MAP IE. The CRC is masked by the Station ID.

_________________________________End proposed text change #3_________________________________

________________________________Begin proposed text change #4_________________________________

Page 525, starting on Line 20

# 16.3.6.5.2.4.14 UL CSM beamforming A-MAP IE

Table 840 describes the fields in a UL CSM Beamforming Assignment A-MAP IE.

Table 840 UL CSM Beamforming A-MAP IE*

| Syntax | Size in bits | Description/Notes |
| --- | --- | --- |
| UL_CSM_Beamforming_A-MAP_IE() { | - | - |
| A-MAP IE Type | 4 | UL CSM_Beamforming A-MAP IE |
| ISizeOffset | 5 | Offset used to compute burst size index |
| CSM Format | 4 | For non CSM, modes, TNS = Mt. For CSM modes the following combinations apply for upto 4 streams 0b0000: TNS = 2, Mt=1, SI =1 0b0001: TNS = 2, Mt =1, SI =2 0b0010: TNS = 3, Mt =1, SI =1 0b0011: TNS = 3, Mt =1, SI =2 0b0100: TNS = 3, Mt =1, SI =3 0b0101: TNS = 3, Mt =2, SI =1 0b0110: TNS = 3, Mt =2, SI =2 0b0111: TNS = 4, Mt =1, SI =1 0b1000: TNS = 4, Mt =1, SI =2 0b1001: TNS = 4, Mt =1, SI =3 0b1010: TNS = 4, Mt =1, SI =4 0b1011: TNS = 4, Mt =2, SI =1 0b1100: TNS = 4, Mt =2, SI =2 0b1101: TNS = 4, Mt =2, SI =3 0b1110: TNS = 4, Mt =3, SI =1 0b1111: TNS = 4, Mt =3, SI =2 |
| PMI | 6 | 4 bits PMI for Nt = 2, first 2 MSB set to 0 6 bits PMI for Nt = 4 |
| Resource Index | 11 | 5 MHz: 0 in first 2 MSB bits + 9 bits for resource index 10 MHz: 11 bits for resource index 20 MHz: 11 bits for resource index Resource index includes location and allocation size |
| Long TTI Indicator | 1 | Indicates number of subframes spanned by the allocated resource. 0b0: 1 subframe (default) 0b1: 4 UL subframes for FDD or all UL subframes for TDD If number of DL subframes, Dis less than number of UL subframes, U, Long TTI Indicator= 0b1 |
| HFA | 3 | HARQ Feedback Allocation |
| AI_SN | 1 | HARQ identifier sequence number |
| ACID | 4 | HARQ channel identifier |
| Reserved | 1 | Reserved bit |
| } | - | - |

*A 16 bit CRC is generated based on the contents of the UL CSM Beamforming A-MAP IE. The CRC is masked by the Station ID.

The Resource Index field in the UL CSM Beamforming A-MAP IE is interpreted as in the DL Basic Assignment

A-MAP IE, with 'DL' specific terminology replaced by 'UL' equivalents.

________________________________End proposed text change #4_________________________________