| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Uplink pilot pattern structures for type-2 and type-3 subframes (Section 16.3.8 of the IEEE 802.16m/D4) |  |
| Date Submitted | 2010-03-05 |  |
| Source(s) | Alexei Davydov Jong-kae Fwu Intel Corporation | alexei.davydov@intel.com |
| Re: | Category: LB31 / Area: Chapter 16.3.8 |  |
| Abstract | The contribution uplink pilot patterns for type-2 and type-3 subframe |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m Advanced Air Interface (Draft 5) |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Uplink pilot pattern structures for for type-2 and type-3 subframe (Section 16.3.8 of the IEEE 802.16m/D4)

Alexei Davydov, Jong-kae Fwu

Intel Corporation

# Problem Statement

Uplink pilot patterns for type-3 subframe (5 symbols) and type-2 subframe (7 symbols) are not defined for CRU allocations (N-LRU and S-LRU). Although type-3 subframe is not explicitly used in frame configuration, it is required to support sounding operation or relay zone with transmission gap in uplink type-1 subframe. In this contribution we propose to reuse the CRU pilot patterns designed for type-2 and type-3 downlink subframes for CRU pilot patterns for uplink type-2 and type-3 subframes.

# Text proposal for inclusion in the 802.16m/D5

--------------------------------------------------  Text Start #1  ------------------------------------------------

{Add text on page 603, line 26, section 16.3.8.4 as follows}

The 1 stream uplink pilot patterns of type-2 and type-3 AAI subframes is the same as 1 stream downlink pilot patterns of type-2 and type-3 AAI subframes with pilot pattern set 0 and stream set 0 defined in section 16.3.5.4.1

--------------------------------------------------  Text End #1 ------------------------------------------------

--------------------------------------------------  Text Start #2  ------------------------------------------------

{Add text on page 603, line 55, section 16.3.8.4 as follows}

The 2 streams uplink pilot patterns of type-2 and type-3 AAI subframes is the same as 2 stream downlink pilot patterns of type-2 and type-3 AAI subframes with pilot pattern set 0 defined in section 16.3.5.4.1

--------------------------------------------------  Text End #2 ------------------------------------------------

--------------------------------------------------  Text Start #3  ------------------------------------------------

{Add text on page 604, line 63, section 16.3.8.4 as follows}

The uplink pilot patterns of type-2 and type-3 AAI subframe for 3 and 4 streams are the same as downlink pilot patterns of type-2 and type-3 AAI subframe for 3 and 4 streams defined in section 16.3.5.4.1

--------------------------------------------------  Text End #3 ------------------------------------------------