| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Global values in 16m (10.1) |  |
| Date Submitted | 2010-03-18 |  |
| Source(s) | Hyunjeong Kang, Kyungkyu Kim, Jaehyuk Jang, Yeongmoon Son, Youngkyo Baek, Youngbin Chang Samsung Electronics Co., Ltd. | hyunjeong.kang@samsung.com +82-31-279-5084 |
| Re: | IEEE 802.16-10/0011: “IEEE 802.16 Working Group Letter Ballot #31: Announcement” |  |
| Abstract | The contribution proposes global values of timers/parameters in 16m base station and 16m mobile station. |  |
| Purpose | To be discussed and adopted by WG LB |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Global Values in 16m

Hyunjeong Kang, Kyungkyu Kim, Jaehyuk Jang,

Yeongmoon Son, Youngkyo Baek, Youngbin Chang

Samsung Electronics Co., Ltd.

- Introduction

In current 16m specification, we have timers and parameters that are globally defined for 16m base stations and 16m mobile stations. The timers and parameters should be presented in subclause 10.1 Global Values and the values of the timers/parameters (i.e., default value, minimum value, maximum value) should be met by the 16m base stations and 16m mobile stations.

This contribution proposes a list of timers/parameters to be inserted in subclause 10.1 Global values.

- Text change

-------------------------------  Text Start  ---------------------------------------------------

[Add the followings into Table 554 on line 12 of page 31:]

| System | Name | Time Reference | Minimum Value | Default Value | Maximum Value |
| --- | --- | --- | --- | --- | --- |
| AMS | Contention Ranging Retries | Number of retries on contention Ranging Requests by ranging preamble code | 16 | - | - |
| AMS | Ranging Request Retries | Number of retries on ranging requests by AAI_RNG-REQ messages | 3 | - | 16 |
| AMS | Request Retries | Number of retries on bandwidth allocation requests | 16 | - | - |
| AMS | Registration Request Retries | Number of retries on registration requests | 3 | - | - |
| ABS, AMS | DSx Request Retries | Number of Timeout Retries on AAI_DSA/DSC/DSD Requests | - | 3 | - |
| ABS, AMS | DSx Response Retries | Number of Timeout Retries on AAI_DSA/DSC/DSD Responses | - | 3 | - |
| AMS | T2 | Wait for broadcast ranging timeout | - | - | - |
| AMS | T3 | Ranging response reception timeout following the transmission of a ranging request or UL BW allocation timeout following the transmission of a fragment of ranging request. | - | - | - |
| AMS | Tranging_code_response | Ranging response reception timeout following the transmission of a ranging preamble code | - | - | - |
| AMS | T4 | Time interval between periodic ranging | - | - | 35s |
| AMS | T6 | Wait for registration response | - | - | 3s |
| ABS, AMS | T7 | Wait for AAI_DSA/DSC/DSD Response timeout | 10ms | - | 1s |
| ABS, AMS | T8 | Wait for AAI_DSA/DSC Acknowledge timeout | 10ms | - | 300ms |
| ABS | T9 | The time allowed between the ABS sending a AAI_RNG-RSP (include temp STID) to an AMS, and receiving an AAI_SBC-REQ from that same AMS | 300ms | 300ms | - |
| ABS, AMS | T10 | Wait for Transaction End timeout | 600ms | - | 3s |
| AMS | T14 | Wait for AAI_DSX-RVD Timeout | - | - | 100ms |
| ABS | T17 | Time allowed for AMS to complete AMS Authorization and Key Exchange |  | 1min | - |
| AMS | T18 | Wait for AAI_SBC-RSP timeout | - | 50ms | <<T9 |
| ABS, AMS | T22 | Wait for AAI_ARQ-Reset | - | - | 0.5s |
| AMS | SBC Request Retries | Number of retries on SBC Request | 3 | 3 | 16 |
| ABS | Ranging Correction Retries | Number of Ranging Correction Retries | - | 16 | 2.5ms |
| AMS | RNG-RSP Processing Time | Time allowed for an AMS following receipt of an AAI_RNG-RSP before it is expected to apply the corrections instructed by the ABS Minimum value. | - | - | - |
| AMS, ABS | MIN_Sleep_Cycle | Minimum size of Sleep Cycle | >= Listening Window | - | 16frames |
| AMS, ABS | MAX_Sleep_Cycle | Maximum size of Sleep Cycle | >= MIN_Sleep_Cycle | - | 1024frames |
| AMS, ABS | Listening Window | The time duration during which the AMS, after waking up and synchronizing with the DL transmissions, can demodulate DL transmissions and decide whether to stay awake or go back to sleep. | 1 frame | - | 16 frames |
| ABS | AAI_NBR_ADV Interval | Nominal time between transmission of AAI_NBR-ADV messages. | - | - | 30s |
| AMS | Idle Mode Timer | AMS timed interval to conduct location update. Timer recycles on successful idle mode location update. | 128s | 4096s | - |
| AMS | T43 | Time the AMS waits for AAI_SLP-RSP or SCH/SCEH | - | - | - |
| AMS | T44 | Time the AMS waits for AAI_SCN-RSP. | - | - | - |
| AMS | T45 | Time the AMS waits for AAI_DREG-RSP | - | 250ms | 500ms |
| ABS | Management Resource Holding Timer | Time the ABS maintain connection information with the AMS after the ABS send AAI_DREG-RSP message to the AMS. | - | 500ms | 1s |
| AMS | DREG Request Retry Count | Number of retries on DREG Request message | 3 | 3 | 16 |
| ABS | DREG CMD Retry Count | Number of retries on DREG Response message. | 3 | 3 | 16 |
| ABS | T46 | Time the ABS waits for AAI_DREG-REQ in case of unsolicited idle mode initiation from ABS | 50ms | - | - |
| AMS | Max Dir Scan Time | Maximum scanning time of neighbor BSs by AMS before reporting any results. | - | - | - |
| ABS, AMS | Key agreement MSG#1 Timer | Time prior to resend of Key agreement MSG#1 | 0.5s | 1s | 2.0s |
| ABS, AMS | Key agreement MSG#1 MAX Resends | Maximum number of transmissions of Key agreement MSG#1. | 1 | 3 | 3 |
| ABS, AMS | Key agreement MSG#2 Timer | Time prior to resend of Key agreement MSG#2. | 0.1s | 0.3s | 1.0s |
| ABS, AMS | Key agreement MSG#2 MAX Resend | Maximum number of transmissions of Key agreement MSG#2. | 1 | 3 | 3 |
| AMS | Fresh Key Agreement Timer | Time prior to resend of refresh-PMK |  | 10s |  |
| ABS | AAI_LBS-ADV Interval | Nominal time between transmission of AAI_LBS-ADV messages. | 2sec | 10sec | 1800sec |
| AMS | Nms_max_neighbors | Maximum size of neighbor list. | 32 | - | 255 |
| ABS | AAI_SCD Interval | Nominal time between transmission of AAI_SCD messages | - | - | - |
| ABS | AAI_MC-ADV Interval | Nominal time between transmission of AAI_MC-ADV messages | - | - | - |
| AMS | T_AMS | This timer is for Listening Window Extension of AMS | 0 frame | - | 31 frames |
| ABS | T_ABS | This timer is for Listening Window Extension of ABS | >= T_AMS | - | - |
| ABS | AAI_SII-ADV Interval | Nominal time between transmission of AAI_SII-ADV messages | - | - | - |
| ABS | T58 | Minimum duration for ABS to wait for AAI_RNG-CFM message | - | - | - |
| ABS, AMS | BR ACK offset | BR ACK A-MAP IE reception time offset from the transmission of a BR preamble sequence. BR ACK offset shall be less than BR timer | - | - | - |
| AMS | BR timer | UL allocation waiting time for BR header after receiving a positive acknowledgement for a BR preamble sequence and a negative acknowledgement for quick access message | - | - | - |

-------------------------------  Text End  ---------------------------------------------------