| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clean up MAC control messages for Idle mode (16.2.3.21) |  |
| Date Submitted | 2010-03-17 |  |
| Source(s) | Hyunjeong Kang, Youngbin Chang Samsung Electronics Co., Ltd. | hyunjeong.kang@samsung.com +82-31-279-5084 |
| Re: | IEEE 802.16-10/0011: “IEEE 802.16 Working Group Letter Ballot #31: Announcement” |  |
| Abstract | The contribution proposes a template for table and ASN.1 format of MAC control messages for idle mode operation. |  |
| Purpose | To be discussed and adopted by WG LB |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clean up MAC control messages for Idle mode

Hyunjeong Kang, Youngbin Chang

Samsung Electronics Co., Ltd.

- Introduction

MAC control messages in D4 are defined in various formats (e.g. ASN.1 TLV, tables, lists) that do not provide sufficient information to proceed ASN.1 conversion. This contribution proposes a template to standardize the message definition format and make sure all attributes contain complete information, such as size, value, conditions.

Each table contains the following fields

- Table x: message name

- M/O: indicate if such attribute is mandatory or optional. If it is optional, then the condition for such attribute to be present should be included in the Conditions field.

- Attribute / Array of attributes: indicate the attribute or an array of attributes.

  - [8]: indicate it is a fix size array. Its dimension is 8

  - [1..8]: indicate it is a variable size array. Its dimension can be 1 to 8.

- Value / Note: indicate the value of the attribute. Additional description can be added to the note

- Conditions: Conditions for optional attributes to be present

Here, we propose a table template and ASN.1 format for MAC control messages to be used for idle mode operation.

- Text change

-------------------------------  Text Start  ---------------------------------------------------

[Remedy 1:]

[Add the following table on line 34 of page 101 in section 16.2.3.21:]

Table x AAI_DREG-REQ message

| M/O | Attributes / Array of attributes | Size(bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| M | De-registration_Request_Code | 3 | Used to indicate the purpose of this message 0x00: AMS deregistration request from ABS and network 0x01: request for AMS deregistration from serving ABS and initiation of AMS idle mode. 0x02: response for the unsolicited AAI_DREG-RSP message with action code 0x05 by the ABS. 0x03: reject for the unsolicited AAI_DREG-RSP message with action code 0x05 by the ABS. This code is applicable only when an AMS has a pending UL data to transmit. 0x04: request for AMS deregistration from serving ABS to enter DCR mode 0x05: response for the unsolicited AAI_DREG-RSP message with action code 0x00, 0x01, 0x02 or 0x03 0x06-0x07: reserved | NA |
| O | Paging cycle request | 4 | Used to indicate Paging cycle recommended by MS 0b0000 – 0b1111: reserved | Shall be present when De-registration_Request_Code of AAI_DREG-REQ message is set to 0x01 |
| O | Idle Mode Retain Information element | 4 | Provided as part of this message indicative only. Network reentry from idle mode process requirements may change at time of actual reentry. For each bit location, a value of 0 indicates the information for the associated reentry control messages shall not be retained and managed; a value of 1 indicates the information for the associated reentry control message shall be retained and managed. Bit 0: Retain MS service and operational information associated with AAI_SBC-REQ/RSP messages. Bit 1: Retain MS service and operational information associated with AAI_PKM-REQ/RSP messages. Bit 2: Retain MS service and operational information associated with AAI_REG-REQ/RSP messages. Bit 3: Retain MS service and operational information associated with network address. Bit 4: Retain MS state information. The information retained by setting bit 4 does not include the information associated with AAI_SBC-REQ/RSP messages, AAI_PKM-REQ/RSP messages, AAI_REG-REQ/RSP messages, network address unless otherwise specified by setting one or more bits 0–3. This information does not include blocks currently in the ARQ window or associated timers. The MS state information does include SFIDs and related description (QoS descriptors and CS classifier information) for all Service Flows that the AMS has currently established as well as any SAs with their related keying information. | Shall be Present when De-registration_Request_Code is set to 0x01 or 0x04 |
| O | Mobility information | 2 | Used to indicate MS’s mobility level 0b00: Fast 0b01: Medium 0b10: Slow 0b11: reserved | May be present when De-registration_Request_Code is set to 0x01 |

[Remedy 2:]

[Add the following table on line 14 of page 102 in section 16.2.3.22:]

Table x AAI_DREG-RSP message

| M/O | Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| M | Action code | 4 | Used to indicate the purpose of this message 0x00: AMS shall immediately terminate service with the ABS and should attempt network entry at another ABS 0x01: AMS shall listen to the current ABS but shall not transmit until a RES-CMD message or AAI_DREG-RSP message with action code 0x02 or 0x03 is received. 0x02: AMS shall listen to the current ABS but only transmit on the control connection. 0x03: AMS shall return to normal operation and may transmit on any of its active connections. 0x04: This option is valid in response to a AAI_DREG-REQ message with De-Registration Request Code=0x00. The AMS shall terminate current Normal Operation with the ABS. 0x05: AMS shall begin idle mode initiation: a) to signal AMS to begin idle mode in unsolicited manner or b) to allow AMS to transmit AMS-initiated idle mode request at the REQ-Duration expiration 0x06: This option is valid only in response to a AAI_DREG-REQ message with De-Registration Code 0x01: a) to reject AMS-initiated idle mode request or b) to allow AMS to transmit AMS-initiated idle mode request at the REQ-Duration expiration 0x07: This option is valid in response to a AAI_DREG-REQ message with De-registration-request-code= 0x01. 0x08: This option is valid only in response to an AAI_DREG-REQ message with De-Registration Request Code 0x04 to allow retention of the AMS’s connection information 0x09: This option is valid only in response to an AAI_DREG-REQ message with De-Registration Request Code 0x04 to reject retention of the AMS’s connection information. 0x10-0x15: reserved | NA |
| O | Paging cycle | 4 | Used to indicate Paging cycle for the AMS 0x00: 8 superframes 0x01: 16 superframes 0x02: 32 superframes 0x03-0x15: reserved | Shall be present when Action code in AAI_DREG-RSP message is set to 0x05 or 0x07 |
| O | Paging offset | - | Used to indicate Paging offset for the AMS | Shall be present when Action code in AAI_DREG-RSP message is set to 0x05 or 0x07 |
| O | Paging controller ID | 48 | Used to indicate Paging controller which manages and retains the MS’s idle mode information 0..248-1 | Shall be present when Action code in AAI_DREG-RSP message is set to 0x05 or 0x07 |
| O | Paging group ID | 16 | Used to indicate Paging group which the MS is located in 0..216-1 | Shall be present when Action code in AAI_DREG-RSP message is set to 0x05 or 0x07 |
| O | Deregistration ID | 10 | Used to indicate Deregistration ID used to identify the MS in idle mode 0..210-1 | Shall be present when Action code in AAI_DREG-RSP message is set to 0x05 or 0x07 |
| O | Idle Mode Retain Information element | 4 | Provided as part of this message indicative only. Network reentry from idle mode process requirements may change at time of actual reentry. For each bit location, a value of 0 indicates the information for the associated reentry control messages shall not be retained and managed; a value of 1 indicates the information for the associated reentry control message shall be retained and managed. Bit 0: Retain MS service and operational information associated with AAI_SBC-REQ/RSP messages. Bit 1: Retain MS service and operational information associated with AAI_PKM-REQ/RSP messages. Bit 2: Retain MS service and operational information associated with AAI_REG-REQ/RSP messages. Bit 3: Retain MS service and operational information associated with network address. Bit 4: Retain MS state information. The information retained by setting bit 4 does not include the information associated with AAI_SBC-REQ/RSP messages, AAI_PKM-REQ/RSP messages, AAI_REG-REQ/RSP messages, network address unless otherwise specified by setting one or more bits 0–3. This information does not include blocks currently in the ARQ window or associated timers. The MS state information does include SFIDs and related description (QoS descriptors and CS classifier information) for all Service Flows that the AMS has currently established as well as any SAs with their related keying information. | Shall be Present when Action code in AAI_DREG-RSP message is set to 0x05 or 0x07 or 0x08 |
| O | REQ-Duration | 8 | Used to indicate waiting value for the AAI_DREG-REQ message with De-registration_Request_Code=0x01 0..28-1 : measured in frames | May be present when Action code is set to 0x05 |

[Remedy 3:]

[Make a new subclause under section 16.2.3 and move the table 767 on line 18 of page 314 in section 16.2.17.2.1 into the new subclause, and replace the Table 767 as follows:]

16.2.3.54 AAI_PAG-ADV (paging advertisement) Message

Table 767 – AAI_PAG-ADV message

| M/O | Attributes / Array of attributes |  | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- | --- |
| M | Paging_Group_IDs bitmap |  | - | Indicate that the paging information for the corresponding paging group is included in AAI_PAG-ADV message The length of Paging_Group_IDs bitmap is same as Num_PGIDs in PGID_Info message. 0: the paging information for the corresponding PGID is not included 1: the paging information for the corresponding PGID is included | NA |
| O | Num_AMSs |  | 5 | Indicate the number of paged MSs in a corresponding paging group 1..32 | Shall be present when Paging_Group_IDs bitmap of a corresponding PGID is set to 1 |
| M | Paged MS Info[1..32] | Deregistration Identifier | 10 | Used to indicate Deregistration ID for the MS to be paged 0..210-1 | Shall be present unless Num_AMSs is 0 |
| M |  | Paging Cycle | 4 | Used to indicate Paging cycle for the AMS to be paged 0x00: 8 superframes 0x01: 16 superframes 0x02: 32 superframes 0x03-0x0F: reserved | Shall be present unless Num_AMSs is 0 |
| M |  | Action Code | 1 | Used to indicate the purpose of the AAI_PAG-ADV message 0b0: perform network reentry 0b1: perform ranging for location update | Shall be present unless Num_AMSs is 0 |
| M | Extension Flag |  | 1 | Used to indicate the remaining part of the AAI_PAG-ADV message exists 0b0: there is no remaining part of the AAI_PAG-ADV message 0b1: the remaining part of the AAI_PAG-ADV message will be transmitted in the subsequent subframe or frame | NA |
| M | Emergency Alert Indication |  | 1 | Used to indicate the presence of emergency information 0b0: there is no emergency information 0b1: there is emergency information | NA |

[Remedy 4:]

[Make a new subclause under section 16.2.3 and move the table 768 on line 40 of page 316 in section 16.2.17.2.3 into the new subclause, and replace the Table 768 as follows:]

16.2.3.55 PGID_Info (paging group information) Message

Table 768 – PGID_Info message

| M/O | Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| M | Num_PGIDs | 2 | Indicate the number of PGID included in PGID_Info message 1..4 | NA |
| M | PGID | 16 | Indicate Paging group identifier where the BS belongs 0..216-1 | NA |
| M | m | 2 | Indicate m in the equation Npaging frame = AMS’s deregistration identifier mod m to determine a predefined paging frame for the MS 1..4 | NA |
| O | Paging carrier indication bit | Number of carriers in AAI_Global-Config | This bitmap is used to specify if carriers are a paging carrier in the ABS or not. The size of paging carrier indication bitmap equals to the number of carriers in the AAI_Global_Config message. The physical carrier index of each bit in bitmap corresponds to the ascending order of physical carrier index of the carriers in the AAI_Global_Config message. The value N used to determine the paging carrier index is equal to the number of bits set to 1. | Present only for multiple carrier operation |

[Remedy 5:]

[Add the followings after line 21 of page 763:]

Annex P.2.x AAI_DREG-REQ message

DREGREQ DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- DeRegistration Request Message

AAI-DREG-REQ ::= SEQUENCE {

messageType         OCTET STRING(SIZE(1)),

-- DeRegistration Request Code

deRegReqCode        ENUMERATED {

code0(0), -- AMS deregistration request from ABS and network

code1(1), -- AMS deregistration from serving ABS and initiation of AMS idle mode.

code2(2), -- Response for the unsolicited AAI_DREG-RSP message with action code 0x05 by the ABS.

code3(3), -- Reject for the unsolicited AAI_DREG-RSP message with action code 0x05 by the ABS.

code4(4), -- AMS deregistration from serving ABS to enter DCR mode

code5(5), -- response for the unsolicited AAI_DREG-RSP message with action code 0x00, 0x01, 0x02 or 0x03

code6(6), -- Reserved

code7(7)  -- Reserved

},

-- Paging Cycle Request

pgCyclReq           INTEGER (0 .. 15) OPTIONAL,

-- Idle Mode Retain IE

idleModeRetainIe  BIT STRING {

b0(0), -- Info associated with SBC msgs

b1(1), -- Info associated with PKM msgs

b2(2), -- Info associated with REG msgs

b3(3), -- Info associated with NW addr

b4(4)  -- MS state info

} SIZE (5) OPTIONAL,

-- Mobility Info

mobilityInfo         ENUMERATED {

fast(0),

medium(1),

slow(2),

reserved(3)

}OPTIONAL

}

END

Annex P.2.(x+1) AAI_DREG-RSP message

DREGRSP DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- DeRegistration Response Message

AAI-DREG-RSP ::= SEQUENCE {

messageType         OCTET STRING(SIZE(1)),

-- Action Code - Indicates the purpose of the message

actionCode          ENUMERATED {

rspActionCode0(0),

rspActionCode1(1),

rspActionCode2(2),

rspActionCode3(3),

rspActionCode4(4),

rspActionCode5(5),

rspActionCode6(6),

rspActionCode7(7),

rspActionCode8(8),

rspActionCode9(9)

},

-- Paging Info

pgInfo              PagingInfo OPTIONAL,

-- DeRegistration ID

deRegId             BIT STRING SIZE(10) OPTIONAL,

-- Idle Mode Retain IE

idleModeRetainIe    BIT STRING {

b0(0), -- Info associated with SBC msgs

b1(1), -- Info associated with PKM msgs

b2(2), -- Info associated with REG msgs

b3(3), -- Info associated with NW addr

b4(4)  -- MS state info

} SIZE (5) OPTIONAL,

-- Request Duration

reqDuration          INTEGER (0 .. 255) OPTIONAL

}

PagingInfo ::= SEQUENCE {

-- Paging Cycle

pgCycl              ENUMERATED {

cycle08(0),

cycle16(1),

cycle32(2),

cycle0(3),

cycle1(4),

cycle2(5),

cycle3(6),

cycle4(7),

cycle5(8),

cycle6(9),

cycle7(10),

cycle8(11),

cycle9(12),

cycle10(13),

cycle11(14),

cycle12(15)

},

-- Paging Offset

pgOffSet            INTEGER (..),

-- Paging Controller ID

pgCtrlrId            OCTET STRING SIZE(6),

-- Paging Group ID

pgGrpId              INTEGER (0 .. 65535)

}

END

Annex P.2.(x+2) AAI_PAG-ADV message

PAGADV DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- Paging Advertisement Message

AAI-PAG-ADV ::= SEQUENCE {

messageType         OCTET STRING(SIZE(1)),

-- Paging Information for the Paging Group

-- Indicate whether the paged MS(s) exist in a paging group

pgIdBmp             BIT STRING (SIZE (NumPGID)),

-- Number of MSs to be paged in a paging group in the AAI_PAG-ADV

numMS               NumMS OPTIONAL,

-- MS Info to be paged

pagedMSList        PagedMSInfoLst OPTIONAL,

-- Indication of remaining AAI_PAG-ADV

extensionFlag      BIT STRING SIZE(1),

-- Indication of emergency alert info

emergencyAlert     BIT STRING SIZE(1)

}

-- Number of PGID in the PGID_Info message

NumPGID ::= INTEGER (1..4)

-- Number of MSs to be paged in a paging group in the AAI_PAG-ADV

NumMS ::= INTEGER (1..32)

-- the information of MSs to be paged in the AAI_PAG-ADV

PagedMSInfoLst ::= SEQUENCE (SIZE (NumMS)) OF PagedMSInfo

PagedMSInfo ::= SEQUENCE {

-- Paged MS DeReg ID

deRegId            BIT STRING (SIZE(10)),

-- Paged MS paging Cycle

pagingCycle        INTEGER (0..15),

-- Perform action code 0: Nw entry, 1: Loc Upd

actionCode         BIT STRING SIZE(1)

}

END

Annex P.2.(x+3) PGID_Info message

PGIDINFO DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- PagingID Info Message

AAI-PGID-INFO ::= SEQUENCE {

messageType         OCTET STRING(SIZE(1)),

-- Number of Paging groups in the ABS

numPgId             INTEGER (1 .. 4),

-- Paging Identifier

pgId                BIT STRING SIZE(16),

-- Pre-determined paging frame identifier 'm'

m                   INTEGER (1 .. 4),

-- paging carrier indicator bitmap

pagingCarrierInd BIT STRING {

carrier1(1),

..

carriern(n)

} SIZE(number of carriers in AAI_Global-Config) OPTIONAL

}

END

-------------------------------  Text End  ---------------------------------------------------