| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clean up AAI_SCD message (16.2.3.29) |  |
| Date Submitted | 2010-03-17 |  |
| Source(s) | Hyunjeong Kang, Jaehyuk Jang, Youngbin Chang Samsung Electronics Co., Ltd. | hyunjeong.kang@samsung.com +82-31-279-5084 |
| Re: | IEEE 802.16-10/0011: “IEEE 802.16 Working Group Letter Ballot #31: Announcement” |  |
| Abstract | The contribution proposes a template for table and ASN.1 format of AAI_SCD message. |  |
| Purpose | To be discussed and adopted by WG LB |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clean up AAI_SCD message

Hyunjeong Kang, Jaehyuk Jang, Youngbin Chang

Samsung Electronics Co., Ltd.

- Introduction

MAC control messages in D4 are defined in various formats (e.g. ASN.1 TLV, tables, lists) that do not provide sufficient information to proceed ASN.1 conversion. This contribution proposes a template to standardize the message definition format and make sure all attributes contain complete information, such as size, value, conditions.

Each table contains the following fields

- Table x: message name

- M/O: indicate if such attribute is mandatory or optional. If it is optional, then the condition for such attribute to be present should be included in the Conditions field.

- Attribute / Array of attributes: indicate the attribute or an array of attributes.

  - [8]: indicate it is a fix size array. Its dimension is 8

  - [1..8]: indicate it is a variable size array. Its dimension can be 1 to 8.

- Value / Note: indicate the value of the attribute. Additional description can be added to the note

- Conditions: Conditions for optional attributes to be present

Here, we propose a table template and ASN.1 format for the AAI_SCD MAC control message.

- Text change

-------------------------------  Text Start  ---------------------------------------------------

[Remedy 1:]

[Add the following table after line 11 of page 115 in section 16.2.3.29:]

Table x AAI_SCD message

| M/O | Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| M | Change Configuration Count | 4 | The value is increased whenever the contents of this message is changed The value rolls over from 0 to 15 | NA |
| M | BS Restart Count | 4 | The value is incremented by one whenever BS restarts The value rolls over from 0 to 15 | N/A |
| M | SA Preamble Partition for BS type | TBD | Indicates the SA-Preamble partition information | N/A |
| M | Default Trigger Averaging Parameter |  | Provides default trigger averaging parameter of the BS 0 - 9 0x0: 1 0x1: 1/2 0x2: 1/4 0x3: 1/8 0x4: 1/16 0x5: 1/32 0x6: 1/64 0x7: 1/128 0x8: 1/256 0x9: 1/512 | Used in equation (1) |
| M | Normalized CINR | - | - | - |
| O | OL Region ON | 1 | 0 or 1 | OL MIMO parameters |
| O | OL Rank1 Configuration | 3 | 0 to 7 | OL MIMO parameters |
| O | SB OL Region 2 Size | 4 | 0 to 15 | OL MIMO parameters |
| M | T_ReTx_Interval |  | 0 – 8 |  |
| M | Sounding Multiplexing Type | 1 | 0: Decimation_valueD 1: Max_cycle_shift_indexP | Sounding sequence |
| O | Decimation_valueD | 3 | Decimation value for frequence division multiplexing 000: decValueD0 (=4) 001: decValueD1 (=6) 010: decValueD2 (=8) 011: decValueD3 (=9) 100: decValueD4 (=12) 101: decValueD5 (=16) 110: decValueD6 (=18) 111: decValueD7 (=36) | Sounding sequence: Presents when Sounding Multiplexing Type is set to Decimation_ValueD |
| O | Max_cycle_shift_indexP | 3 | Maximum cyclic shift for code division multiplexing 000: csIndexP0 (=4) 001: csIndexP1 (=6) 010: csIndexP2 (=8) 011: csIndexP3 (=9) 100: csIndexP4 (=12) 101: csIndexP5 (=16) 110: csIndexP6 (=18) 111: csIndexP7 (=36) | Sounding sequence: Presents when Sounding Multiplexing Type is set to Max_Cycle_Shift_IndexP |
| M | Shift value for sounding symbol | 8 | 0 – 255 |  |
| O | 16m Relay zone AMS allocation indicator | 1 | 0 – 1 | Relay support parameters |

-------------------------------  Text End  ---------------------------------------------------