| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clean up MAC control messages for multicarrier (16.2.3.51) |  |
| Date Submitted | 2010-03-17 |  |
| Source(s) | Hyunjeong Kang, Youngbin Chang Samsung Electronics Co., Ltd. | hyunjeong.kang@samsung.com +82-31-279-5084 |
| Re: | IEEE 802.16-10/0011: “IEEE 802.16 Working Group Letter Ballot #31: Announcement” |  |
| Abstract | The contribution proposes a table template and ASN.1 format of MAC control messages for multicarrier operation. |  |
| Purpose | To be discussed and adopted by WG LB |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clean up MAC control messages for multicarrier

Hyunjeong Kang, Youngbin Chang

Samsung Electronics Co., Ltd.

- Introduction

MAC control messages in D4 are defined in various formats (e.g. ASN.1 TLV, tables, lists) that do not provide sufficient information to proceed ASN.1 conversion. This contribution proposes a template to standardize the message definition format and make sure all attributes contain complete information, such as size, value, conditions.

Each table contains the following fields

- Table x: message name

- M/O: indicate if such attribute is mandatory or optional. If it is optional, then the condition for such attribute to be present should be included in the Conditions field.

- Attribute / Array of attributes: indicate the attribute or an array of attributes.

  - [8]: indicate it is a fix size array. Its dimension is 8

  - [1..8]: indicate it is a variable size array. Its dimension can be 1 to 8.

- Value / Note: indicate the value of the attribute. Additional description can be added to the note

- Conditions: Conditions for optional attributes to be present

Here, we propose a table template and ASN.1 format for MAC control messages to be used for multicarrier operation.

About AAI_MC-RSP message, the AAI_MC-RSP message is incorrectly implemented in 16m/D4, the AAI_MC-RSP message from the accepted contribution C802.16m-09/2897r3 (see comment #265 of La Jolla meeting) is proposed.

- Text change

-------------------------------  Text Start  ---------------------------------------------------

[Remedy 1:]

[Move the whole section 16.2.8.2.3.1.1 under section 16.2.3 and add the following table after Table 748- AAI_MC-ADV MAC Control Message Format:]

Table x Contents of AAI_MC-ADV message

| M/O | Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| M | Multicarrier configuration change count | 4 | The value is incremented by 1 upon each update |  |
| M | Serving BS carrier number | 3 | Indicates the number of carriers the ABS carriers |  |
| M | MAC protocol version | 8 | Indicates MAC protocol version that the current carrier supports: it is consistent with REV.2 definition with new MAC protocol version 9 defined for 16m |  |
| O | Physical carrier index | 6 | Indicates the carrier index that the ABS carries except current carrier: the physical carrier index refers to AAI_Global-Config message | Shall be present when Serving BS carrier number is more than 1 |
| O | SA-Preamble index | 10 | Indicates the SA-Preamble index of the carrier | Shall be present when Serving BS carrier number is more than 1 |
| O | SFH encoding format | 2 | 0b00: full SFH subpacket and AAI_SCD information 0b01: delta encoding of SFH_Info (SFH subpacket and AAI_SCD) with reference to the information of current carrier which transmits this AAI_MC-ADV message 0b10: delta encoding of SFH_Info (SFH subpacket and AAI_SCD) with reference to the information of preceding carrier 0b11: reserved | Shall be present when Serving BS carrier number is more than 1 |
| O | S-SFH change count | 4 | Change count of S-SFH SP IE(s) | Shall be present when Serving BS carrier number is more than 1 |
| O | Change count for AAI_SCD | 4 | Change configuration count of AAI_SCD | Shall be present when Serving BS carrier number is more than 1 |
| O | Control bitmap | 4 | Each bit maps to one SFH subpacket and AAI_SCD information Bit[0]: SFH SP1 Bit[1]: SFH SP2 Bit[2]: SFH SP3 Bit[3]: AAI_SCD | Shall be present when SFH encoding format is set to 0b00 |
| O | SFH subpacket 1 | Variable depending on FFT size | SFH subpacket 1 is included | Shall be present when SFH encoding format is set to 0b00 and the 1st MSB of control bitmap is set to 1 |
| O | SFH subpacket 2 | Variable depending on FFT size | SFH subpacket 2 is included | Shall be present when SFH encoding format is set to 0b00 and the 2nd MSB of control bitmap is set to 1 |
| O | SFH subpacket 3 | Variable | SFH subpacket 3 is included | Shall be present when SFH encoding format is set to 0b00 and the 3rd MSB of control bitmap is set to 1 |
| O | AAI_SCD_Info | Variable | Information from AAI_SCD is included | Shall be present when SFH encoding format is set to 0b00 and the 4th MSB of control bitmap is set to 1 |
| O | Delta information | Variable | Delta encoding w.r.t. the reference carrier (the current carrier transmitting this message if SFH_encoding_format = 0b01, the preceding carrier if SFH_encoding_format=0b10) | May be present when SFH encoding format is set to 0b01 or 0b10 |

[Remedy 2:]

[Move the section 16.2.8.2.11.5.1 under section 16.2.3 and add the following table after Table 749 – AAI_CM-CMD MAC Control Message Format:]

Table x Contents of AAI_CM-CMD message

| M/O | Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| M | Action code | 1 | Indicate the purpose of this message 0b0: secondary carrier management 0b1: primary carrier change | NA |
| O | Indication Type | 2 | Indicate the corresponding secondary carrier is activated or deactivated 00: no action 01: deactivation only 10: activation only 11: both activation and deactivation | Shall be present when Action code is set to 0b0 in AAI_CM-CMD message |
| O | Activation Deadline | 6 | LSB bits of Superframe number after the AAI_CM-CMD is sent for the AMS to confirm the activation of secondary carrier by sending the AAI_CM-IND message | Shall be present when Indication Type is set to 10 or 11 in AAI_CM-CMD message |
| O | Num of target carrier | 3 | Indicate the number of secondary carrier(s) to be activated or deactivated 1..8 | Shall be present when Indication Type is set to 01 or 10 or 11 in AAI_CM-CMD message |
| O | Target carrier index | 3 | Indicate the secondary carrier index to be activated or deactivated 1..8 | Shall be present when Indication Type is set to 01 or 10 or 11 in AAI_CM-CMD message |
| O | Activation of DL/UL | 1 | Indicate the activation of DL or UL in the corresponding secondary carrier 0b0: Both DL/UL of the secondary carrier are activated 0b1: DL of the secondary carrier is activated but UL of the secondary carrier is not activated | Shall be present when Indication Type is set to 10 or 11 in AAI_CM-CMD message |
| O | Ranging indicator | 1 | Indicate the periodic ranging is required for the carrier 0b0: no initial or periodic ranging is required for the carrier 0b1: periodic ranging is required for the carrier | Shall be present when Indication Type is set to 10 or 11 in AAI_CM-CMD message or shall be present when Action code is set to 0b1 in AAI_CM-CMD message |
| O | Deactivation of DL/UL | 1 | Indicate the deactivation of DL or UL in the corresponding secondary carrier 0b0: Both DL/UL of the secondary carrier are deactivated 0b1: UL of the secondary carrier is deactivated but DL of the secondary carrier is kept active | Shall be present when Indication Type is set to 01 or 11 in AAI_CM-CMD message |
| O | Physical carrier index of Target carrier | 6 | Indicate the physical carrier index of a target carrier for primary carrier change. If the MS supports multicarrier operation, the carrier shall be one of the assigned carriers 0..63 | Shall be present when Action code is set to 0b1 in AAI_CM-CMD message |
| O | Action Time | 3 | Indicate a time to switch to the target primary carrier. The value shall be set to the value more than the retransmission timer for AAI_CM-CMD message 0..7: LSB of superframe | Shall be present when Action code is set to 0b1 in AAI_CM-CMD message |
| O | Next state of serving primary carrier | 1 | Indicate the deactivation of serving primary carrier after primary carrier change. If the MS does not support carrier aggregation, this field shall be always set to 0b0. 0b0: serving primary carrier will be deactivated after primary carrier change. 0b1: serving primary carrier is kept active after primary carrier change | Shall be present when Action code is set to 0b1 in AAI_CM-CMD message |

[Remedy 3:]

[Move the whole section 16.2.8.2.11.5.2 under section 16.2.3 and add the following table after Table 750 – AAI_CM-IND MAC Control Message Format:]

Table x Contents of AAI_CM-IND message

| M/O | Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| M | Action code | 1 | Indicate the purpose of this message 0b0: secondary carrier management 0b1: primary carrier change | NA |

[Remedy 4:]

[Add the followings after line 21 of page 763:]

Annex P.2.z AAI_MC-ADV message

MCADV DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- MultiCarrier Advertisement Message

AAI-MC-ADV ::= SEQUENCE {

messageType          OCTET STRING SIZE(1),

--Multi-Carrier Configuration Count

multiCxrCCnt         INTEGER ( 1 .. 16),

--Multi-Carrier Configuration

mcCfg                McCfg

}

McCfg ::= SEQUENCE {

-- Serving BS Carrier Number

servBSCxrNum        INTEGER ( 0 .. 7 ),

macVer             MacVer,

-- BS Info List

bsInfo              BsInfoList

}

--BsCxrInfo 1 -> (bsCxrNumber-1)

BsInfoList ::= SEQUENCE (SIZE(1 .. 7)) OF BsCxrInfo

BsCxrInfo ::= SEQUENCE {

phyCxrIdx          PhyCxrIdx,

preambleIdx        INTEGER (0 .. 1023),

sfhCCC        INTEGER (0..15),

scdCCC        INTEGER (0..15),

-- SFH Encoding Format

sfhEncFmt        BIT STRING SIZE(2),

-- Control Bitmap

ctrlBmp          BIT STRING {

sfhSubPkt1(0),

sfhSubPkt2(1),

sfhSubPkt3(2),

sfhSubPkt4(3)

}SIZE(4) OPTIONAL,

-- SFH subpacket and AAI_SCD Information

sfhPkt1            SEQUENCE { …} OPTIONAL,

sfhPkt2            SEQUENCE {..} OPTIONAL,

sfhPkt3            SEQUENCE {..} OPTIONAL,

scdInfo            SEQUENCE {..} OPTIONAL,

-- Delta Information

deltaInfo        SEQUENCE { ... } OPTIONAL

}

MacVer ::= SEQUENCE {

verNumber          INTEGER ( 0 .. 255 )

}

PhyCxrIdx ::= INTEGER ( 0 .. 63 )

END

Annex P.2.(z+1) AAI_CM-CMD message

CMCMD DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- Carrier Management Command Message

AAI-CM-CMD ::= SEQUENCE {

messageType         OCTET STRING(SIZE(1)),

--Action Code indicating the purpose of the message

actionCode          ENUMERATED {

secCxrMgmt(0),

priCxrChng(1)

},

-- Indication Type

indicationType      ENUMERATED {

noAction(0),

deAct(1),

act(2),

actDeAct(3)

} OPTIONAL ,

-- Target Carrier List

tgtCxrMgmt          TgtCxrLst OPTIONAL,

-- Activation deadline for secondary carrier activation

activeDeadline     BIT STRING SIZE (6) OPTIONAL,

--Physical Carrier Index of Target Carrier

tgtPhyCxrIdx        PhyCxrIdx OPTIONAL,

-- Action Time

axnTime             BIT STRING SIZE (3) OPTIONAL,

-- Next State of serving primary carrier

-- serving carrier DeAct/Act after primary cxr chng

servCxrNxtState     CxrState OPTIONAL

}

--Carrier State

CxrState ::= ENUMERATED {

cxrDeAct(0),

cxrAct(1)

}

--Physical Carrier Index

PhyCxrIdx ::= INTEGER (0 .. 63)

-- Number of Target Carriers

NumTargetCarrier ::= INTEGER (1..8) -- Range TBD

-- Target Carrier List

TgtCxrLst ::= SEQUENCE (SIZE(NumTargetCarrier)) OF TgtCxrInfo

-- Target Carrier Index

TgtCxrIdx ::= INTEGER (1 .. 8)

--Activation-DeActivation Indication

ActDeActInd ::= BIT STRING SIZE(1)

-- Targer Carrier Info

TgtCxrInfo ::= SEQUENCE {

-- Target Carrier Index

tgtCxrIdx           TgtCxrIdx,

-- Activation of DL - UL

-- 0: Both DL/UL Activated 1: Only DL is Activated

dlulAct             ActDeActInd OPTIONAL,

-- DeActivation of DL - UL

-- 0: Both DL/UL DeActivated 1: Only UL is DeActivated

dlulDeAct           ActDeActInd OPTIONAL,

-- Ranging Indicator for Target Carrier

-- 0: No rng reqd 1: Periodic rng reqd

rngInd              BIT STRING SIZE(1)  OPTIONAL

}

END

Annex P.2.(z+2) AAI_CM-IND message

CMIND DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- Carrier Management Indication Message

AAI-CM-IND ::= SEQUENCE {

messageType         OCTET STRING(SIZE(1)),

--Action Code indicating the purpose of the message

actionCode          ENUMERATED {

secCxrMgmt(0),

priCxrChng(1)

}

}

END

-------------------------------  Text End  ---------------------------------------------------