| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Encoding format of SFH deltas in AAI_NBR-ADV (16.2.3) |  |
| Date Submitted | 2010-03-17 |  |
| Source(s) | Daniel Cohn, Vladimir Yanover Alvarion | daniel.cohn@alvarion.com |
| Re: | Letter Ballot #31 on the Draft Amendment (IEEE P802.16m/D4) |  |
| Abstract | Define how to encode neighbor SFH delta information in AAI_NBR-ADV message |  |
| Purpose | To be discussed and adopted by IEEE 802.16m TG |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Encoding format of SFH deltas in AAI_NBR-ADV (16.2.3)

Daniel Cohn, Vladimir Yanover
Alvarion

Problem Statement

Problem #1:

802.16m/D4 allows ABS to transmit SFH information of neighbor ABSs to connected AMSs in order to expedite network re-entry when the AMS performs handover to a neighbor ABS. To save overhead, the standard allows the ABS to transmit only differences between the neighbor ABS SFH and the first ABS of the same cell type (“delta encoding”) in the AAI_NBR-ADV message.

However, the delta encoding format is not specified in D4.

Problem #2:

In D4, the S-SFH message is partitioned into three subpackets (SPs) which contain different S-SFH elements. SP1 includes elements required for network re-entry, SP2 includes elements required for network entry and discovery and SP3 contains all the remaining elements. The SPs are periodically transmitted by the ABS, with different period length (e.g. 40 ms, 80 ms and 160 ms for SP1, SP2 and SP3 respectively as shown in the figure).

S-SFH SPs are transmitted in the first subframe within a superframe, together with the P-SFH. P-SFH and S-SFH can occupy up to 24 DLRUs (Distributed Logical Resource Units) in the subframe.  This motivation for this limitation is to ensure the SFH can be transmitted in a single subframe, even for the lowest channel bandwidth supported (5 MHz), in a coding rate that provides reasonable coverage.

The SFH size limitation imposes a rigid restriction on the size of the SPs. This limitation makes SP design difficult and in particular precludes future extension of the SPs (e.g. to support additional features in later standard revisions).

Suggested Solution

Remedy #1:

802.16m uses ASN.1 format to encode MAC messages. In this particular case, however, the use of ASN.1 format would result in the transmission of a very large bitmap indicating which delta SFH parameters are being transmitted. Since typically SFH differences among ABSs in a deployment are minor, TLV format is much more efficient than the native ASN.1 format. Therefore, a TLV format is suggested for delta SFH encoding.

It is also suggested that dynamic SFH fields (such as MSB of superframe number) are not transmitted in the AAI_NBR-ADV message.

It should be noted that the proposed encoding is also used by the MC-ADV message.

Remedy #2:

A new S-SFH structure is proposed, based on the concept of S-SFH containers. An S-SFH channel is a container for SP fragments, where for example S-SFH channel #1 can carry data currently associated with SP1. S-SFH channels are transmitted periodically with the same period length defined in D4 for the different SPs. When SDUs within S-SFH channels are fragmented, fragments are transmitted in consecutive superframes. The advantage of the proposed structure is that SP size is no longer limited by the capacity of a single subframe and therefore SPs can be extended without this limitation.

Suggested Standard Modifications

Remedy #1:

1.- Add the following text at the end of page 84 in section 16.2.3.12

Dynamic SFH information such as MSB of superframe number shall not be transmitted in the AAI_NBR-ADV message.

2.- Add the following text and table at the end of section 16.2.3.12, in page 86 after line 58.

Delta information is encoded as a sequence of TLVs. The encoding of the one-byte type field for S-SFH elements can be found in table XXX. The length field should equal the element length as specified in the S-SFH, rounded up to an integer number of bytes.

The delta information field should be padded so its length will be an integer number of bytes.

Table XXX – Type values for delta SFH encoding in AAI_NBR-ADV and MC-ADV messages

| Type | Name |
| --- | --- |
| 1 | LSBs of 48 bit ABS MAC ID |
| 2 | Number of UL ACK/NACK channels per HARQ feedback region |
| 3 | Number of DL ACK/NACK channels per HF-A-MAP region |
| 4 | Power control channel resource size indicator |
| 5 | Primary frequency partition location |
| 6 | A-A-MAP MCS selection |
| 7 | DCASSB0 |
| 8 | DCASMB0 |
| 9 | DCASi |
| 10 | Frame configuration index |
| 11 | WirelessMAN-OFDMA support |
| 12 | Allocation periodicity of ranging channel |
| 13 | Subframe offset of ranging channel |
| 14 | Start code information of ranging channel |
| 15 | Ranging preamble code partition information |
| 16 | UL_Permbase |
| 17 | Allocation periodicity of ranging channel for synchronized AMSs |
| 18 | Start code information of ranging channel for synchronized AMSs |
| 19 | Allocation periodicity of ranging channel for non-synchronized AMSs |
| 20 | Start code information of ranging channel for non-synchronized AMSs |
| 21 | Ranging preamble code partition information for non-synchronized AMSs |
| 22 | Number of cyclic shifted ranging preamble codes per root index for non-synchronized AMSs |
| 23 | Ranging channel formats for non-synchronized AMSs |
| 24 | UCASSB0 |
| 25 | UCASMB0 |
| 26 | UCASi |
| 27 | Uplink AAI subframes for sounding |
| 28 | ABS EIRP |
| 29 | Cell bar information |
| 30 | UL_N_MAX_ReTx |
| 31 | DL_N_MAX_ReTx |
| 32 | TUL_Rx_Processing |
| 33 | UL carrier frequency |
| 34 | UL bandwidth |
| 35 | MSB bytes of 48 bit ABS MAC ID |
| 36 | MAC protocol revision |
| 37 | DSAC |
| 38 | DFPC |
| 39 | DFPSC |
| 40 | USAC |
| 41 | UFPC |
| 42 | UFPSC |
| 43 | AMS Transmit Power Limitation Level |
| 44 | EIRxPIR,min |
| 45 | Rate of change of S-SFH info |
| 46 | SA-Preamble sequence soft partitioning information |
| 47 | FFR partition resource metrics |
| 48 | IoT correction value for UL power control |
| 49 | Number of Distributed LRUs for UL feedback channel per a UL AAI subframe |
| 50 | # Tx antenna |
| 51 | SP scheduling periodicity information |
| 52 | HO Ranging backoff start |
| 53 | HO Ranging backoff end |
| 54 | Initial ranging backoff start |
| 55 | Initial ranging backoff end |
| 56 | UL BW REQ channel information |
| 57 | Bandwidth request backoff start |
| 58 | Bandwidth request backoff end |
| 59 | fpPowerConfig |
| 60 | BS_Restart_Count |
| 61 | SA_PreamblePartitionforBStype |
| 62 | Trigger TLV encoding |
| 63 | DefaultTrigger_Averagingparameter |
| 64 | NormalizedCINR |
| 65 | OL-Region-ON |
| 66 | OL-Rank1-Configuration |
| 67 | SB-OL-Region-2Size |
| 68 | periodicityOfRngChSync |
| 69 | cntlStartCodeOfRngChSync |
| 70 | rangingPreambleCodeSync |
| 71 | GAMMA_IOT_FP0 |
| 72 | GAMMA_IOT_FP1 |
| 73 | GAMMA_IOT_FP2 |
| 74 | GAMMA_IOT_FP3 |
| 75 | Alpha (a) |
| 76 | Beta (ß) |
| 77 | SINRmin_Data |
| 78 | SINRmax_Data |
| 79 | SINR_Target_HARQ |
| 80 | SINR_Target_SyncRanging |
| 81 | SINR_Target_PFBCH |
| 82 | SINR_Target_SFBCH_BASE |
| 83 | SINR_Target_SFBCH_Delta |
| 84 | SINR_Target_BWRequest |
| 85 | GAMMA_IOT_SOUNDING |
| 86 | SINRmin_SOUNDING |
| 87 | T_ReTx_Interval |
| 88 | BR_Channel Configuration MIN Access Class |
| 89 | SoundingMultiplexingType |
| 90 | shiftValueUForSoundingSymbol |
| 91 | 16m_Relay_zone_AMS_allocation_indicator |

Remedy #2:

1.-Make the following changes in section 16.2.23, page 335, lines 61-65

16.2.23 Update of S-SFH IEs

The S-SFH channel subpacket (SP) IEs (i.e., S-SFH SP1 IE, SP 2 IE and SP3 IE) are transmitted by ABS at the scheduled period of each channel SP, respectively, with different periodicity (i.e., TC1< TC2<TC3TSP1 < TSP2 < TSP3). This SP scheduling periodicity information is transmitted in the S-SFH SP3 IE. Otherwise, an AMS may implicitly know each S-SFH SP IE's scheduling period by receiving each subsequent S-SFH SP IE.

Each S-SFH channel (channels 1, 2 and 3) carries fragments of the corresponding S-SFH subpackets (SP1, SP2 and SP3 respectively). If an S-SFH subpacket is comprised of Nfrag fragments, the corresponding S-SFH channel is transmitted over consecutive Nfrag superframes. This is indicated by the More fragments field of the S-SFH IE.

2.-Make the following changes in section 16.3.6.2.1.2, page 434, lines 1-26

16.3.6.2.1.2 Secondary Superframe Header

The Secondary Superframe Header (S-SFH) may be transmitted in every superframe

If the S-SFH is present, the S-SFH shall be mapped to the NS-SFH distributed LRUs following the NP-SFHdistributed LRUs. The value of NS-SFH is indicated in P-SFH IE.

The S-SFH can be repeated over two consecutive superframes.

The information transmitted in S-SFH is divided into three sub-packets, which are carried by three S-SFH channels.

The channels sub-packets of S-SFH are transmitted periodically where each channels sub-packet has a different transmission periodicity as illustrated in Figure 515.

The “channel SP scheduling periodicity information" field of S-SFH SP3 is used to indicate the transmission periodicity

of the S-SFH channels SPs (1, 2, 3). Table 807 shows the transmission periodicity of different S-SFH channels SPs for different values of " channel SP scheduling periodicity information" field.

S-SFH subpackets can be fragmented. If an S-SFH subpacket is comprised of Nfrag fragments, the corresponding S-SFH channel is transmitted over consecutive Nfrag superframes. This is indicated by the More fragments field of the S-SFH IE.

[Make the following changes in table 807, page 435, lines 3-12]

Table 807—Transmission Periodicity of S-SFH SPs

| Channel SP scheduling periodicity information | Transmission periodicity of S-SFH Channel SP1 | Transmission periodicity of S-SFH Channel SP2 | Transmission periodicity of S-SFH Channel SP3 |
| --- | --- | --- | --- |

3.-Make the following changes in section 16.3.6.5.1.2, page 44 , lines 8-52

16.3.6.5.1.2 S-SFH IE

The S-SFH IE is mapped to the S-SFH. Essential system parameters and system configuration information belonging to the S-SFH are categorized into three S-SFH subpacket IEs such as SP1, SP2 and SP3. These SPs are carried by S-SFH channels 1, 2 and 3 respectively.These channels SPs are transmitted in different timing and periodicity. The periodicity of each channel SP (TCiTSP) is determined with TSP1 < TSP2 < TSP3.  TC1< TC2<TC3

The S-SFH IE format are shown in Table 810.

Each S-SFH subpacket IE is of a fixed size.

Table 810—S-SFH IE format

| - Syntax | - Size (bit) | - Notes |
| --- | --- | --- |
| S-SFH IE format () { |  |  |
| If (1st bit of S-SFH Scheduling information bitmap == 1) { |  |  |
| Fragment index | 4 | The index of the SP1 fragment transmitted in this superframe. Values are 0..15 |
| More fragments | 1 | When set, indicates another fragment of SP1 will be transmitted in the following superframe header. |
| S-SFH SP1 IE () |  |  |
| } |  |  |
| Fragment index | 4 | The index of the SP2 fragment transmitted in this superframe. Values are 0..15 |
| More fragments | 1 | When set, indicates another fragment of SP2 will be transmitted in the following superframe header. |
| if (2nd bit of S-SFH Scheduling information bitmap == 1) { |  |  |
| S-SFH SP2 IE () |  |  |
| } |  |  |
| Fragment index | 4 | The index of the SP3 fragment transmitted in this superframe. Values are 0..15 |
| More fragments | 1 | When set, indicates another fragment of SP3 will be transmitted in the following superframe header. |
| S-SFH SP 3 IE() |  |  |
| } |  |  |

4.-Make the following changes in table 813, page 453, lines 44-46

Table 813—S-SFH SP3 IE format

| Channel SP scheduling periodicity information | TBD |  |
| --- | --- | --- |