| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed changes related to MIMO feedback extended header (16.2.2.2.7) |  |
| Date Submitted | 2010-03-14 |  |
| Source(s) | Jisoo Park, Namsuk Lee, Sookjin Lee, Yongseouk Choi, Nakwoon Sung, Jaesun Cha ETRI | jsp@etri.re.kr |
| Re: | “P802.16m/D4,” in response to the IEEE 802.16 Working Group Letter Ballot #31: Announcement, IEEE 802.16-10/0011 |  |
| Abstract | The contribution proposes the text changes to MIMO feedback extended header (MFEH) in 802.16m/D4. |  |
| Purpose | To discuss and adopt the proposed text in the next revision of 802.16m amendment draft. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed changes related to MIMO feedback extended header (16.2.2.2.7)

Jisoo Park, Namsuk Lee, Sookjin Lee, Yongseouk Choi, Nakwoon Sung, Jaesun Cha

ETRI

- Introduction

This contribution proposes the text changes to Feedback Polling A-MAP IE defined in the current 802.16m draft standard [1]. The ABS schedules MIMO feedback transmission by the AMS according to the requested feedback information through this A-MAP IE. At last meeting, there were several changes for resource allocation/de-allocation and feedback content allocation of any MIMO feedback mode in [2]. Specially, it was proposed that the MIMO feedback of the transmit correlation matrix and any MIMO feedback mode can be jointly allocated and multiple allocation of MIMO feedback modes(MFM) with a single IE in the same frame and AAI subframe index can be enabled by MFM_bitmap.

As this reason, if a MIMO feedback header can accommodate multiple feedback contents for the allocated multiple MFMs by Feedback Polling A-MAP, it is necessary to consider it for reducing overhead by packing MAC PDUs with any MFM.

Additionally, in draft standard, since it is specified that the extended headers shall not be present in a MAC PDU without payload, then AMS may not send the MIMO feedbacks using the extended header by the stand-alone format. Specially, if a dedicated UL allocation with designated resource index is reserved for the MIMO feedback, the extended header such as MIMO Feedback Extended Header (MFEH) and Correlation Matrix Feedback Extended Header (CMFEH) cannot be sent to by AMS.

If the MIMO feedback is moved to MAC signaling header with 6 bytes, it can accommodate at least two MFMs. But, multiple MIMO feedback contents cannot be accommodated in a MFEH extended header, because the extended header size is restricted within 3 bytes for feedback contents.

So, we should consider that the contents of extended headers for MIMO feedback are moved to the type of MAC signaling header in order to send it by the stand-alone MAC PDU with multiple MFMs, and MIMO feedback extended headers are removed.

In this contribution, we propose the text changes related to MIMO feedback for MFEH in the extended header section.

- To move the MIMO feedback header to signaling header

- To include one feedback content or multiple feedback contents up to three MFMs for wideband information of MFM 0, 1, 4, 7 or two MFMs for subband information for one subband of MFM 2, 3, 5, 6 within 6 bytes

- To remove previous extended header section

- References

- IEEE P802.16m/D4, “Draft Amendment to IEEE Standard for Local and Metropolitan Area Networks- Part 16: Air Interface for Broadband Wireless Access Systems,” February 3, 2010.

- David Mazzarese, Sangheon Kim, Hokyu Choi, Heewon Kang, Jisoo Park, Namsuk Lee, Sookjin Lee, Yongseouk Choi, Nakwoon Sung, “Proposed changes to the feedback polling A-MAP IE (16.3.6),” C802.16m-09/2799r3, January 12, 2010.

- Proposed Changes for inclusion in the 802.16m/D4

[Added texts and figures marked in blue font with underline and removed texts and figures marked in red font with strikeout]

------------------------------------------------------ Start of the Text -------------------------------------------------------

[Remedy 1: the text changes for section 16.2.2.2.7 MIMO feedback extended header on line 33, page 56 as follows]

16.2.2.1.3.x2.7 MIMO Feedback Report extended Header (MFEH)

This MIMO feedback report extended header format is defined in Table XXX 671 This signaling header is used by AMS as a response to a Feedback Polling A-MAP IE requesting to feedback only the wideband information for MFM 0, 1, 4 or 7, or only the subband information for one subband for MFM 2, 3, 5 or 6 by including one feedback contents or multiple feedback contents for differently requested MFMs. Variables MaxMt, Codebook_subset, Codebook_coordination, short period p, long period q and Measurement Method Indication are indicated in the Feedback Polling A-MAP IE. Nt is the number of transmit antennas at the ABS, indicated in S-SFH SP3. The number of subbands YSB is described in section 16.3.9.3.1.5. The MIMO feedback report headerMFEH shall be padded by zeros for byte alignment to 63 bytes.

The decimal value of subband and wideband STC rate is encoded with 2 or 3 bits in binary representation, but its allowed values depend on Measurement Method Indication and MaxMt. The decimal value of subband and wideband PMI is encoded with 4 or 6 bits in binary representation, but its allowed values depend on Nt and Codebook_subset. The decimal value of subband stream index is always encoded with 2 bits, but its allowed values depend on Measurement Method Indication and MaxMt. The decimal value of logical subband index is always encoded with 5 bits, but its allowed values are limited to the interval [0, YSB-1].

Table 671– MFEH format

Table XXX– MIMO Feedback Report Header Format

| Syntax | Size (bits) | Notes |
| --- | --- | --- |
| MIMO Feedback Report Header MFEH () { |  |  |
| FID | 4 | Flow Identifier. |
| Type | 4 | MAC signaling header MFEH type |
| MFM_bitmap MFM | 83 | Bitmap to indicate the MFMs for which the AMS is sending feedback. It shall be consistent with current feedback allocations corresponding to the MFM requested by Feedback Polling A-MAP IE. LSB #0: MFM 0 LSB #1: MFM 1 LSB #2: MFM 2 LSB #3: MFM 3 LSB #4: MFM 4 LSB #5: MFM 5 LSB #6: MFM 6 LSB #7: MFM 7 |
| If (LSB#0 in MFM_bitmap == 1 MFM == 0){ |  |  |
| Wideband CQI | 4 |  |
| Wideband STC rate | 3 |  |
| Zero padding | 10 |  |
| } |  |  |
| If (LSB#1 in MFM_bitmap == 1 MFM == 1){ |  |  |
| Wideband CQI | 4 |  |
| Zero padding | 13 |  |
| } |  |  |
| If (LSB#2 in MFM_bitmap == 1 MFM == 2){ |  |  |
| Subband index | 5 |  |
| Subband CQI | 4 |  |
| Subband STC rate | 3 |  |
| Zero padding | 5 |  |
| } |  |  |
| If (LSB#3 in MFM_bitmap == 1 MFM == 3 and Nt == 2 or 4){ |  |  |
| Subband index | 5 |  |
| Subband CQI | 4 |  |
| Subband STC rate | 2 |  |
| Subband PMI | 6 |  |
| } |  |  |
| If (LSB#3 in MFM_bitmap == 1 MFM == 3 and Nt == 8){ |  |  |
| Subband index | 5 |  |
| Subband CQI | 4 |  |
| Subband STC rate | 3 |  |
| Subband PMI | 4 |  |
| Zero padding | 1 |  |
| } |  |  |
| If (LSB#4 in MFM_bitmap == 1 MFM == 4){ |  |  |
| Wideband CQI | 4 |  |
| Wideband STC rate | 3 |  |
| Wideband PMI | 6 |  |
| Zero padding | 4 |  |
| } |  |  |
| If (LSB#5 in MFM_bitmap == 1 MFM == 5){ |  |  |
| Subband index | 5 |  |
| Subband CQI | 4 |  |
| Subband stream index | 2 |  |
| Zero padding | 6 |  |
| } |  |  |
| If (LSB#6 in MFM_bitmap == 1 MFM == 6){ |  |  |
| Subband index | 5 |  |
| Subband CQI | 4 |  |
| Subband PMI | 6 |  |
| Zero padding | 2 |  |
| } |  |  |
| If (LSB#7 in MFM_bitmap == 1 MFM == 7){ |  |  |
| Wideband CQI | 4 |  |
| Wideband PMI | 6 |  |
| Zero padding | 7 |  |
| } |  |  |
| Reserved | variable | For byte alignment |
| } |  |  |

[Remedy 2: Move the above ‘Remedy 1’ to the part of section 16.2.2.1.3 MAC Signaling Header]

Move the text with above ‘Remedy 1’ in section 16.2.2.1.3 MAC Signaling Header.

[Remedy 3: the text changes for section 16.2.2.1.3 MAC Signaling Header on line 40, page 40 as follows]

16.2.2.1.3 MAC Signaling Header

Table 656—Type field encodings for MAC signaling header type

| Type field (4 bits) | MAC Signaling Header Type |
| --- | --- |
| 0000 | BR with STID |
| 0001 | BR without STID |
| 0010 | Service specific BR without STID |
| 0011 | Sleep Control |
| 0100 | AMS Battery Level Report |
| 0101 | MIMO Feedback Report |
| 01010110- 1111 | Reserved |

------------------------------------------------------- End of the Text -------------------------------------------------------