| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed changes related to AAI_SingleBS_MIMO_FBK message (16.2.3.37) |  |
| Date Submitted | 2010-03-05 |  |
| Source(s) | Jisoo Park, Namsuk Lee, Sookjin Lee, Yongseouk Choi, Nakwoon Sung ETRI | jsp@etri.re.kr |
| Re: | “P802.16m/D4,” in response to the IEEE 802.16 Working Group Letter Ballot #31: Announcement, IEEE 802.16-10/0011 |  |
| Abstract | The contribution proposes the text changes to AAI_SingleBS_MIMO_FBK message in 802.16m/D4. |  |
| Purpose | To discuss and adopt the proposed text in the next revision of 802.16m amendment draft. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed changes related to AAI_SingleBS_MIMO_FBK message (16.2.3.37)

Jisoo Park, Namsuk Lee, Sookjin Lee, Yongseouk Choi, Nakwoon Sung

ETRI

- Introduction

This contribution proposes the text changes to Feedback Polling A-MAP IE defined in the current 802.16m draft [1]. The ABS schedules MIMO feedback transmission by the AMS according to the requested feedback information through this A-MAP IE. At last meeting, there were several changes for resource allocation/de-allocation and feedback content allocation of any MIMO feedback mode in [2]. Specially, it was proposed that the MIMO feedback of the transmit correlation matrix and any MIMO feedback mode can be jointly allocated and multiple allocation of MIMO feedback modes with a single IE in the same frame and AAI subframe index can be enabled by MFM_bitmap.

In draft standard, since it is specified that the MIMO feedback of the transmit correlation matrix and any MIMO feedback mode can be jointly allocated and multiple allocation of MIMO feedback modes with a single A-MAP IE in the same frame and same AAI subframe index can be enabled by MFM_bitmap, we can consider to revise AAI_SingleBS_MIMO_FBK in order to include feedback contents of multiple allocation for the transmit correlation matrix with Nt = 8 and MFM 0, 1, 4, or 7 as well as MFM 2, 3, 4, or 6. This can reduce the overhead for the multiple feedback transmission without packing multiple MAC PDUs of feedback headers.

As this reason, we propose the text changes related to the MIMO feedback of AAI_SingleBS_MIMO_FBK message including some corrections for readability in this contribution.

- References

- IEEE P802.16m/D4, “Draft Amendment to IEEE Standard for Local and Metropolitan Area Networks- Part 16: Air Interface for Broadband Wireless Access Systems,” February 3, 2010.

- David Mazzarese, Sangheon Kim, Hokyu Choi, Heewon Kang, Jisoo Park, Namsuk Lee, Sookjin Lee, Yongseouk Choi, Nakwoon Sung, “Proposed changes to the feedback polling A-MAP IE (16.3.6),” C802.16m-09/2799r3, January 12, 2010.

- Proposed Changes for inclusion in the 802.16m/D4

[Added texts and figures marked in blue font with underline and removed texts and figures marked in red font with strikeout]

------------------------------------------------------ Start of the Text -------------------------------------------------------

[Remedy: the text changes for section 16.2.3.37 AAI_SingleBS_MIMO_FBK on line 54, page 123 as follows]

The AAI_SingleBS_MIMO_FBK message format is defined in Table 713. This message is used by AMS as a response to a Feedback Polling A-MAP IE requesting to feedback the subband information for MFM 0, 1, 2, 3, 4, 5, 6 or 76. It is also used for the feedback of the transmit correlation matrix when the ABS is equipped with 2, 4, or 8 transmit antennas. Variables MaxMt, Codebook_subset, Codebook_mode, Num_best_subbands and Measurement Method Indication are indicated in the Feedback Polling A-MAP IE. Nt is the number of transmit antennas at the ABS, indicated in S-SFH SP3. Best_subbands_index is encoded as described in section 16.3.9.3.1.4. The feedback information reported in the AAI_SingleBS_MIMO_FBK message depends on currently assigned feedback processes and reporting times.

Table 713– AAI_SingleBS_MIMO_FBK message format

| Syntax | Size (bits) | Notes |
| --- | --- | --- |
| AAI_SingleBS_MIMO_FBK_Message_Format { |  |  |
| Management Message Type | 8 |  |
| If (((MFM == 3) and (q > 0)) or ((MFM == 6) and (q > 0)) or ((MFM == 4) and (q > 0)) or ((MFM == 7) and (q > 0))){ If ( (q > 0) and ((MFM == 3) or (MFM == 6) or (MFM == 4) or (MFM == 7)) ){ |  | MFM and long period q are indicated in Feedback Polling A-MAP IEs relevant to currently assigned feedback processes. |
| For (i=1; i <= Nt; i++){ |  |  |
| i-th diagonal entry of correlation matrix | 1 | As defined in section 16.3.7.2.5.6 |
| For (j=i+1; j <= Nt; j++){ |  |  |
| (i,j)-th entry of correlation matrix | 4 | As defined in section 16.3.7.2.5.6 |
| } |  |  |
| } |  |  |
| } |  |  |
| MFM_bitmap | 48 | Bitmap to indicate the MFMs for which the AMS is sending feedback. It shall be consistent with current feedback allocations corresponding to the MFM requested by Feedback Polling IE. LSB #0: MFM 02 LSB #1: MFM 13 LSB #2: MFM 25 LSB #3: MFM 36 LSB #4: MFM 4 LSB #5: MFM 5 LSB #6: MFM 6 LSB #7: MFM 7 |
| If (LSB #0 in MFM_bitmap == 1){ |  | MFM 0 as specified in Feedback Polling A-MAP IE |
| Wideband CQI | 4 |  |
| Wideband STC rate | 3 |  |
| } |  |  |
| If (LSB #1 in MFM_bitmap == 1){ |  | MFM 1 as specified in Feedback Polling A-MAP IE |
| Wideband CQI | 4 |  |
| } |  |  |
| If (LSB #02 in MFM_bitmap == 1){ |  | MFM 2 as specified in Feedback Polling A-MAP IE |
| Best_subbands_index | variable |  |
| STC_Rate | variable | If Measurement Method Indication=0b0: MaxMt = 2: 1 bit MaxMt = 3 or 4: 2 bits MaxMt > 4: 3 bits This field shall be omitted if Measurement Method Indication=0b1 or if MaxMt = 1. |
| For (m=0; m < Num_best_subbands; m ++){ |  | The subbands are sorted in order of increasing logical index |
| Subband CQI | 4 | MCS of m-th subband indicated by Best_subbands index |
| } |  |  |
| } |  |  |
| If (LSB #13 in MFM_bitmap == 1){ |  | MFM 3 as specified in Feedback Polling A-MAP IE |
| Best_subbands_index | variable |  |
| STC_Rate | variable | MaxMt = 2: 1 bit MaxMt = 3 or 4: 2 bits MaxMt > 4: 3 bits This field shall be omitted if MaxMt = 1. |
| For (m=0; m < Num_best_subbands; m ++){ |  | The subbands are sorted in order of increasing logical index |
| Subband CQI | 4 | MCS of m-th subband indicated by Best_subbands_index |
| Subband PMI | variable | PMI of m-th subband indicated by Best_subbands_index Nt = 2: 3 bits Nt = 4 and CS = 0b0: 6 bits Nt = 4 and CS = 0b1: 4 bits Nt = 8: 4 bits |
| } |  |  |
| } |  |  |
| If (LSB #4 in MFM_bitmap == 1){ |  | MFM 4 as specified in Feedback Polling A-MAP IE |
| Wideband CQI | 4 |  |
| Wideband STC rate | 3 |  |
| Wideband PMI | 6 |  |
| } |  |  |
| If (LSB #25 in MFM_bitmap == 1){ |  | MFM 5 as specified in Feedback Polling A-MAP IE |
| Best_subbands_index | variable |  |
| For (m=0; m < Num_best_subbands; m ++){ |  | The subbands are sorted in order of increasing logical index |
| Subband CQI | 4 | MCS of m-th subband indicated by Best_subbands_index |
| Stream index | variable | Best stream index of m-th subband indicated by Best_subbands_index If Measurement Method Indication=0b0: MaxMt = 2: 1 bit MaxMt = 3 or 4: 2 bits If Measurement Method Indication=0b1: 1 bit |
| } |  |  |
| } |  |  |
| If (LSB #36 in MFM_bitmap == 1){ |  | MFM 6 as specified in Feedback Polling A-MAP IE |
| Best_subbands_index | variable |  |
| For (m=0; m < Num_best_subbands; m ++){ |  | The subbands are sorted in order of increasing logical index |
| Subband CQI | 4 | MCS of m-th subband indicated by Best_subbands_index |
| Subband PMI | variable | PMI of m-th subband indicated by Best_subbands_index Nt = 2: 3 bits Nt = 4 and CS = 0b0: 6 bits Nt = 4 and CS = 0b1: 4 bits Nt = 8: 4 bits |
| } |  |  |
| } |  |  |
| If (LSB #7 in MFM_bitmap == 1){ |  | MFM 7 as specified in Feedback Polling A-MAP IE |
| Wideband CQI | 4 |  |
| Wideband PMI | 6 |  |
| } |  |  |
| } |  |  |

------------------------------------------------------- End of the Text -------------------------------------------------------