| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Changes Related to Feedback Polling A-MAP IE in 802.16m/D4 (16.3.6.5.2.4.11) |  |
| Date Submitted | 2010-03-05 |  |
| Source(s) | Jisoo Park, Namsuk Lee, Sookjin Lee, Yongseouk Choi, Nakwoon Sung, Jaesun Cha ETRI | jsp@etri.re.kr |
| Re: | “P802.16m/D4,” in response to the IEEE 802.16 Working Group Letter Ballot #31: Announcement, IEEE 802.16-10/0011 |  |
| Abstract | The contribution proposes the text changes to Feedback Polling A-MAP IE in 802.16m/D4. |  |
| Purpose | To discuss and adopt the proposed text in the next revision of 802.16m amendment draft. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Changes Related to Feedback Polling A-MAP IE in 802.16m/D4 (16.3.6.5.2.4.11)

Jisoo Park, Namsuk Lee, Sookjin Lee, Yongseouk Choi, Nakwoon Sung, Jaesun Cha

ETRI

- Introduction

This contribution proposes the text changes to Feedback Polling A-MAP IE defined in the current 802.16m draft [1]. The ABS schedules MIMO feedback transmission by the AMS according to the requested feedback information through this Feedback Polling A-MAP IE. At last meeting, there were several changes for resource allocation/de-allocation and feedback content allocation of MIMO feedback mode in [2]. Specially, it was proposed and accepted that the MIMO feedback of the transmit correlation matrix and any MIMO feedback mode can be jointly allocated and multiple allocation of MIMO feedback modes with a single IE in the same frame and AAI subframe index can be enabled by MFM_bitmap. In addition, the rules of the short feedback period (p) and long feedback period (q) setting, uplink HARQ retransmission and ACID reservation for MIMO feedback in extended header or MAC control message were specified in draft standard. As this reason, we should consider the follows.

< HARQ process for MIMO feedback >

When (p > 0 and q > 0) and short and long period feedback reports coincide in the same frame, we should consider the selection of an ACID between two ACIDs and resource re-assignment as following, since both the short and long period feedback contents should be sent in the same burst.

- For ACID

An ACID of the combined burst should be specified in this case between the two reserved ACIDs. Although two ACIDs are reserved for MIMO feedback, first ACID is preferred by this contribution in this case.

- For resource re-assignment

The resource size of the combined MIMO feedback reports may be different from that of each period feedback burst by the resource index in A-MAP IE. So, the resource re-assignment with UL Basic Assignment A-MAP IE may be necessary for the combined MIMO feedback reports in this case.

If p is very short and the retransmission process for the previous HARQ burst is not finished before a new HARQ burst with the same ACID is transmitted, we should review the rule of feedback retransmission.

- For retransmission

In the case of that the feedback period is very short (i.e. p = 1 or 2 frame), HARQ retransmission of previous short period feedback cannot be carried on, and it will be almost terminated by the current retransmission rule. If the burst error rate of the first feedback HARQ transmission is 50%, the loss rate of the MIMO feedback information is 50%, even if the error rate of second transmission is very low as not higher 5%. Specially, if UL Rx Processing time for UL data burst is large (i.e. TUL_Rx_Processing = 4 for F= 8 in FDD or D + U = 8 in TDD) or the feedback burst is allocated by long TTI transmission, the retransmission process for the previous burst will be mostly terminated as the above mention because of its latency.

So, the uplink retransmission rules for MIMO feedback should be considered for modification as the adaptive resource allocation for feedback retransmission with changing resource location by UL Basic Assignment A-MAP IE and the assignment of other ACID for a new feedback burst. We prefer 2 ACIDs by rotation for short period burst.

Also, since the feedback contents of both the short period feedback and long period feedback are different from each other, for effectively supporting MIMO transmission mode, the ABS may want to receive many successful number of feedback times with the assigned period in the assigned duration. So, when (p > 0 and q > 0) and even if retransmission process of previous short period feedback is not finished before a new long period feedback is transmitted, or vice versa, we should consider to save the retransmission process for the previous period feedback. This can be easily solved through that the ABS allocates an uplink resource with changed resource index (L) by the UL Basic Assignment A-MAP IE.

< MIMO feedback messages: C80216m-10_0289, 0290, 0291 or latest revision >

- Extended headers

In draft standard, since it is specified that the extended headers shall not be present in a MAC PDU without payload, then AMS may not send the MIMO feedbacks using the extended header by the stand-alone format. Specially, if a dedicated UL allocation with designated resource index is reserved for the MIMO feedback, the extended header such as MIMO Feedback Extended Header (MFEH) and Correlation Matrix Feedback Extended Header (CMFEH) cannot be sent to by AMS.

So, we should consider that the contents of extended headers for MIMO feedback are moved to the type of MAC signaling header in order to send it by the stand-alone MAC PDU and MIMO feedback extended headers are removed. As this reason, we propose the text changes related to MIMO feedback extended headers by another contribution document (C80216m-10_0289, C80216m-10_0290, or latest revision).

- MAC control message: AAI_SingleBS_MIMO_FBK

In draft standard, since it is specified that the MIMO feedback of the transmit correlation matrix and any MIMO feedback mode can be jointly allocated and multiple allocation of MIMO feedback modes with a single A-MAP IE in the same frame and same AAI subframe index can be enabled by MFM_bitmap, we can consider to revise AAI_SingleBS_MIMO_FBK in order to include feedback contents of multiple allocation for the transmit correlation matrix with Nt = 2, 4, or 8 and MFM 0, 1, 4, or 7 as well as MFM 2, 3, 4, or 6.

This can reduce the overhead for the multiple feedback transmission without packing multiple MAC PDUs of feedback headers. As this reason, we propose the text changes related to the MIMO feedback AAI_SingleBS_MIMO_FBK message by another contribution document (C80216m-10_0291 or latest revision).

- Proposal

In this contribution, it is proposed to the text changes to be related to HARQ process for the retransmission of MIMO feedback and the combined allocation of short and long period feedback including some editorial corrections.

- References

- IEEE P802.16m/D4, “Draft Amendment to IEEE Standard for Local and Metropolitan Area Networks- Part 16: Air Interface for Broadband Wireless Access Systems,” February 3, 2010.

- David Mazzarese, Sangheon Kim, Hokyu Choi, Heewon Kang, Jisoo Park, Namsuk Lee, Sookjin Lee, Yongseouk Choi, Nakwoon Sung, “Proposed changes to the feedback polling A-MAP IE (16.3.6),” C802.16m-09/2799r3, January 12, 2010.

- Proposed Changes for inclusion in the 802.16m/D4

[Added texts and figures marked in blue font with underline and removed texts and figures marked in red font with strikeout]

------------------------------------------------------ Start of the Text -------------------------------------------------------

[Remedy: the text changes for section 16.3.6.5.2.4.11 Feedback Polling A-MAP IE on line 16, page 511 as follows]

The information element shown in Table 836 is used by the ABS to schedule MMO MIMO feedback transmission by the AMS. The AMS sends the MIMO feedback using a MAC control message or an extended a signaling header, depending on the requested feedback content.

When the Polling_sub_type bit is set to 0b0, a dedicated UL allocation is included in this IE. The dedicated UL allocation shall be used by the AMS to transmit feedback at the designated feedback transmission frame defined by this IE. When the Polling_sub_type bit is set to 0b1, no dedicated UL allocation is included. Instead, at the designated transmission frame defined by this IE, the AMS shall compose the feedback, and the ABS shall either include a UL allocation for the transmission using UL Basic Assignment A-MAP IE or UL Subband Assignment A-MAP IE, or the AMS shall transmit in a dedicated UL allocation assigned by a previous Feedback Polling A-MAP IE with feedback periods designating the same transmission frames.

The AMS shall send the feedback in an extended a signaling header or MAC control message when the following feedback information is requested:

-	wideband information for MIMO feedback modes 0, 1, 4 and 7

-	subband information for 1 subband for MIMO feedback modes 2, 3, 5 and 6

The AMS shall send the feedback in a MAC control message when the following feedback information is requested:

-	subband information for more than one subband for MIMO feedback modes 2, 3, 5 and 6

-	multi-BS feedback

The coefficients of the quantized transmit correlation matrix should be feedback in Correlation Matrix Feedback Extended Report Header (CMFEH) when no AAI_SingleBS_MIMO_FBK message is sent in the same packet if the ABS is equipped with 2 or 4 transmit antennas. Otherwise, the coefficients of the quantized transmit correlation matrix should be feedback in AAI_SingleBS_MIMO_FBK message. The coefficients of the quantized transmit correlation matrix shall be feedback in AAI_SingleBS_MIMO_FBK message if the ABS is equipped with 8 transmit antennas.

In case of feedback for MIMO feedback modes 0 with Measurement Method Indication = 0b0, and MIMO feedback modes 4 or 7, the AMS shall feedback the CQI for FP0 if FPS0>0, or for FPk if FPS0 = 0 FPS0 > 0, where FPk is determined by k = floor(IDCell/256) )+1 for FPCT=3 or k = floor(IDcell/384)+1 for FPCT=2.

Table 836– Feedback Polling A-MAP IE

| Syntax | Size (bits) | Notes |
| --- | --- | --- |
| Feedback_Polling_A-MAP_IE(){ |  |  |
| A-MAP IE Type | 4 | Feedback Polling A-MAP IE |
| Polling_sub_type | 1 | 0b0: uplink resource allocation or de-allocation. 0b1: feedback mode allocation or de-allocation. |
| If (Polling_sub_type == 0b0){ |  |  |
| Allocation Duration (d) | 3 | The allocation is valid for 2(d–1) superframes starting from the superframe defined by allocation relevance. If d == 0b000, the pre-scheduled feedback header transmission is released. If d == 0b111, the pre-scheduled feedback header transmission shall be valid until the ABS commands to release it. |
| If (d ==0b000){ |  | Feedback de-allocation |
| Resource Index | 11 | Confirmation of the resource index for a previously assigned persistent resource that has been deallocated 5 MHz: 0 in first 2 MSB bits + 9 bits for resource index 10 MHz: 11 bits for resource index 20 MHz: 11 bits for resource index Resource index includes location and allocation size |
| Polling_deallocation_bitmap | 3 |  |
| HFA | 3 | HARQ feedback channel allocation to acknowledge the successful detection of this IE. |
| } else if (d != 0b000){ |  | Feedback allocation |
| ISizeOffset | 5 | Offset used to compute burst size index |
| Resource Index | 11 | 5 MHz: 0 in first 2 MSB bits + 9 bits for resource index 10 MHz: 11 bits for resource index 20 MHz: 11 bits for resource index Resource index includes location and allocation size |
| MEF | 2 | MIMO encoder format for uplink feedback transmission Non-adaptive precoding shall be used at the AMS. 0b00: SFBC 0b01: Vertical encoding with Mt = 2, or Mt =1 if AMS has 1 tranmsit antenna 0b10: CSM with TNS = 2, Mt =1, SI =1 0b11: CSM with TNS = 2, Mt =1, SI =2 |
| Long TTI Indicator | 1 | Indicates number of AAI subframes spanned by the allocated resource. 0b0: AAI subframe (default) 0b1: 4 UL AAI subframes for FDD or all UL AAI subframes for TDD If number of DL AAI subframes, D, is less than number of UL AAI subframes, U, Long TTI Indicator = 0b1 |
| HFA | 3 | Index for HARQ Feedback Allocation |
| ACID | 4 | HARQ channel identifier |
| MFM_allocation_index | 2 | 0b00: MFM 0 with Measurement Method Indication = 0 0b01: MFM 3 with all subbands 0b10: MFM 6 with all subbands 0b11: MFM is defined in Feedback Polling A-MAP IE with Polling_sub-type = 0b1 |
| MaxMt | 1~2 |  |
| Period | 4 | Resource is allocated at frames designated by every short and long period. The short feedback period is p frames. The long feedback period is q superframes. The first allocation shall start two frames later. The AAI subframe index is defined as in 16.2.14.2.2 and the AAI frame index is given by i+2, where i is the index of the frame where the Feedback Polling A-MAP IE is transmitted. The feedback of MIMO feedback modes in MFM_allocation_index is allocated on the short period. The feedback of the transmit correlation matrix is allocated on the long period if q > 0. Short and long period reports shall start at the first allocation. When short and long period feedback reports coincide in the same frame, both short period feedback content and long period feedback content shall be sent in the same burst. 0b0000: p = 1, q = 0 0b0001: p = 2, q = 0 0b0010: p = 4, q = 0 0b0011: p = 8, q = 0 0b0100: p = 16, q = 0 0b0101: p = 1, q = 1 0b0110: p = 2, q = 1 0b0111: p = 1, q = 2 0b1000: p = 2, q = 2 0b1001: p = 4, q = 2 0b1010: p = 1, q = 4 0b1011: p = 2, q = 4 0b1100: p = 4, q = 4 0b1101: p = 0, q = 1 0b1110: p = 0, q = 2 0b1111: p = 0, q = 4 |
| } else { |  | Polling_sub_type == 0b1 |
| ACK Allocation Flag | 1 |  |
| If (ACK Allocation Flag == 0b1) { |  |  |
| HFA | 6 | HARQ feedback channel allocation to acknowledge the successful detection of this IE. |
| } |  |  |
| Allocation Duration (d) | 3 | The allocation is valid for 2(d–1) superframes starting from the superframe defined by allocation relevance. If d == 0b000, the pre-scheduled feedback header transmission is released. If d == 0b111, the pre-scheduled feedback header transmission shall be valid until the BS commands to release it. |
| If (d == 0b000){ |  | Feedback de-allocation |
| Polling_deallocation_bitmap | 3 |  |
| } else { |  | Feedback allocation |
| MIMO_feedback_IE_type | 1 | 0b0: feedback allocation for single-BS MIMO operation 0b1: feedback allocation for multi-BS MIMO operation |
| If (MIMO_feedback_IE_type == 0b0){ |  | Single-BS MIMO feedback request 0b0: feedback allocation for single-BS MIMO operation 0b1: feedback allocation for multi-BS MIMO operation |
| MFM_bitmap | 8 | If a currently allocated MFM is indicated in the MFM_bitmap, it indicates a deallocation and reallocation of this MFM. ACK Allocation Flag shall be set to 0b1 in this case. |
| Period | 4 | 16 combination of short feedback period p (frames) and long feedback period q (superframes). The first report shall start two frames later. The AAI subframe index is defined as in 16.2.14.2.2 and the AAI frame index is given by i+2, where i is the index of the frame where the Feedback Polling A-MAP IE is transmitted. The short period feedback shall be included in the first report. The long period feedback shall also be included in the first report if it is assigned. When short and long period feedback reports coincide in the same frame, both short period feedback content and long period feedback content shall be sent in the same burst. 0b0000: p = 1, q = 0 0b0001: p = 2, q = 0 0b0010: p = 4, q = 0 0b0011: p = 8, q = 0 0b0100: p = 16, q = 0 0b0101: p = 1, q = 1 0b0110: p = 2, q = 1 0b0111: p = 1, q = 2 0b1000: p = 2, q = 2 0b1001: p = 4, q = 2 0b1010: p = 1, q = 4 0b1011: p = 2, q = 4 0b1100: p = 4, q = 4 0b1101: p = 0, q = 1 0b1110: p = 0, q = 2 0b1111: p = 0, q = 4 |
| If (LSB #0 in MFM_bitmap == 1){ |  | MFM 0 |
| MaxMt | 1~2 |  |
| Measurement Method Indication | 1 | 0b0: Use the midamble for CQI measurements 0b1: Use pilots in OL region with MaxMt streams for CQI measurements |
| } |  |  |
| If (LSB #1 in MFM_bitmap == 1){ |  | MFM 1 |
| MaxMt | 2 |  |
| Measurement Method Indication | 1 | 0b0: Reserved 0b1: Use pilots in OL region with MaxMt streams for CQI measurements |
| } |  |  |
| If (LSB #2 in MFM_bitmap == 1){ |  | MFM 2 |
| MaxMt | 1~2 |  |
| Num_best_subbands | 2 | 0b00: report all subbands 0b01: 1 best subband 0b10: min{6, YSB} best subbands 0b11: min{12, YSB} best subbands 1≤ Num_best_subbands ≤ YSB |
| Measurement Method Indication | 1 | 0b0: Use the midamble for CQI measurements 0b1: Use pilots in OL region with MaxMt streams for CQI measurements |
| } |  |  |
| If (LSB #3 in MFM_bitmap == 1){ |  | MFM 3 |
| MaxMt | 1~2 |  |
| Num_best_subbands | 2 | 0b00: report all subbands 0b01: 1 best subband 0b10: min{6, YSB} best subbands 0b11: min{12, YSB} best subbands 1≤ Num_best_subbands ≤ YSB |
| If (q == 0){ |  |  |
| Codebook_coordination | 1 | 0b0: base mode with codebook coordination disabled 0b1: base mode with codebook coordination enabled |
| } |  |  |
| If (Nt == 4){ |  |  |
| Codebook_subset | 1 | 0b0: report PMI from the base codebook or transformed base codebook 0b1: report PMI from codebook subset or transformed codebook subset |
| } |  |  |
| } |  |  |
| If (LSB #4 in MFM_bitmap == 1){ |  | MFM 4 |
| MaxMt | 1~2 |  |
| If (q == 0){ |  |  |
| Codebook_coordination | 1 | 0b0: base mode with codebook coordination disabled 0b1: base mode with codebook coordination enabled |
| } |  |  |
| If (Nt == 4){ |  |  |
| Codebook_subset | 1 | 0b0: report PMI from the base codebook 0b1: report PMI from the codebook subset |
| } |  |  |
| } |  |  |
| If (LSB #5 in MFM_bitmap == 1){ |  | MFM 5 |
| MaxMt | 1~2 |  |
| Num_best_subbands | 2 | 0b00: report all subbands 0b01: 1 best subband 0b10: min{6, YSB} best subbands 0b11: min{12, YSB} best subbands 1≤ Num_best_subbands ≤ YSB |
| Measurement Method Indication | 1 | 0b0: Use the midamble for CQI measurements 0b1: Use pilots in OL region with MaxMt streams for CQI measurements |
| } |  |  |
| If (LSB #6 in MFM_bitmap == 1){ |  | MFM 6 |
| MaxMt | 1~2 |  |
| Num_best_subbands | 2 | 0b00: report all subbands 0b01: 1 best subband 0b10: min{6, YSB} best subbands 0b11: min{12, YSB} best subbands 1≤ Num_best_subbands ≤ YSB |
| If (q == 0){ |  |  |
| Codebook_coordination | 1 | 0b0: base mode with codebook coordination disabled 0b1: base mode with codebook coordination enabled |
| } |  |  |
| If (Nt == 4){ |  |  |
| Codebook_subset | 1 | 0b0: report PMI from the base codebook or transformed base codebook 0b1: report PMI from codebook subset or transformed codebook subset |
| } |  |  |
| } |  |  |
| If (LSB #7 in MFM_bitmap == 1){ |  | MFM 7 |
| MaxMt | 1~2 |  |
| If (q == 0){ |  |  |
| Codebook_coordination | 1 | 0b0: base mode with codebook coordination disabled 0b1: base mode with codebook coordination enabled |
| } |  |  |
| If (Nt == 4){ |  |  |
| Codebook_subset | 1 | 0b0: report PMI from the base codebook 0b1: report PMI from the codebook subset |
| } |  |  |
| } |  |  |
| } else { |  | Multi-BS MIMO feedback request |
| Period (p) | 3 | Transmit feedback header every 4×p frames. The first report shall start at the next frame. |
| TRU | 2 | Target RU indicating which RUs or which type of RU to work on for feedback 0b00: Latest best subbands reported for single BS MIMO 0b01: Whole bandwidth 0b10: FFR partition 0 0b11: boosted FFR partition |
| ICT | 2 | 0b00: PMI restriction for single-BS precoding; 0b01: PMI recommendation for single-BS precoding; 0b10: CL-MD for multi-BS precoding; 0b11: Co-MIMO for multi-BS precoding; |
| If (Nt == 4){ |  |  |
| Codebook_subset | 1 | 0b0: report PMI from the base codebook 0b1: report PMI from the base codebook subset |
| } |  |  |
| N_multiBS_reports | 3 | N_multiBS_reports indicates the number of reports. |
| If (ICT = 0b11) { |  |  |
| MaxUser | 2 | Maximum number of users supported in Co-MIMO in the same resource. 0b00: 2 users 0b01: 3 users 0b10: 4 users 0b11: reserved |
| } |  |  |
| } |  |  |
| } |  |  |
| } |  |  |
| Padding | variable | Padding to reach byte boundary |
| } |  |  |

* A 16 bit CRC is generated based on the contents of the Feedback Polling A-MAP IE. The CRC is masked by the Station ID

Parameters in the Feedback Polling A-MAP IE should be interpreted as for the Feedback Allocation A-MAP IE.

MaxMt: The possible values of MaxMt follow the rules of the Feedback Allocation A-MAP IE.

ACID: HARQ channel identifier. If q=0 or p=0, a single ACID is reserved. Otherwise, two three ACIDs are reserved. The second ACID is obtained as (ACID+1) modulo 1615. The third ACID is obtained as (ACID+2) modulo 16. When two three feedback periods are allocated, the AMS shall send packets at the long period feedback reports with the first ACID, and the AMS shall send packets at the short period feedback reports in rotation with the second ACID and the third ACID. The AMS shall start short period feedback report with the second ACID, and from the next new transmission, the ACID of the feedback burst is changed by rotation between two ACIDs.

When short and long period feedback reports coincide in the same frame, both short period feedback content and long period feedback content shall be sent in the same burst with the first ACID. In this case, if the resource size is changed by combining short and long period feedback content, the ABS shall allocate a changed uplink resource with the UL Basic Assignment A-MAP IE and the first ACID.

For uplink retransmissions of MIMO feedback, the ABS shall allocate a changed uplink resource with the different resource location to avoid collision of resources for periodic feedback and retransmissions by the UL Basic Assignment A-MAP IE and the ACID corresponding to the packet to retransmit. Retransmissions shall obey the following rules:

- If the retransmission process for the previous HARQ burst is not finished before a new HARQ burst with the same ACID is transmitted, the retransmission process for the previous HARQ burst is terminated and the new HARQ burst overrides it.

- If the retransmission process for the previous HARQ burst reporting short-period feedback is not finished before a new HARQ burst reporting short-period feedback with the different ACID is transmitted, the retransmission process for the previous HARQ burst is carried on, and if the decoding of new HARQ burst is successful, a new HARQ burst reporting short-period feedback with the different ACID is terminated.

- If the retransmission process for the previous HARQ burst reporting short-period feedback is not finished before a new HARQ burst reporting long-period feedback is transmitted, the retransmission process for the previous HARQ burst reporting short-period feedback is terminated and the new HARQ burst overrides it.

- If the retransmission process for the previous HARQ burst reporting long-period feedback is not finished before a new HARQ burst reporting short-period feedback is transmitted, the retransmission process for the previous HARQ burst reporting short-period feedback is carried on, but the short-period feedback content of the retransmitted packet shall be discarded by the ABS and the new HARQ burst reporting short-period feedback overrides it.

Polling_deallocation_bitmap: An AMS supports a maximum of 3 distinct concurrent feedback allocations, including one or several MFMs, the transmit correlation matrix, and multiBS MIMO feedback. Ordering of concurrent allocations in the Polling_deallocation_bitmap shall follow the order MFM i, MFM j (j>i), MFM k (k>j), transmit correlation matrix, multiBS MIMO feedback, as shown in Table 837.

Table 837 – Polling Deallocation Bitmap

| Polling_deallocation_bitmap bit #2 (MSB) | Polling_deallocation_bitmap bit #1 | Polling_deallocation_bitmap bit #0 (LSB) |
| --- | --- | --- |
| MFM i | MFM j (j>i) | MFM k (k>j) |
| MFM i | MFM j (j>i) | Transmit correlation matrix |
| MFM i | MFM j (j>i) | multiBS MIMO feedback |
| MFM i | Transmit correlation matrix | multiBS MIMO feedback |
| MFM i | MFM j (j>i) | N.A. |
| MFM i | Transmit correlation matrix | N.A. |
| MFM i | multiBS MIMO feedback | N.A. |
| Transmit correlation matrix | multiBS MIMO feedback | N.A. |
| MFM i | N.A. | N.A. |
| Transmit correlation matrix | N.A. | N.A. |
| multiBS MIMO feedback | N.A. | N.A. |

Period: The setting of short and long periods p and q when Polling_sub_type == 0b0 and MIMO_feedback_IE_type == 0b0 shall obey the rules below.

For MFM 0 or 1: When p > 0, q = 0, the wideband CQI and wideband STC rate shall be transmitted using short feedback period. p > 0 and q > 0 shall not be set at the same time with MFM 0 or 1.

For MFM 2 or 5: When p > 0, q = 0, the Best_subbands_index, subband CQI, and STC rate or Stream index shall be transmitted using short feedback period. p > 0 and q > 0 shall not be set at the same time with MFM 2 or 5.

For MFM 3: when p > 0 and q = 0, the Best_subbands_index, STC_Rate, subband CQI and subband PMI from the base codebook shall be transmitted using short period. When p = 0 and q > 0 p > 0 and q = 0, only the correlation matrix shall be transmitted using long feedback period. When p > 0 and q > 0, the Best_subbands_index, STC_Rate, subband CQI and subband PMI from the transformed codebook shall be transmitted using short period, while the correlation matrix shall be transmitted using long period.

For MFM 6: when p > 0 and q > 0, the Best_subbands_index, STC_Rate, subband CQI and subband PMI from the transformed codebook shall be transmitted using short period, while the correlation matrix shall be transmitted using long period. When p = 0 and q > 0, only the correlation matrix shall be transmitted using long feedback period. The ABS shall not set p > 0 and q = 0 simultaneously unless it has previously allocated feedback for the correlation matrix, in which case the Best_subbands_index, STC_Rate, subband CQI and subband PMI from the transformed codebook shall be transmitted using short period.

For MFM 4: when p > 0 and q = 0, the wideband CQI, STC rate and PMI from the base codebook shall be transmitted using short feedback period. When p > 0 and q > 0, the wideband CQI and STC rate shall be transmitted using short feedback period, and the correlation matrix shall be transmitted using long feedback period. When p = 0 and q > 0, only the correlation matrix shall be transmitted using long feedback period.

For MFM 7: when p > 0 and q = 0, the wideband CQI and PMI from the base codebook shall be transmitted using short feedback period. When p > 0 and q > 0, the wideband CQI shall be transmitted using short feedback period, and the correlation matrix shall be transmitted using long feedback period. When p = 0 and q > 0, only the correlation matrix shall be transmitted using long feedback period.

------------------------------------------------------- End of the Text -------------------------------------------------------