| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text of Transformation Codebook for the UL Single BS Precoding with Multi-BS Coordination (16.2.3.47, 16.5.2.1) |  |
| Date Submitted | 2010-03-05 |  |
| Source(s) | Wooram Shin, Jihyung Kim, Kwang Jae Lim ETRI 138, Gajeongno, Yuseong-gu, Daejeon 305-700, Korea | Voice: +82-42-860-6687 E-mail: w.shin@etri.re.kr |
| Re: | “P802.16m/D4,” in response to the IEEE 802.16 Working Group Letter Ballot #31: Announcement, IEEE 802.16-10/0011 |  |
| Abstract | Codebook transformation for single BS precoding with multi-BS coordination on the IEEE 802.16m Amendment Draft Standard |  |
| Purpose | To discuss and adopt the proposed text in the next revision of 802.16m draft |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text of Codebook Transformation for the UL Single BS Precoding with Multi-BS Coordination (16.2.3.47, 16.5.2.1)

Wooram Shin, Jihyung Kim, Kwang Jae Lim

ETRI

# Introduction

In IEEE 802.16m Amendment Draft Standard [1], for UL single BS precoding with multi-BS coordination, there could be potential achieving more gain if transformation codebook is adjusted as defined for DL MIMO, which exploits interference mitigation capability in conjunction with interference channel as well as desired channel. In order to support interference mitigating transformation codebook, two correlation matrices for the serving ABS and neighboring ABS corresponding to PMImin should be known to AMS, but this makes quite big signaling overhead in the MAC control message such as 56 bits for Nt=4. In case of TDD, however, AMS can measure the correlation matrix of interference channel from the neighboring ABS using downlink reference signals with the help of channel reciprocity if AMS has information which neighboring ABS is required to measure it. In this context, Temp_BSID or IDcell should be signaled via the AAI_SingleBS_PMI_COM message to designate the neighboring ABS corresponding to PMImin. IDcell can represent the every ABS where as it requires more than two bits for signaling than Temp_BSID. Also, usage indication of interference mitigating transformation codebook should be put into the AAI_SingleBS_PMI_COM message, and a type of BS ID used for measurement of correlation matrix to the neighboring ABS with the BS ID.

# Text Proposal for inclusion in the IEEE 802.16m Amendment

Note:

The text in black: the existing text in the 802.16m Amendment Draft Standard

The text in red with strike-out: the removal of existing 802.16m Amendment Draft Standard

The text in blue with under-line: the new text added to the 802.16m Amendment Draft Standard

[----------------------------------------------------Start of Text Proposal----------------------------------------------------]

[Change the texts and Table 730 in Section 16.2.3.47 as the following.]

16.2.3.47 AAI_MultiBS_PMI_COM

The AAI_MultiBS_PMI_ message format is defined in Table 730—. This message is used by ABS to indicate AMS the PMImin and, PMI combination ratio (PCR), and IMTC, BCID_type, and Temp_BSID or Neighbor_IDcell to AMS for the single BS precoding with multi-BS coordination.

Table 730—AAI_MultiBS_PMI_COM message format

| Name | Length (bits) | Description |
| --- | --- | --- |
| AAI_MultiBS_PMI_COM_Message_Format { |  |  |
| Control Message Type = xx |  |  |
| PMIMmin | 4 to 6 | PMI from the rank-1 base codebook£¨ generates minimum interference to the neighboring ABS |
| PCR | 2 | PMI combination ratio |
| IMTC | 1 | Interference mitigating transformation codebook indication 0b0: Base mode 0b1: Interference mitigating transformation mode Note that it shall be set to 0b0 if a duplex mode is not TDD. |
| If (IMTC == 0b1) { |  |  |
| BSID_type |  | Type of BS ID 0b0: Temp_BSID as defined in the AAI_DL_IM message 0b1: Neighbor_IDcell Temp_BSID is broadcasted through AAI_DL_IM message. |
| If (BSID_type == 0b0) { |  |  |
| Temp_BSID | 8 | Diversity set member ID Note that the ABS with Temp_BSID should be corresponding to the neighboring ABS sending PMImin. |
| } else { |  |  |
| Neighbor_IDcell | 10 | 0≤Neighbor_IDcell≤767 Note that the ABS with Neighbor_IDcell should be corresponding to the neighboring ABS sending PMImin. |
| } |  |  |
| } |  |  |
| } |  |  |

[Change the texts and Table 946 in Section 16.5.2.1 as the following.]

16.5.2.1 Single BS precoding with Multi-BS Coordination

This subclause describes interference mitigation techniques applicable withto UL MIMO modes 2 and 4 withof adaptive codebook precoding with rank-1, with additional inter-ABS coordination mechanisms and interference measurement support. The inter-ABS coordination mechanisms in this subclause do not require data forwarding between different ABSs.

16.5.2.1.1 DL Signaling

Table 946 shows the control parameters for UL multi-BS MIMO.

Table 946—Control parameters for UL Multi-BS MIMO

| Parameter | Description | Value | Control channel (IE) | Notes |
| --- | --- | --- | --- | --- |
| PMImin | PMI minimizing interference |  | AAI_MultiBS_PMI_COM messeage |  |
| PCR | PMI combination ratio |  | AAI_MultiBS_PMI_COM messeage |  |
| IMTC | Interference mitigating transformation codebook indication | 0b0: Base mode 0b1: Interference mitigating transformation mode | AAI_MultiBS_PMI_COM messeage | Transformation codebook can be generated as defined in Section 16.3.7.2.5.5.1, therein, but . |
| BSID_type | Type of BS ID | 0b0: Temp_BSID 0b1: Neighbor_IDcell | AAI_MultiBS_PMI_COM messeage |  |
| Temp _BSID | Diversity set member ID | 8 bits | AAI_MultiBS_PMI_COM messeage | This is signaled if IMTC is 0b1 and BSID_type is 0b0. |
| Neighbor_IDcell | Neighbor_IDcell | 10 bits | AAI_MultiBS_PMI_COM messeage | This is signaled if IMTC is 0b1 and BSID_type is 0b1. |

16.5.2.1.2 Operation procedure

Single BS precoding with Multi-BS Coordination is performed by combining two PMIs in TDD or FDD UL transmission for mitigating inter-cell interference (ICI) when CL MIMO precoding is applied for the serving and neighboring cells. One of PMIs maximizes the transmission power of the serving cell; another minimizes interference generated to the neighboring cell. PMI and PMImin may be selected from the transformation codebook for the serving and neighboring ABSs, respectively, if IMTC is 0b1.

The operation procedure of PMI combination follows the steps below.

- PMI combination may be triggered by an ABS in an unsolicited manner. By using UL sounding signals, the serving and neighboring ABSs may measure channels of AMSs. Upon the channel measurements, the serving and neighboring ABSs may determine PMI maximizing transmission power and PMImin minimizing interference, respectively. The neighboring ABS may inform the serving ABS of PMImin through the backhaul network.

Those Ttwo PMIs of the predefined codebook {wi},  and , should be selected based on the following procedure:

(312)

(312)

where HsHS and HIHI denote information of channels from the AMS to its serving and corresponding neighboring ABS, respectively, and vi is the i-th codeword of the base codebook.

If IMTC is 0b1, PMI and PMImin are selected from the transformation codebooks as follows:

(313)

where  and  are the i-th codeword of the transformation codebook with the correlation matrix to the serving and neighboring ABSs, respectively. They are of the form in Equation (314)

(314)

where RS and RI are the correlation matrix to the serving and neighboring ABSs, respectively, and they are measured as the following.

(315)

The correlation matrix to the neighboring ABS is measured and informed to the AMS by it via the backhaul network.

- The serving ABS shall inform the PMI in the UL Basic Assignment A-MAP IE or UL Subband Assignement A MAP-IE, and PMImin and PCR (PMI combination ratio) in anthe AAI_MultiBS_PMI_COM message to the AMS.

The transmitted precoder W combining two precoders  should be generated as follows:

(313)

(316)

(314)

If IMTC is 0b1,  and  are obtained from the transformation codebooks as defined in Equation (314). Hereupon, however, the AMS has no information on the correlation matrices to the serving and neighboring ABSs. In case of TDD, fortunately, the AMS can measure those two correlation matrices using downlink reference signals by virtue of channel reciprocity, and the correlation matrix to which neighboring ABS would be measured is known to the AMS by dedicating it with Temp_BSID or Neighbor_IDcell (depending on BSID_type).

[----------------------------------------------------End of Text Proposal----------------------------------------------------]

# Reference

[1] IEEE P802.16m/D4, “DRAFT Amendment to IEEE Standard for Local and Metropolitan Area Networks; Part 16: Air Interface for Broadband Wireless Access Systems; Advanced Air Interface,” February 2010