| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Power Status Reporting Change for “BR Without STID Header” Message (IEEE 802.16m/D4 Amendment) |  |
| Date Submitted | 2010-03-05 |  |
| Source(s) | Rongzhen Yang, Jie Hui, Joey Choi, Hujun Yin, Yang-seok Choi Intel Corp. | Email : Rongzhen.yang@intel.com |
| Re: | Comments on P802.16m/D4 |  |
| Abstract | This contribution provides the fixed version of “UL Tx Power” field for message “BR Without STID Header”. |  |
| Purpose | To be discussed and adopted into P802.16m/D4 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Power Status Reporting Change for “BR Without STID Header” Message (IEEE 802.16m/D4 Amendment)

Rongzhen Yang, Jie Hui, Joey Choi, Hujun Yin, Yang-seok Choi
Intel Corp.

- Introduction

The field “UL Tx Power” in message “BR Without STID Header” needs to be updated to 16m power status reporting format.

- Text proposal

Instruction to editor;

Black text: existing D4 text

Red and strike out text: for deletion

Blue and underline text: for addition

[Remedy 1:]

[Change the text from line 22 on the page 41, section 16.2.2.1.3.2 as follows:]

============================== Start of Proposed Text =================================

16.2.2.1.3.2 BR without STID Header

BR without STID Header is sent through dedicated UL resource assigned to the AMS. BR type shall be included to indicate whether BR type is increment or aggregate. BR format without STID header is defined in Table 658.

Table 658—BR without STID Header Format

| Syntax | Size(bit) | Notes |
| --- | --- | --- |
| BR Without STDI Header() { |  |  |
| FID | 4 | Flow Identifier. Set to 0001. |
| Type | 4 | MAC signaling header type = 0b0001 |
| BR Type | 1 | Indicates whether the requested bandwidth is incremental or aggregate: 0: incremental. 1: aggregate. |
| BR Size | 19 16 | Bandwidth request size in bytes. |
| BR FID | 4 | The FID for which UL bandwidth is requested. |
| TI | 1 | 0: No Tx Power report 1: Tx Power information present |
| If (TI = 1) {. |  |  |
| UL Tx Power | 8 | UL Tx power level in dBm for the burst that carries this header (as described in 11.1.1). |
| Reserved | 7 | Shall be filled by 0 |
| } else {. |  |  |
| Reserved | 15 | Shall be filled by 0 |
| } |  |  |

| M/O | Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| M | FID | 4 | Flow Identifier. Set to 0001. | N/A |
| M | Type | 4 | MAC signaling header type = 0b0001 | N/A |
| M | BR Type | 1 | Indicates whether the requested bandwidth is incremental or aggregate: 0: incremental. 1: aggregate. | N/A |
| M | BR Size | 16 | Bandwidth request size in bytes. | N/A |
| M | BR FID | 4 | The FID for which UL bandwidth is requested. | N/A |
| M | TI | 1 | 0: No Tx Power report 1: Tx Power information present | N/A |
|  | If (TI = 1) { |  |  |  |
| O | txPowerPsdBase | 8 | txPowerPsdBase (PSD(base) ) is coded using 8 bits in 0.5 dBm steps ranging from -74 dBm (coded 0x00) to 53.5 dBm (coded 0xFF). Detail reference to 16.3.9.4.7. | Present when TI = 1 |
| O | txSirDownlink | 10 | txSirDownlink (SIRDL ) is coded using 10 bits in 1/16 dB steps ranging from -12dB (coded 0x000) to 51.9375 dB (coded 0x3ff). Detail reference to 16.3.9.4.7. | Present when TI = 1 |
|  | } else { |  |  |  |
| O | Reserved | 18 | Shall be filled by 0 | Present when TI = 0 |
|  | } |  |  |  |

============================== End of Proposed Text =================================

[Remedy 2:]

Annex P.

(Normative)

Definition of AAI MAC control messages

Annex P.2 MAC Control Message Definitions

============================== Start of Proposed Text =================================

[Add the following ASN.1 code to Annex P.2]

============================== End of Proposed Text =================================