| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Signaling Support for 16m ULPC Initial Stage (IEEE 802.16m/D4 Amendment) |  |
| Date Submitted | 2010-03-05 |  |
| Source(s) | Rongzhen Yang, Xiangying Yang, Hujun Yin, Yang-seok Choi Intel Corp. | Email : Rongzhen.yang@intel.com |
| Re: | Comments on P802.16m/D4 |  |
| Abstract | This contribution provides the cleanup version for message AAI_UL_POWER_ADJUST. |  |
| Purpose | To be discussed and adopted into P802.16m/D4 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Signaling Support for 16m ULPC Initial Stage

Rongzhen Yang, Xiangying Yang, Hujun Yin, Yang-seok Choi
Intel Corp.

- Introduction

In current 16m ULPC signaling design, AMS how to get the power control parameters during initial entry and handover is not defined. This contribution tries to define the related signaling field in related messages to support seamless uplink power control in all cases.

- Text proposal

Instruction to editor;

Black text: existing D4 text

Blue and strike out text: for deletion

Red and underline text: for addition

[Remedy 1:]

[Change the text from line 1 on the page 100, section 16.2.3.3 as follows:]

============================== Start of Proposed Text I =================================

Table 678—Parameters for AAI_RNG-ACK

| Name | Value | Description |
| --- | --- | --- |
| Adjustment parameters indication (API) | Bit#0: Time offset adjustment indication. Bit#1: Power level adjustment indication Bit#2: Frequency offset adjustment indication | When the AAI_RNG-ACK is sent with broadcast STID, this parameter shall be included for each received ranging code when Ranging status is 0b01. When the AAI_RNG-ACK is sent with unicast STID, the parameter is included for the specific user when Ranging status is 0b01, and in this case Bit#2 is always set to 0. |
| Timing offset adjustment | 10 bits | Shall be included when Time offset adjustment indication bit is 1 |
| Power level adjustment | 3 bits | Shall be included when Power level adjustment indication bit is 1 |
| Frequency offset adjustment | 8 bits | Shall be included when Frequency offset adjustment indication bit is 1 |
| UlpcDataChannelSet Exclude all “gammaIoTFp” field | 12 bits | Present when one “success” ranging status exist, Power control parameters for data channel, detail reference to 16.2.3.29 AAI_SCD message |
| UlpcControlChannelSet | 35 bits | Present when one “success” ranging status exist, Power control parameters for control channel, detail reference to 16.2.3.29 AAI_SCD message |
| offsetData | 6 bits | Present when one “success” ranging status exist, offsetData initial value setting |
| offsetControl | 6 bits | Present when one “success” ranging status exist, offsetControl initial value setting |

============================== End of Proposed Text I=================================

[Remedy 2:]

[Change the text from line 51 on the page 83, include following text:]

============================== Start of Proposed Text 2 =================================

Table 686-Parameters for AAI_HO-CMD message

| Name | Value | Usage |
| --- | --- | --- |
| … | … | … |
| UlpcDataChannelSet | 28 bits | Present when CDMA_RNG_FLAG=0, Power control parameters for data channel of target handover ABS, detail reference to 16.2.3.29 AAI_SCD message |
| UlpcControlChannelSet | 35 bits | Present when CDMA_RNG_FLAG=0, Power control parameters for control channel of target handover ABS, detail reference to 16.2.3.29 AAI_SCD message |
| offsetData | 6 bits | Present when CDMA_RNG_FLAG=0, offsetData initial value setting in target ABS. |
| offsetControl | 6 bits | Present when CDMA_RNG_FLAG=0, offsetControl initial value setting in target ABS. |
| gammaIotFp | 4 | Present when CDMA_RNG_FLAG=0, gammaIotFp (IoT) is the fairness and IoT control factor for initial AMS resource assignment in target ABS. It has 4 bits to represent the value among {0, 0.1, 0.2, 0.3, 0.4, 0.5, 0.6, 0.7, 0.8, 0.9, 1.0, 1.1, 1.2, 1.3, 1.4, 1.5}, detail reference to 16.2.3.29 AAI_SCD message |

============================== End of Proposed Text 2=================================

[Remedy 3:]

[Change the text from line 16 on the page 504, modify table as follows:]

============================== Start of Proposed Text 3 =================================

Table 832—CDMA Allocation A-MAP IE

| … | … | … |
| --- | --- | --- |
| iotFP | 7 bits | IoT value of Frequency Partition used for initial AMS resource assignment, quantized in 0.5 dB steps as IoT level from 0 dB to 63.5 dB., detail reference to 16.2.3.30 AAI_ULPC_NI message |
| gammaIotFp | 4 | gammaIotFp (IoT) is the fairness and IoT control factor for initial AMS resource assignment. It has 4 bits to represent the value among {0, 0.1, 0.2, 0.3, 0.4, 0.5, 0.6, 0.7, 0.8, 0.9, 1.0, 1.1, 1.2, 1.3, 1.4, 1.5}, detail reference to 16.2.3.29 AAI_SCD message |
| Reserved | 154 | Reserved bits |

============================== End of Proposed Text 3=================================