| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Signaling Support for 16m ULPC Initial Stage (IEEE 802.16m/D4 Amendment) |  |
| Date Submitted | 2010-03-15 |  |
| Source(s) | Rongzhen Yang, Xiangying Yang, Hujun Yin, Yang-seok Choi Intel Corp. Tim Wilson Motorola Wookbong Lee, Dongcheol Kim LG Electronics Jeongho Park Samsung Zheng Yan-Xiu, Chang-Lan Tsai, Chung-Lien Ho, Yu-Chuan Fang ITRI Yu-Hao Chang, Yih-Shen Chen MediaTek | Email : Rongzhen.yang@intel.com Email : Tim.Wilson@motorola.com Email:wookbong.lee@lge.com, dongcheol.kim@lge.com Email : jeongho.jh.park@samsung.com E-mail: zhengyanxiu@itri.org.tw E-mail: yihshen.chen@mediatek.com |
| Re: | Comments on P802.16m/D4 |  |
| Abstract | This contribution provides the signaling design for 16m ULPC Initial Stage. |  |
| Purpose | To be discussed and adopted into P802.16m/D4 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Signaling Support for 16m ULPC Initial Stage

Rongzhen Yang, Xiangying Yang, Hujun Yin, Yang-seok Choi
Intel Corp.

Tim Wilson

Motorola

Wookbong Lee, Dongcheol Kim

LG Electronics

Jeongho Park

Samsung

Zheng Yan-Xiu, Chang-Lan Tsai, Chung-Lien Ho, Yu-Chuan Fang

ITRI

Yu-Hao Chang, Yih-Shen Chen

MediaTek

- Introduction

In current 16m ULPC signaling design, AMS how to get the power control parameters during initial entry and handover is not defined. This contribution tries to define the related signaling field in related messages to support seamless uplink power control in all cases.

- Text proposal

Instruction to editor;

Black text: existing D4 text

Blue and strike out text: for deletion

Red and underline text: for addition

[Remedy 1:]

[Change the text from line 51 on the page 83, include following text:]

============================== Start of Proposed Text 1 =================================

Table 686-Parameters for AAI_HO-CMD message

| Name | Value | Usage |
| --- | --- | --- |
| … | … | … |
| offsetData | 7 bits | Optional, Present when CDMA_RNG_FLAG=0. It represents the value among -31.5 to 32 dB with 0.5 dB step |
| offsetControl | 7 bits | Optional, Present when CDMA_RNG_FLAG=0. It represents the value among -31.5 to 32 dB with 0.5 dB step |

============================== End of Proposed Text 1=================================

[Remedy 2:]

[Change the text from line 16 on the page 504, modify table as follows:]

============================== Start of Proposed Text 2 =================================

Table 832—CDMA Allocation A-MAP IE

| … | … | … |
| --- | --- | --- |
| iotFP | 7 bits | IoT value of Frequency Partition used for AMS resource assignment, quantized in 0.5 dB steps as IoT level from 0 dB to 63.5 dB., detail reference to 16.2.3.30 AAI_ULPC_NI message |
| offsetControl | 6 | offsetControl is the transmit power adjustment value transmitted by the ABS. It represents the value among -15.5 to 16 dB with 0.5 dB step |
| Reserved | 152 | Reserved bits |

============================== End of Proposed Text 2=================================

[Remedy 3:]

[Insert the text start from line 34 on the page 661, one new subsection:]

============================== Start of Proposed Text 3 =================================

16.3.9.4.8 Uplink Power Control in Initial Network Entry and ReEntry

During initial network entry and reentry, after successful initial ranging process, ABS needs to send CDMA Allocation AMAP IE to AMS as shown in Figure 442 “State Machine of the ABS for the initial NW Entry Process”.

The key power control parameters NI and offsetControl, which are signaled by CDMA Allocation AMAP IE, are directly belong to the FP of assigned data burst when multi-FPs are used.

When AMS has only successful received the CDMA Allocation AMAP IE, AMS will set NI and offsetControl by the values from CDMA Allocation AMAP IE. Other uplink power control parameters are set by the default value listed in Table xxx.

Table xxx Default ULPC Parameters

| Type | Parameters | Default Value |
| --- | --- | --- |
| Data Channel Control Parameters | Alpha | 1/2 |
|  | Beta | 0 |
|  | Gamma | 0.5 |
|  | dataSinrMin | 3 dB |
|  | dataSinrMax | 40 dB |
| Control Channel Target SINR Value | targetHarqSinr | 2 dB |
|  | targetSyncRangingSinr | -3 dB |
|  | targetPfbchSinr | 0 dB |
|  | targetSfbchBaseSinr | 1.5 dB |
|  | targetSfbchDeltaSinr | 0.24 dB |
|  | targetBwRequestSinr | 0.5 dB |
|  | Sounding Channel SINR Target Value | 10 dB |
| Offset Initialization | offsetData | 0 |

Whenever AMS successfully receive AAI_SCD, AAI_ULPC_NI and AAI_UL_POWER_ADJ messages, the signaled uplink power control parameters will be updated accordingly.

16.3.9.4.9 Uplink Power Control in Handover

During the handover processing, if CDMA_RNG_FLAG in message AAI_HO-CMD is set to 0, CDMA based ranging will be skipped, the power offset values for target ABS will be included the AAI_HO-CMD, as shown in Table 686 “Parameters for AAI_HO-CMD message”. Other uplink power control parameters for target ABS are set by using the serving ABS value.

If CDMA_RNG_FLAG in message AAI_HO-CMD is set to 1, CDMA based ranging shall be performed, the power control parameters of target ABS will be got as the process defined in section 16.3.9.4.8.

Before the handover procedure has been finished, two sets of uplink power control parameters are kept in AMS to support uplink transmission to serving ABS and target ABS separately.

After handover, whenever AMS successfully receive AAI_SCD, AAI_ULPC_NI and AAI_UL_POWER_ADJ messages from target ABS, the signaled uplink power control parameters will be updated accordingly.

============================== End of Proposed Text 3=================================