| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Message Cleanup for Uplink Power Status Reporting (IEEE 802.16m/D4 Amendment) |  |
| Date Submitted | 2010-03-10 |  |
| Source(s) | Tim Wilson Motorola Rongzhen Yang, Shi Liu, Joey Chou, Hujun Yin, Yang-seok Choi Intel Corp Dongcheol Kim,Wookbong Lee LG Electronics | Email : Tim.Wilson@motorola.com Email : Rongzhen.yang@intel.com Voice : +82-31-450-7919 E-mail: dongcheol.kim@lge.com, wookbong.lee@lge.com |
| Re: | Comments on P802.16m/D4 |  |
| Abstract | This contribution provides the cleanup version for Uplink Power Status Reporting messages. |  |
| Purpose | To be discussed and adopted into P802.16m/D4 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Message Cleanup for Uplink Power Status Reporting (IEEE 802.16m/D4 Amendment)

Tim Wilson

Motorola

Rongzhen Yang, Shi Liu, Joey Chou, Hujun Yin, Yang-seok Choi 
Intel Corp.

Dongcheol Kim,Wookbong Lee

LG Electronics

- Introduction

The current text in AAI_UL_PSR_Config needs to be modified for message format and ASN.1 coding style. In addition, the expression range of reported parameter SIRDL is refined with detail analysis: -10dB~41.15dB is changed to -12dB~51.9375dB.

- Key Reported Parameters

There are two key parameters for power status reporting: the base uplink transmission PSD and the SIRDL defined in section 16.3.9.4.1.

The base uplink transmission PSD is derived from equation (287) by setting SINRTarget=0 and Offset=0:

PSD(base)=L+NI

The parameter is reported in dBm and is coded using 8 bits in 0.5 dBm steps ranging from -74 dBm (coded 0x00) to 53.5 dBm (coded 0xFF).

The reported SIRDLvalue is coded using 10 bits in 1/16 dB steps ranging from -12dB (coded 0x000) to 51.9375 dB (coded 0x3ff).

After the ABS has received PSD(base) and SIRDL from the AMS, the PSD for all uplink channels can be estimated in ABS.

For the range of reporting value, the PSD(base) is easily derived and directly related to the AMS transmission power, so we discuss the range of SIRDL in the detail:

Firstly we can give some definitions for better expression:

db(x)≡10log10(x)undb(x)≡10x/10y≡SINRtargetx≡db(SIRDL)

The equation for SINRtarget (equation 288) can be written as:

y=db(γ⋅undb(x)−α)−β⋅db(TNS)=db(undb(x+db(γ))−α)−β⋅db(TNS)

Note: the max[] part is omitted because it is not directly related to accuracy.

The derivative of y vs. x can be found to be:

dydx=1+αundb(y+db(TNS))

To find the maximum value of dy/dxwe set α to its maximum value ( =1) and undb(y+db(TNS)) to its minimum value. We can use y = -10dB and TNS=1, leading to a maximum value of dy/dxof 11. This means that, worst-case, the quantization step of y is 11 times larger than the quantization step of x.

The problem then becomes finding a suitable range and quantization step size for reporting db(SIRDL) that can represent SINRtarget over a suitable range and resolution, for all possible values of α, γ, and TNS.

The following figure shows the relation between SINRtarget and db(SIRDL)for extreme values of α,γ and TNS.γ=0 is not considered because in that case SINRtarget is just SINR_min and the value of db(SIRDL)is not important.

Fig.1 The relationship of SIRDL and SINRTarget

From the figure it is clear that if the AMS reports db(SIR_DL) using 10 bits, in 1/16 dB steps over the range [-12 .. 51.9375] dB, then the ABS can derive the value of SINR_target over the range [-10 .. 35.9] dB with worst-case 0.6875 dB quantization, for all cases of α, γ, and TNS.

- Text proposal

Instruction to editor;

Black text: existing D4 text

Red and strike out text: for deletion

Blue and underline text: for addition

[Remedy 1:]

[Change the text from line 24 on the page 119, section 16.2.3.32~33 as follows:]

============================== Start of Proposed Text =================================

16.2.3.32 AAI_Uplink Power Status Reporting Configuration (AAI_UL PSR_Config) message

An AAI_UL_PSR_Config message is used to config the AMS uplink power status reporting by unicasting. It is defined as:

---ASN.1START

AAI_UL_PSR_Config:: = CHOICE {

Disable_Report	NULL,

Enable_Report	SEQUENCE {

Tx_Power_Report_Threshold_dBINTEGER (0..15)

Tx_Power_Report_Minimum_IntervalINTEGER (0..15)

Tx_Power_Report_Periodical_Interval INTEGER (0..15)

}

}

---ASN.1STOP

Table 71011—AAI_UL_PSR_Config AAIUL-PSR-CFG message Field Descriptions

| Parameters | Description |
| --- | --- |
| Tx_Power_Report_Threshold_dB | Tx_Power_Report_Threshold_dB is a 4 bit unsigned integer value in 0.5 dB steps, the specific value "0b1111" shall be interpreted as "infinite" |
| Tx_Power_Report_Minimum_Interval | Tx_Power_Report_Minimum_Interval is coded by 4 bit unsigned integer values d representing 2d frames, the specific value d=0b1111 shall be interpreted as "infinite" |
| Tx_Power_Report_Periodical_Interval | Tx_Power_Report_Periodical_Interval is coded by 4 bit unsigned integer values d representing 2d frames, the specific value d=0b1111 shall be interpreted as "infinite" |

| M/O | Attributes / Array of attributes |  | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- | --- |
| M | uplinkPowerStatusReport |  | 1 | disable (0), enable (1) | N/A |
| O | psrReportConfigParameters | txPowerReportThreshold | 4 | txPowerReportThreshold is a 4 bit unsigned integer value in 0.5 dB steps, the specific value “0b1111” shall be interpreted as “infinite” | Present when uplinkPowerStatusReport is enabled |
|  |  | txPowerReportMinimumInterval | 4 | txPowerReportMinimumInterval is coded by 4 bit unsigned integer values d representing 2d frames, the specific value d=0b1111 shall be interpreted as "infinite" |  |
|  |  | txPowerReportPeriodicalInterval | 4 | txPowerReportPeriodicalInterval is coded by 4 bit unsigned integer values d representing 2d frames, the specific value d=0b1111 shall be interpreted as "infinite" |  |

============================== End of Proposed Text =================================

[Remedy 2:]

============================== Start of Proposed Text =================================

16.2.3.33 AAI_Uplink Power Status Report (AAI_UL-PSR) message

An AAI_UL-PSR message is used by the AMS to report uplink power status to ABS. It is defined as:

---ASN.1START

AAI_UL_PSR ::= SEQUENCE {

Tx_Power_PSD_Base 	INTEGER (0..255),

Tx_SIR_Download 		INTEGER (0..1024)

}

---ASN.1STOP

Table 71112—AAI_UL-PSR message Field Descriptions

| Parameters | Description |
| --- | --- |
| Tx_Power_PSD_Base | Tx_Power_PSD_Base (PSD(base) ) is coded using 8 bits in 0.5 dBm steps ranging from -74 dBm (coded 0x00) to 53.5 dBm (coded 0xFF). |
| Tx_SIR_Download | Tx_SIR_Download (SIRDL ) is coded using 10 bits in 0.05 dB steps ranging from -10dB (coded 0x000) to 41.15 dB (coded 0x3ff). |

| M/O | Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| M | txPowerPsdBase | 8 | txPowerPsdBase (PSD(base) ) is coded using 8 bits in 0.5 dBm steps ranging from -74 dBm (coded 0x00) to 53.5 dBm (coded 0xFF). | N/A |
| M | txSirDownlink | 10 | txSirDownlink (SIRDL ) is coded using 10 bits in 1/16 dB steps ranging from -12dB (coded 0x000) to 51.9375 dB (coded 0x3ff). | N/A |

============================== End of Proposed Text =================================

[Remedy 3:]

[Change the text from line18 on the page 660, section 16.3.9.4.7 as follows:]

============================== Start of Proposed Text =================================

16.3.9.4.7 Uplink Power Status Reporting

16.3.9.4.7.1 Power Status Reporting Information

There are two key parameters for power status reporting: the base uplink transmission PSD and the SIRDL defined in section 16.3.9.4.1.

The base uplink transmission PSD is derived from equation (287) by setting SINRTarget=0 and Offset=0:

PSD(base)=L+NI				(292)

The parameter is reported in dBm and is coded using 8 bits in 0.5 dBm steps ranging from -74 dBm (coded 0x00) to 53.5 dBm (coded 0xFF). The NI value used here is the NI value of the first FP supported by home sector.

The reported SIRDLvalue is coded using 10 bits in 0.05 1/16 dB steps ranging from -10 -12 dB (coded 0x000) to 41.15 51.9375 dB (coded 0x3ff).

After the ABS has received PSD(base) and SIRDL from the AMS, the PSD for all uplink channels can be estimated in ABS.

16.3.9.4.6.2 Status Transmission Condition

The AMS shall send the status reporting message AAI_UL_PSR_Message (TBD) AAI-UL-PSR (defined in 16.2.3.33) to the ABS when the conditions listed below are met:

M(nlast)−M(n)≥Tx_Power_Report_Threshold(dB)	(293)

M(nlast)−M(n)≥txPowerReportThreshold	(293)

and

n−nlast≥Tx_Power_Report_Minimum_Interval		(294)

n−nlast≥txPowerReportMinimumInterval		(294)

or

n−nlast≥Tx_Power_Report_Periodical_Interval		(295)

n−nlast≥txPowerReportPeriodicalInterval		(295)

Where,

M is defined in equation 296:

M=L+SINRTarget(Reported)			(296)

L is the pathloss defined in equation (287)

nlast is the frame index when the last AMS status reporting message was sent,.

n is the current frame index.

Tx_Power_Report_Threshold(dB) txPowerReportThreshold is a 4 bit unsigned integer value in 0.5 dB steps, the specific value “0b1111” shall be interpreted as “infinite”;

Tx_Power_Report_Minimum_Interval txPowerReportMinimumIntervaland Tx_Power_Report_Periodical_IntervaltxPowerReportPeriodicalIntervalare coded by 4 bit unsigned integer values d representing 2dframes, the specific value d=0b1111 shall be interpreted as “infinite”

The status reporting configuration parameters Tx_Power_Report_Threshold(dB), Tx_Power_Report_Minimum_Interval and Tx_Power_Report_Periodical_Interval txPowerReportThreshold, txPowerReportMinimumInterval and txPowerReportPeriodicalIntervalare sent from ABS to AMS in the message AAI_UL_PSR_Config (TBD) AAI-UL-PSR-CFG (defined in 16.2.3.32).

============================== End of Proposed Text =================================

[Remedy 4:]

Annex P.

(Normative)

Definition of AAI MAC control messages

Annex P.2 MAC Control Message Definitions

============================== Start of Proposed Text =================================

[Add the following ASN.1 code to Annex P.2]

-- Uplink Power Status Reporting Configuration

AAI-UL-PSR-CFG ::= SEQUENCE {

messageType                    OCTET STRING (SIZE(1)),

uplinkPowerStatusReport        ENUMERATED {

disable (0),

enable (1)},

-- Present when uplinkPowerStatusReport is enabled

psrReportConfigParameters      SEQUENCE {

txPowerRptThreshold          INTEGER (0..15),

txPowerRptMinInterval        INTEGER (0..15),

txPowerRptPeriodicInterval   INTEGER (0..15)} OPTIONAL

}

-- Uplink Power Status Report

AAI-UL-PSR ::= SEQUENCE {

messageType                    OCTET STRING (SIZE(1)),

txPowerPsdBase                 INTEGER (0..255),

txSirDownlink                  INTEGER (0..1024)

}

============================== End of Proposed Text =================================