| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Priority Based Uplink Random Access Strategy |  |
| Date Submitted | 2010-03-05 |  |
| Source(s) | Howon Lee, Jung-Min Moon, Dong-Ho Cho KAIST Institute for IT Convergence Bang Chul Jung Gyeongsang National University | hwlee@itc.kaist.ac.kr, jmmoon@comis.kaist.ac.kr bcjung@kaist.ac.kr |
| Re: | IEEE 802.16-10/0011, IEEE 802.16 Working Group Letter Ballot #31 |  |
| Abstract | This contribution proposes a priority based uplink random access strategy for contention-based random access bandwidth request. |  |
| Purpose | To be discussed and adopted to P802.16m/D4. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Priority Based Uplink Random Access Strategy

Howon Lee, Jung-Min Moon, Dong-Ho Cho

KAIST Institute for IT Convergence

Bang Chul Jung

Gyeongsang National University

# Introduction

IEEE 802.16m/D4 [1] describes a contention-based random access bandwidth request in Section 16.2.11.1.1. In order to utilize this mechanism for bandwidth requests, an access class is assigned to a service flow when the service flow is established or modified. In addition, a minimum access class, which is a minimum priority allowed to perform the contention-based random access bandwidth request, is advertised via an AAI_SCD message. Based on the access class of the service flow and the minimum access class informed by an ABS, the following diagram shows the overall procedure of the contention-based random access bandwidth request.

(Figure 1) Overall procedure of contention-based random access bandwidth request

The contention-based random access bandwidth request is useful with respect to the fact that the different amount of random access opportunities for each class of users (for example, high- and low-priority users) can be provided according to the minimum access class specified in an AAI_SCD message. However, this mechanism has the following drawbacks.

- Problem (a)

For the case where an AMS fails to perform the contention-based random access bandwidth request because of its lower access class than the minimum access class specified in an AAI_SCD message, the AMS should wait for the next AAI_SCD message and can perform the contention-based random access bandwidth request successfully only if the minimum access class in the next AAI_SCD message is lower than the AMS’s access class. However, there is no method to know the next minimum access class. Therefore, it can happen that the AMS may wait for the next several AAI_SCD messages but finally may fail to perform the random access within a delay bound of the packet the AMS intends to send.

- Problem (b)

For the case where the minimum access class is set to be a low value to provide more opportunities for low-priority users, any users whose access class is higher than the minimum access class specified in an AAI_SCD message can perform a contention-based random access bandwidth request. Then, it can happen that the high-priority users may fail to request the bandwidth while the low-priority users may success, although the high-priority users are considered more importantly.

# Proposed Remedy

In order to solve the problems stated in the previous section, we propose the following enhancement of the conventional contention-based random access bandwidth request, which is explained in the previous section.

- Pre-notification of a sequence of minimum access classes

In Problems in the previous section, we mentioned a problem caused by unknown values of future minimum access classes. In order to solve this problem, we suggest that an ABS may advertise not a minimum access class, but a sequence of minimum access classes through an AAI_SCD message. In other words, the ABS decides a sequence of minimum access classes that are correspondingly used to a series of successive bandwidth request channels. Then, these minimum access classes are announced at the same time. Fig. 2 shows an example in which a sequence of minimum access classes, which are applied 4 bandwidth request channels in each frame of a super frame, are simultaneously advertised. By using this method, a set of AMSs who have different access classes can select more preferable bandwidth request channels and can perform the contention-based random access bandwidth requests in these channels. Therefore, the overall success probability of bandwidth requests will be increased.

(Figure 2) An example of our proposed remedy

# Proposed Text

[Modify the text from line 26 on page 116 as follows:]

16.2.3.29 AAI_System Configuration Descriptor (SCD) Message

-- ASN1 START

AAI_SCD Message :: SEQUENCE {

…

BR_Channel Configuration MIN Access Class				INTERGER (0..4)	OPTIONAL

BR_Channel Configuration MIN Access Class of the (i+0)-th frame	INTERGER (0..4)	OPTIONAL

BR_Channel Configuration MIN Access Class of the (i+1)-th frame	INTERGER (0..4) 	OPTIONAL

BR_Channel Configuration MIN Access Class of the (i+2)-th frame	INTERGER (0..4) 	OPTIONAL

BR_Channel Configuration MIN Access Class of the (i+3)-th frame	INTERGER (0..4) 	OPTIONAL

…

}

-- ASN1 STOP

[Modify the text from line 36-49 on page 254 as follows:]

16.2.11.1.1 Contention-based random access bandwidth request

The ABS may advertise a minimum access class a sequence of minimum access classes in the BR Channel Configuration MIN Access Class element elements within the AAI_SCD for each frame in a superframe. This sequence of minimum access classes is maintained until another advertisement with the AAI_SCD. Based on the sequence of minimum access classes, the AMS can select the frame used for the contention-based random access in order to minimize collision. If no minimum access class is classes are advertised in the AAI_SCD that means that all access classes are allowed. An access class is assigned to a service flow via DSx MAC control messages during service flow establishment / modification. When an AMS has information to send and decides to use the contention-based random access bandwidth request, the AMS shall check if the information the AMS has to send is for an access class with priority higher than or equal to the minimum access class advertised by BR channel configuration in the AAI_SCD. If it is not (the minimum access class is classes are not sufficiently low such that the AMS access class is allowed), then the AMS shall wait until the BR channel configuration in the AAI_SCD advertises a sequence of minimum access classes, one of which is less than or equal to the access class of the data and the AMS. When the AMS access class is allowed, the AMS shall set its internal backoff window equal to Bandwidth request backoff start (or Ranging for initial ranging) and send a bandwidth request when the backoff timer expires.

# Reference

[1] IEEE P802.16m/D4, “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks: Part 16: Air Interface for Fixed and Mobile Broadband Wireless Access Systems”, Feb. 2010.