| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification of aGP definition (16.2.12.3.1) |  |
| Date Submitted | 2010-03-05 |  |
| Source(s) | Vladimir Yanover Alvarion Ltd | Voice: E-mail: vladimir.yanover@alvarion.com |
| Re: | LB#31 |  |
| Abstract | Clarification of aGP definition |  |
| Purpose | To be discussed and adopted by TGm for 802.16m amendment |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Clarification of aGP definition

# Introduction

Section 16.2.12.3.1 "Adaptive granting and polling service" in 802.16m/D4 is using term "stringent" with regard to QoS requirements. This language should be clarified or removed.  The parameters are GPI and Primary Grant Size. If the ABS doubles GPI and triples Primary Grant Size, should it be considered more or less stringent QoS requirements?  It's higher data rate but greater delay. This concerns also "minimal QoS  guarantees"

The contribution contains two possible solutions. 802.16m TG is requested to select one of them.

# Solution 1, Proposed Text with change marks

-------------------------- Beginning of the text

16.2.12.3.1 Adaptive granting and polling service

The set of QoS parameters associated with adaptive granting and polling services are categorized into primary QoS parameters and secondary QoS parameters. The ABS may grant or poll AMS periodically and may negotiate only primary SF QoS parameters, or both primary and secondary QoS parameters with the AMS. Initially, ABS uses primary SF QoS parameters including Primary Grant and Polling Interval (GPI) and Primary Grant Size. When both primary and secondary QoS parameters are negotiated, primary QoS parameters shall have more stringent QoS requirements than secondary QoS parameters. The admission control shall be done by considering the most stringent QoS requirement defined by the primary QoS parameters.  The  secondary  QoS  parameters  are  the  minimal  QoS  guarantees  that  the  scheduling  service  shall always provide.

During the service, the traffic characteristics and QoS requirement may change, for example silence-suppression  enabled  VoIP  alternates  between  talk  spurt  and  silence  period,  which  triggers  adaptation  of  the scheduling service  state  machine  as  described  below.  Adaptation of scheduling state includes switching between using primary and using secondary SF QoS parameters or changing of GPI/Grant size to values within the QoS flexibility range, i.e., without exceeding the maximal QoS requirement or violating the minimal QoS guarantee, that are defined by the primary or secondary SF QoS parameters when the traffic can be characterized by more than two QoS states.

---------------------- End of the text

# Solution 2, Proposed Text with change marks

-------------------------- Beginning of the text

16.2.12.3.1 Adaptive granting and polling service

The set of QoS parameters associated with adaptive granting and polling services are categorized into primary QoS parameters and secondary QoS parameters. The ABS may grant or poll AMS periodically and may negotiate only primary SF QoS parameters, or both primary and secondary QoS parameters with the AMS. Initially, ABS uses primary SF QoS parameters including Primary Grant and Polling Interval (GPI) and Primary Grant Size. When both primary and secondary QoS parameters are negotiated, primary QoS parameters shall have more stringent QoS requirements than secondary QoS parameters., which means that traffic rate requirement based on primary QoS parameters (Primary Grant Size /Primary GPI) is higher than the traffic rate requirement based on secondary QoS parameters (Secondary Grant Size/Secondary GPI).

The admission control shall be done by considering the most stringent QoS requirement defined by the primary QoS parameters.  The  secondary  QoS  parameters  are  the  minimal  QoS  guarantees  that  the  scheduling  service  shall always provide.

During the service, the traffic characteristics and QoS requirement may change, for example silence-suppression  enabled  VoIP  alternates  between  talk  spurt  and  silence  period,  which  triggers  adaptation  of  the scheduling service  state  machine  as  described  below.  Adaptation of scheduling state includes switching between using primary and using secondary SF QoS parameters or changing of GPI/Grant size to values within the QoS flexibility range, i.e., without exceeding the maximal QoS requirement or violating the minimal QoS guarantee, that are defined by the primary or secondary SF QoS parameters when the traffic can be characterized by more than two QoS states.

within the range outlined by GPI and Primary Grant Size values in primary and secondary QoS parameters sets.

---------------------- End of the text