| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | AAI_DSX-xxx messages clean up |  |
| Date Submitted | 2010-03-17 |  |
| Source(s) | Joey Chou Jie Hui Intel | E-mail: joey.chou@intel.com Jie.Hui@intel.com |
| Re: | TGm AWD: |  |
| Abstract | This contribution proposes text to Clean up AAI_DSX-xxx |  |
| Purpose | Adopt proposed text. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

- AAI_DSX-xxx message clean up

- Joey Chou, Jie Hui Intel

- Introduction

This contribution is intended to clean up AAI_DSX-xxx message by providing missing information, such as size, value, conditions for attrbutes that are required to build the message. It includes an attribute table and ASN.1 code.

- Proposed text

1. 	Proposed Text I

16.2.3.45 DSx MAC Control Message

16.2.3.45.1 AAI_DSA-REQ

------------------------------------------------- Start of proposed text I --------------------------------------------------

[Add table 687 at page 138, line 37]

Table 687: AAI_DSA-REQ

| M/O | Attributes / Array of attributes |  |  |  |  | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| M | Control Message Type |  |  |  |  | 8 |  | NA |
| M | Change Count |  |  |  |  | 4 | The change count of this transaction assigned by the sender. If new transaction is started, FID Change Count is incremented by one (modulo 16) by the sender. | NA |
| O | SFID |  |  |  |  | 32 |  | Present when an ABS initiates AAI_DSA-REQ |
| O | FID |  |  |  |  | 4 |  | Present when an ABS initiates AAI_DSA-REQ |
| M | Uplink/Downlink Indicator. | 1 | 0: uplink; 1:downlink | NA |
| O | Service Class Nameuplink |  |  |  |  | 2 to 128 | Null-terminated string of ASCII characters. The length of the string, including null-terminator may not exceed 128 bytes | Present when a predefined BS service configuration is used for this service flow. |
| O | Global Service Class Name field |  |  |  |  | variable (see 6.3.14.4.1) | Variable: Refer to Table 186 of 6.3.14.4.1 | Present when a predefined BS service configuration to be used for this service flow and are synchronized among all BS. |
| M | QoS parameter set type |  |  |  |  | 8 | Bit 0: Provisioned Set Bit 1: Admitted Set Bit 2: Active Set |  |
| O | QoS parameters |  | Traffic Priority parameter |  |  | 3 | 0 to 7—Higher numbers indicate higher priority Default 0 |  |
| O |  |  | Maximum Sustained Traffic Rate parameter |  |  | 32 | Rate (in bits per second) |  |
| O |  |  | Maximum Traffic Burst parameter |  |  | 32 | Burst size (bytes) |  |
| O |  |  | Minimum Reserved Traffic Rate parameter |  |  | 32 | Rate (in bits per second) |  |
| O |  |  | Maximum Latency parameter |  |  | 32 | Milliseconds |  |
| O |  |  | Fixed-Length Versus Variable-Length SDU Indicator parameter |  |  | 8 | 0: Variable-length SDUs 1: Fixed-length SDUs Default = 0 |  |
| O |  |  | Paging Preference parameter |  |  | 8 | 0: No paging generation 1: Paging generation |  |
| O |  |  | Request/Transmission Policy parameter |  |  | 8 | Bit 0: If this bit is set to 1, the service flow shall not use broadcast BR opportunities. (UL only) (see 6.3.5 and 6.3.5) Bit 1: If this bit is set to 1, the service flow shall not use multicast BR opportunities. (UL only) (see 6.3.5 and 6.3.5) Bit 2: If this bit is set to 1, the service flow shall not piggyback requests with data. (UL only) (see 6.3.5 and 6.3.5) Bit 3: If this bit is set to 1, the service flow shall not fragment data. Bit 4: If this bit is set to 1, the service flow shall not suppress payload headers (CS parameter). If bit 4 is set to’0’ and both the SS and the BS support PHS (according to 11.7.7.3), each SDU for this service flow shall be prefixed by a PHSI field, which may be set to 0 (see 5.2). If bit 4 is set to ‘1’, none of the SDUs for this service flow shall have a PHSI field. Bit 5: If this bit is set to 1, the service flow shall not pack multiple SDUs (or fragments) into single MAC PDUs. Bit 6: If this bit is set to 1, the service flow shall not include CRC in the MAC PDU. Bit 7: If this bit is set to 1, the service flow shall not compress payload headers using ROHC. If bit 7 is set to’0’ and both the SS and the BS support ROHC (according to 11.7.7.4), each SDU for this service flow shall be compressed using ROHC. If bit 7 is set to ‘1’, none of the SDUs shall be compressed. |  |
| O |  |  | UL Grant Scheduling Type parameter |  |  | 8 | 0: Reserved 1: Undefined (BS implementation-dependenta) 2: BE (default) 3: nrtPS 4: rtPS 5: Extended rtPS 6: UGS 7: aGP Service | Present for UL |
| O |  |  | Tolerated Jitter parameter |  |  | 32 | Milliseconds | Present when needed |
| O |  |  | Unsolicited Grant Interval parameter |  |  | 16 | Milliseconds | Present when UL scheduling type is UGS or ertPS |
| O |  |  | Unsolicited Polling Interval parameter |  |  | 16 | Milliseconds | Present when UL scheduling type is rtPS |
| O |  |  | Primary Grant Polling Interval parameter |  |  | 16 | Milliseconds | Present when UL scheduling type is aGP Service |
| O |  |  | Primary Grant Size parameter |  |  | 16 | Number of bytes. | Present when UL scheduling type is aGP Service |
| O |  |  | Secondary Grant Polling Interval parameter |  |  | 16 | Milliseconds | Present when UL scheduling type is aGP Service and if secondary parameter set is needed |
| O |  |  | Secondary Grant Size parameter |  |  | 16 | Number of bytes. | Present when UL scheduling type is aGP Service and if secondary parameter set is needed |
| O |  |  | Adaptation Method |  |  | 2 | 00:Implicit adaptation 01:Explicit adaptation, sustained 10: Explicit adaptation, one time only. | Present when UL scheduling type is aGP Service |
| O |  |  | Access Class |  |  | 2 | This parameter specifies the priority assigned to a service flow. This priority is used in prioritizing access requests as described in paragraph 16.2.11.1.1.. | Present for UL |
| O |  |  | Differentiated BR Timer |  |  | 6 | Grant reception timeout before contention-based BR is attempted again for the service flow. Value of range 1 ~ 64 frame(s). | Present for UL |
| O |  |  | Quick Access Predefined BR Index Parameter |  |  | variable | The mapping of predefined BR index used in quick access message to BR size and BR actions | Present for UL |
|  |  |  | MAC in-order delivery indicator. | 1 | Indicate whether or not the order of delivery in non-ARQ connection is preserved by the MAC. 0 : not preserved 1 : preserved. |  |
| O |  |  | Vendor Specific QoS Parameter |  |  | variable |  |  |
| O | Type of Data Delivery Services parameter |  |  |  |  | 8 | 0: Unsolicited Grant Service 1: Real-Time Variable Rate Service 2: Non-Real-Time Variable Rate Service 3: Best Effort Service 4: Extended Real-Time Variable Rate Service |  |
| O | SDU Inter-Arrival Interval parameter |  |  |  |  | 16 | SDU inter-arrival interval in the resolution of 0.5 ms |  |
| O | Time Base parameter |  |  |  |  | 16 | Time base in milliseconds |  |
| O | SDU Size parameter |  |  |  |  | 8 | Number of bytes. Default = 49. |  |
| O | Target SAID parameter |  |  |  |  | 16 | SAID onto which service flow is mapped |  |
| O | SCID |  |  |  |  | 4 | Sleep cycle ID |  |
| O | ARQ Parameters | ARQ Enable |  |  |  | 8 | 0 = ARQ Not Requested/Accepted 1 = ARQ Requested/Accepted |  |
| O |  | ARQ_WINDOW_SIZE |  |  |  | 16 | >0 and <=(ARQ_BSN_MODULUS/2) |  |
| O |  | ARQ_BLOCK_LIFETIME |  |  |  | 16 | 0 = Infinite 1–6553500 µs (100 µs granularity) |  |
| O |  | ARQ_SYNC_LOSS_TIMEOUT |  |  |  | 16 | 0 = Infinite 1–6553500 µs (100 µs granularity) |  |
| O |  | ARQ_RX_PURGE_TIMEOUT |  |  |  | 16 | 0 = Infinite 1–6553500 µs (100 µs granularity) |  |
| O |  | ARQ_SUB_BLOCK_SIZE |  |  |  | 8 |  |  |
| O |  | ARQ_ERROR_DETECTION_TIMEOUT. | 16 |  |  |
| O |  | ARQ_FEEDBACK_POLL_RETRY_TIMEOUT |  |  |  | 16 |  |  |
| O |  | RECEIVER_ARQ_ACK_PROCESSING_TIME |  |  |  | 1 | 0–255 |  |
|  | CS Specification parameter |  |  |  |  | 8 | 0: GPCS (Generic Packet Convergence Sublayer) 1: Packet, IPv4 2: Packet, IPv6 3: Packet, IEEE 802.3/Etherneta 4: Reserved 5: Packet, IPv4 over IEEE 802.3/Etherneta 6: Packet, IPv6 over IEEE 802.3/Etherneta 7: Reserved 8: Reserved 9: ATM 10: Reserved 11: Reserved 12: Reserved 13: Reserved 14: Packet, IPb 15: Multiprotocol flow 16–255 Reserved |  |
|  | CS encodingRulr |  |  |  |  | variable | 99 ATM 100 Packet, IPv4 101 Packet, IPv6 102 Packet, IEEE 802.3/Ethernet 104 Packet IPV4 over IEEE 802.3/Ethernet 105 Packet IPV6 over IEEE 802.3/Ethernet 106 Reserved 107 Reserved 108 Reserved 109 Reserved 110 Reserved 111 Reserved 112 GPCS (Generic Packet Convergence Sublayer) 113 Packet IP |  |
| O | Packet Classification Rule parameter |  | Classification Rule Priority field |  |  | 8 | 0–255 |  |
| O |  |  | Protocol field |  |  | 8 | protocol |  |
| O |  |  | IP Masked Source Address parameter |  |  | 8 (IPv4) or 32 (IPv6) | src, smask |  |
| O |  |  | IP Masked Destination Address parameter |  |  | 8 (IPv4) or 32 (IPv6) | dst, dmask |  |
| O |  |  | Protocol Source Port Range field |  |  | 32 | sportlow, sporthigh |  |
| O |  |  | Protocol Destination Port Range field |  |  | 32 | dportlow, dporthigh |  |
| O |  |  | IEEE 802.3/Ethernet Destination MAC Address parameter |  |  | 96 | dst, msk |  |
| O |  |  | IEEE 802.3/Ethernet Source MAC Address parameter |  |  | 96 | src, msk |  |
| O |  |  | Ethertype/IEEE 802.2 SAP |  |  | 24 | type, eprot1, eprot2 |  |
| O |  |  | IEEE 802.1D User Priority field |  |  | 16 | pri-low, pri-high Valid range: 0–7 for pri-low and pri-high |  |
| O |  |  | IEEE 802.1Q VLAN ID field |  |  | 16 | vlan_id1, vlan_id2 |  |
| O |  |  | Associated PHSI field |  |  | 8 | Index value |  |
| O |  |  | Packet Classification Rule Index field |  |  | 16 | Packet Classification Rule Index |  |
| O |  |  | Vendor-specific classification parameters |  |  | variable | Compound |  |
| O |  |  | IPv6 Flow Label field |  |  | 24 | Flow Label |  |
| O |  |  | Classification Action Rule |  |  | 8 | Bit 0: 0 = none. 1 = Discard packet Bit 1–7: Reserved. |  |
| O |  |  | IP Type of Service |  |  | 8 | Bit 0–Bit 1: reserved. Shall be set to 0b00 Bit 2–Bit 7: DSCP value |  |
| O |  |  | PHS DSC Action field |  |  | 2 | 0: Add PHS Rule 1: Set PHS Rule 2: Delete PHS Rule 3: Delete all PHS Rules |  |
| O |  |  | PHS Rule field |  | PHSI field | 8 | Index value |  |
| O |  |  |  |  | PHSF field | n | String of bytes suppressed |  |
| O |  |  |  |  | PHSM field | n | Bit 0: 0 = Don’t suppress first byte of the suppression field 1 = Suppress first byte of the suppression field Bit 1: 0 = Don’t suppress second byte of the suppression field 1 = Suppress second byte of the suppression field Bit x: 0 = Don’t suppress (x + 1) byte of the suppression field 1 = Suppress (x + 1) byte of the suppression field |  |
| O |  |  |  |  | PHSS field | 8 | Number of bytes in the suppression string |  |
| O |  |  |  |  | PHSV field | 1 | 0: Verify 1: Don’t verify |  |
| O |  |  |  |  | Vendor-specific PHS parameters | variable | Compound |  |
| O | FSN size TLV |  |  |  |  | 8 | 0: 3-bit FSN 1: 11-bit FSN Default = 1 |  |
| O | MBS service TLV |  |  |  |  | 8 | 0: No available MBS 1: MBS in Serving BS Only 2: MBS in a multi-BS Zone supporting macro-diversity 3: MBS in a multi-BS Zone not supporting macro-diversity 4–255: Reserved |  |
| O | MBS Zone Identifier Assignment parameter |  |  |  |  | 8 | MBS zone identifier (bits 6 through 0 are the MBS Zone Identifier, bit 7 is set to 0) |  |
| O | MBS contents ID. | variable |  |  |
| O | SN Feedback Enabled field |  |  |  |  | 8 | 0: SN feedback is disabled (default) 1: SN feedback is enabled |  |
| O | HARQ Service Flows field |  |  |  |  | 8 | 0: Non-HARQ (default) 1: HARQ connection |  |
| O | HARQ Channel Mapping |  |  |  |  | variable |  |  |
| O | PDU SN Extended Subheader for HARQ Reordering |  |  |  |  | 2 |  |  |
| O | ROHC Parameter |  |  | ROHC Max Context ID |  | 16 | Non-negative integer |  |
| O |  |  |  | Large Context IDs |  | 8 | 0: FALSE (Small Context ID) 1: TRUE (Large Context ID) 2–255: Reserved |  |
| O |  |  |  | ROHC MRRU |  | 16 | 0: no segmentation Otherwise: MRRU |  |
| O | Packet Error Rate (PER) |  |  |  |  | 8 | MSB (bit 7): 0 – PER measured by the application, post-ARQ and post- HARQ process 1 – PER measured on the airlink, before the application of ARQ and HARQ Bit 6: 0 – Interpret bits 0–5 as an integer %, i.e., if bits 0–5 are the binary representation of the integer N, then the PER = N/100 (= N%) 1–Interpret bits 0–5 as 10 times a negative exponent of 10, i.e., if bits 0–5 are the binary representation of the integer N, then the PER = 10-N/10 LSB 6 bits (bits 0–5): PER value If bit 6 =0, [0 to 63%] PER If bit 6=1, [~5x10–7 to 1x10–0] PER |  |
| O | PSC assignment |  |  |  |  | 8 | Bit 0–5: The Power_Saving_Class_ID of the PSC to which the CID of this service flow is to be added. Bit 6–7: Reserved |  |
| O | Regional Emergency Indication parameter |  |  |  |  | 8 | Bit 0: Emergency indication Bits 1–7: Reserved |  |
| O | MAC Header Type. | 1 | Indicates whether AGMH or CMH is presented at the start of MPDUs of the service flow 0 = AGMH (Advanced Generic MAC Header) 1 = CMH (Compact MAC header). |  |
| O | E-MBS Service |  |  |  |  | 1 | Indicates whether the MBS service is being requested or provided for the connection that is being setup. |  |
| O | E-MBS_Zone_ID. | 7 | Indicates an E-MBS zone where the connection for associated service flow is valid. |  |
| O | E-MBS Service Flow Parameter: |  |  |  |  | variable | Mapping of Multicast STID (MSTID) and FID |  |
| O | Physical Carrier Index |  |  |  |  | 6 | Target carrier which the AMS switches or is redirected by ABS to, only included in ABS initiated DSA-REQ | only included in ABS initiated DSA-REQ |

------------------------------------------------- End of proposed text I --------------------------------------------------

2. 	Proposed Text II

16.2.3.45.2 AAI_DSA-RSP

------------------------------------------------- Start of proposed text II --------------------------------------------------

[Add table 688 at page 139, line 30]

Table 688: AAI_DSA-RSP

| M/O | Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| M | Control Message Type | 8 |  | NA |
| M | FID Change Count | 4 | FID Change Count from corresponding the AAI_DSA-REQ message | NA |
| M | Confirmation Code | 1 | Zero indicates the request was successful. Nonzero indicates failure |  |

------------------------------------------------- End of proposed text II --------------------------------------------------

3. 	Proposed Text III

16.2.3.45.3 AAI_DSA-ACK

------------------------------------------------- Start of proposed text III --------------------------------------------------

[Add table 689 at page 139, line 41]

Table 689: AAI_DSA-ACK

| M/O | Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| M | Control Message Type | 8 |  | NA |
| M | FID Change Count | 4 | FID Change Count from corresponding the AAI_DSA-RSP message | NA |
| M | Confirmation Code | 1 | Zero indicates the request was successful. Nonzero indicates failure |  |

------------------------------------------------- End of proposed text III --------------------------------------------------

4. 	Proposed Text IV

16.2.3.45.4 AAI_DSC-REQ

------------------------------------------------- Start of proposed text IV --------------------------------------------------

[Add table 690 at page 139, line 65]

Table 690: AAI_DSC-REQ

| M/O | Attributes / Array of attributes |  |  |  | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- | --- | --- | --- |
| M | Control Message Type |  |  |  | 8 |  | NA |
| M | Change Count |  |  |  | 4 | The change count of this transaction assigned by the sender. If new transaction is started, FID Change Count is incremented by one (modulo 16) by the sender. | NA |
| O | SFID |  |  |  | 32 |  | Present when an ABS initiates AAI_DSA-REQ |
| O | FID |  |  |  | 4 |  | Present when an ABS initiates AAI_DSA-REQ |
| M | Uplink/Downlink Indicator. | 1 | 0: uplink; 1:downlink | NA |
| O | Service Class Name |  |  |  | 2 to 128 | Null-terminated string of ASCII characters. The length of the string, including null-terminator may not exceed 128 bytes | Present when a predefined BS service configuration is used for this service flow. |
| O | Global Service Class Name field |  |  |  | variable (see 6.3.14.4.1) | Variable: Refer to Table 186 of 6.3.14.4.1 | Present when a predefined BS service configuration to be used for this service flow and are synchronized among all BS. |
| M | QoS parameter set type |  |  |  | 8 | Bit 0: Provisioned Set Bit 1: Admitted Set Bit 2: Active Set |  |
| O | QoS parameters | Traffic Priority parameter |  |  | 3 | 0 to 7—Higher numbers indicate higher priority Default 0 | Present when needed |
| O |  | Maximum Sustained Traffic Rate parameter |  |  | 32 | Rate (in bits per second) | Present when needed |
| O |  | Maximum Traffic Burst parameter |  |  | 32 | Burst size (bytes) | Present when needed |
| O |  | Minimum Reserved Traffic Rate parameter |  |  | 32 | Rate (in bits per second) | Present when needed |
| O |  | Maximum Latency parameter |  |  | 32 | Milliseconds | Present when needed |
| O |  | Fixed-Length Versus Variable-Length SDU Indicator parameter |  |  | 8 | 0: Variable-length SDUs 1: Fixed-length SDUs Default = 0 | Present when needed |
| O |  | Paging Preference parameter |  |  | 8 | 0: No paging generation 1: Paging generation |  |
| O |  | Request/Transmission Policy parameter |  |  | 8 | Bit 0: If this bit is set to 1, the service flow shall not use broadcast BR opportunities. (UL only) (see 6.3.5 and 6.3.5) Bit 1: If this bit is set to 1, the service flow shall not use multicast BR opportunities. (UL only) (see 6.3.5 and 6.3.5) Bit 2: If this bit is set to 1, the service flow shall not piggyback requests with data. (UL only) (see 6.3.5 and 6.3.5) Bit 3: If this bit is set to 1, the service flow shall not fragment data. Bit 4: If this bit is set to 1, the service flow shall not suppress payload headers (CS parameter). If bit 4 is set to’0’ and both the SS and the BS support PHS (according to 11.7.7.3), each SDU for this service flow shall be prefixed by a PHSI field, which may be set to 0 (see 5.2). If bit 4 is set to ‘1’, none of the SDUs for this service flow shall have a PHSI field. Bit 5: If this bit is set to 1, the service flow shall not pack multiple SDUs (or fragments) into single MAC PDUs. Bit 6: If this bit is set to 1, the service flow shall not include CRC in the MAC PDU. Bit 7: If this bit is set to 1, the service flow shall not compress payload headers using ROHC. If bit 7 is set to’0’ and both the SS and the BS support ROHC (according to 11.7.7.4), each SDU for this service flow shall be compressed using ROHC. If bit 7 is set to ‘1’, none of the SDUs shall be compressed. |  |
| O |  | UL Grant Scheduling Type parameter |  |  | 8 | 0: Reserved 1: Undefined (BS implementation-dependenta) 2: BE (default) 3: nrtPS 4: rtPS 5: Extended rtPS 6: UGS 7: aGP Service | Present for UL |
| O |  | Tolerated Jitter parameter |  |  | 32 | Milliseconds | Present when needed |
| O |  | Unsolicited Grant Interval parameter |  |  | 16 | Milliseconds | Present when UL scheduling type is UGS or ertPS |
| O |  | Unsolicited Polling Interval parameter |  |  | 16 | Milliseconds | Present when UL scheduling type is rtPS |
| O |  | Primary Grant Polling Interval parameter |  |  | 16 | Milliseconds | Present when UL scheduling type is aGP Service |
| O |  | Primary Grant Size parameter |  |  | 16 | Number of bytes. | Present when UL scheduling type is aGP Service |
| O |  | Secondary Grant Polling Interval parameter |  |  | 16 | Milliseconds | Present when UL scheduling type is aGP Service and if secondary parameter set is needed |
| O |  | Secondary Grant Size parameter |  |  | 16 | Number of bytes. | Present when UL scheduling type is aGP Service and if secondary parameter set is needed |
| O | SDU Inter-Arrival Interval parameter |  |  |  | 16 | SDU inter-arrival interval in the resolution of 0.5 ms | Present when needed |
| O | Time Base parameter |  |  |  | 16 | Time base in milliseconds | Present when needed |
|  | CS encodingRulr |  |  |  | variable | 99 ATM 100 Packet, IPv4 101 Packet, IPv6 102 Packet, IEEE 802.3/Ethernet 104 Packet IPV4 over IEEE 802.3/Ethernet 105 Packet IPV6 over IEEE 802.3/Ethernet 106 Reserved 107 Reserved 108 Reserved 109 Reserved 110 Reserved 111 Reserved 112 GPCS (Generic Packet Convergence Sublayer) 113 Packet IP |  |
| O | Classifier DSC Action |  |  |  | 8 | 0: DSC Add Classifier 1: DSC Replace Classifier 2: DSC Delete Classifier | Present for AAI_DSC-REQ |
| O | Packet Classification Rule parameter | Classification Rule Priority field |  |  | 8 | 0–255 |  |
| O |  | Protocol field |  |  | 8 | protocol |  |
| O |  | IP Masked Source Address parameter |  |  | 8 (IPv4) or 32 (IPv6) | src, smask |  |
| O |  | IP Masked Destination Address parameter |  |  | 8 (IPv4) or 32 (IPv6) | dst, dmask |  |
| O |  | Protocol Source Port Range field |  |  | 32 | sportlow, sporthigh |  |
| O |  | Protocol Destination Port Range field |  |  | 32 | dportlow, dporthigh |  |
| O |  | IEEE 802.3/Ethernet Destination MAC Address parameter |  |  | 96 | dst, msk |  |
| O |  | IEEE 802.3/Ethernet Source MAC Address parameter |  |  | 96 | src, msk |  |
| O |  | Ethertype/IEEE 802.2 SAP |  |  | 24 | type, eprot1, eprot2 |  |
| O |  | IEEE 802.1D User Priority field |  |  | 16 | pri-low, pri-high Valid range: 0–7 for pri-low and pri-high |  |
| O |  | IEEE 802.1Q VLAN ID field |  |  | 16 | vlan_id1, vlan_id2 |  |
| O |  | Associated PHSI field |  |  | 8 | Index value |  |
| O |  | Packet Classification Rule Index field |  |  | 16 | Packet Classification Rule Index |  |
| O |  | Vendor-specific classification parameters |  |  | variable | Compound |  |
| O |  | IPv6 Flow Label field |  |  | 24 | Flow Label |  |
| O |  | Classification Action Rule |  |  | 8 | Bit 0: 0 = none. 1 = Discard packet Bit 1–7: Reserved. |  |
| O |  | IP Type of Service |  |  | 8 | Bit 0–Bit 1: reserved. Shall be set to 0b00 Bit 2–Bit 7: DSCP value |  |
| O |  | PHS DSC Action field |  |  | 2 | 0: Add PHS Rule 1: Set PHS Rule 2: Delete PHS Rule 3: Delete all PHS Rules |  |
| O |  | PHS Rule field |  | PHSI field | 8 | Index value |  |
| O |  |  |  | PHSF field | n | String of bytes suppressed |  |
| O |  |  |  | PHSM field | n | Bit 0: 0 = Don’t suppress first byte of the suppression field 1 = Suppress first byte of the suppression field Bit 1: 0 = Don’t suppress second byte of the suppression field 1 = Suppress second byte of the suppression field Bit x: 0 = Don’t suppress (x + 1) byte of the suppression field 1 = Suppress (x + 1) byte of the suppression field |  |
| O |  |  |  | PHSS field | 8 | Number of bytes in the suppression string |  |
| O |  |  |  | PHSV field | 1 | 0: Verify 1: Don’t verify |  |
| O |  |  |  | Vendor-specific PHS parameters | variable | Compound |  |
| O | MBS service TLV |  |  |  | 8 | 0: No available MBS 1: MBS in Serving BS Only 2: MBS in a multi-BS Zone supporting macro-diversity 3: MBS in a multi-BS Zone not supporting macro-diversity 4–255: Reserved |  |
| O | MBS Zone Identifier Assignment parameter |  |  |  | 8 | MBS zone identifier (bits 6 through 0 are the MBS Zone Identifier, bit 7 is set to 0) |  |
| O | SN Feedback Enabled field |  |  |  | 8 | 0: SN feedback is disabled (default) 1: SN feedback is enabled |  |
| O | ROHC Parameter |  | ROHC Max Context ID |  | 16 | Non-negative integer |  |
| O |  |  | Large Context IDs |  | 8 | 0: FALSE (Small Context ID) 1: TRUE (Large Context ID) 2–255: Reserved |  |
| O |  |  | ROHC MRRU |  | 16 | 0: no segmentation Otherwise: MRRU |  |
| O | Packet Error Rate (PER) |  |  |  | 8 | MSB (bit 7): 0 – PER measured by the application, post-ARQ and post- HARQ process 1 – PER measured on the airlink, before the application of ARQ and HARQ Bit 6: 0 – Interpret bits 0–5 as an integer %, i.e., if bits 0–5 are the binary representation of the integer N, then the PER = N/100 (= N%) 1–Interpret bits 0–5 as 10 times a negative exponent of 10, i.e., if bits 0–5 are the binary representation of the integer N, then the PER = 10-N/10 LSB 6 bits (bits 0–5): PER value If bit 6 =0, [0 to 63%] PER If bit 6=1, [~5x10–7 to 1x10–0] PER |  |
| O | PSC assignment |  |  |  | 8 | Bit 0–5: The Power_Saving_Class_ID of the PSC to which the CID of this service flow is to be added. Bit 6–7: Reserved |  |
| O | Regional Emergency Indication parameter |  |  |  | 8 | Bit 0: Emergency indication Bits 1–7: Reserved |  |
| O | ? |  |  |  | 8 | Bit 0–3: indicates the class of un-scheduled alert. When the bit is set to 1, it indicates the type of alert to be transmitted in this service flow. Bit 0: Priority 0 Bit 1: Priority 1 Bit 2: Priority 2 Bit 3: Priority 3 Bits 4–7: Reserved |  |

------------------------------------------------- End of proposed text IV --------------------------------------------------

5. 	Proposed Text V

16.2.3.45.5 AAI_DSC-RSP

------------------------------------------------- Start of proposed text V --------------------------------------------------

[Add table 691 at page 140, line 31]

Table 691: AAI_DSC-RSP

| M/O | Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| M | Control Message Type | 8 |  | NA |
| M | FID Change Count | 4 | FID Change Count from corresponding the AAI_DSC-REQ message | NA |
| O | SFID | 32 |  | Present when an ABS initiates AAI_DSA-REQ |
| O | Confirmation Code | 1 | Zero indicates the request was successful. Nonzero indicates failure |  |

------------------------------------------------- End of proposed text V --------------------------------------------------

6. 	Proposed Text VI

16.2.3.45.6 AAI_DSC-ACK

------------------------------------------------- Start of proposed text VI --------------------------------------------------

[Add table 692 at page 140, line 42]

Table 692: AAI_DSC-ACK

| M/O | Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| M | Control Message Type | 8 |  | NA |
| M | FID Change Count | 4 | FID Change Count from corresponding the AAI_DSC-RSP message | NA |
| O | Confirmation Code | 1 | Zero indicates the request was successful. Nonzero indicates failure |  |

------------------------------------------------- End of proposed text  VI --------------------------------------------------

7. 	Proposed Text VII

16.2.3.45.7 AAI_DSD-REQ

------------------------------------------------- Start of proposed text VII --------------------------------------------------

[Add table 693 at page 140, line 54]

Table 693: AAI_DSD-REQ

| M/O | Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| M | Control Message Type | 8 |  | NA |
| M | Change Count | 4 | The change count of this transaction assigned by the sender. If new transaction is started, FID Change Count is incremented by one (modulo 16) by the sender. | NA |
| O | FID | 4 | Flow identifier to be deleted. | Present when an ABS initiates AAI_DSA-REQ |

------------------------------------------------- End of proposed text VII --------------------------------------------------

8. 	Proposed Text VIII

16.2.3.45.5 AAI_DSD-RSP

------------------------------------------------- Start of proposed text VIII --------------------------------------------------

[Add table 694 at page 141, line 4]

Table 694: AAI_DSD-RSP

| M/O | Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| M | Control Message Type | 8 |  | NA |
| M | FID Change Count | 4 | FID Change Count from corresponding the AAI_DSD-REQ message | NA |
| O | FID | 4 |  | Present when an ABS initiates AAI_DSA-REQ |
| O | Confirmation Code | 1 | Zero indicates the request was successful. Nonzero indicates failure |  |

------------------------------------------------- End of proposed text  VIII --------------------------------------------------

9. 	Proposed Text IX

Annex P.

(Normative)

Definition of AAI MAC control messages

Annex P.2 MAC Control Message Definitions

------------------------------------------------- Start of proposed text IX --------------------------------------------------

[Add the following ASN.1 code Page 763 Line 22]

-- AAI_DSA-REQ Message

AAI-DSA-REQ ::= SEQUENCE {

messageType                    OCTET STRING (SIZE(1)),

sfid                           INTEGER (0..4294967295) OPTIONAL,

fid                            INTEGER (0..15) OPTIONAL,

directionIndicator             DirectionIndicatorSet,

serviceClassName               ServiceClassNameSet OPTIONAL,

globalServiceClass             GlobalServiceClassName OPTIONAL,

qosPrarmeterSetType            QosPrarmeterSet OPTIONAL,

qoSParameters                  QosPrameterSet OPTIONAL,

sduSize                        INTEGER (0..255) OPTIONAL,

targetSaid                     INTEGER (0..65535) OPTIONAL,

sleepCycleId                   INTEGER (0..15) OPTIONAL,

arqAttributes                  ArqAttributeSet OPTIONAL,

csSpecificationType            CsSpecificationSet OPTIONAL,

csEncodingRule                 CsEncodingRuleSet OPTIONAL,

classificationRules            ClassificationRuleSet OPTIONAL,

fsnSize                        FsnSizeSet OPTIONAL,

mbsService                     MbsServiceSet OPTIONAL,

dataDeliveryServices           DataDeliveryServicesSet OPTIONAL,

sduInterArrival                INTEGER (0..65535) OPTIONAL, -- 0.5ms

timeBase                       INTEGER (0..65535) OPTIONAL, -- ms

mbsZoneIdentifier              INTEGER (0..64) OPTIONAL,

snFeedback                     SnFeedbackSet OPTIONAL,

harqServiceFlow                HarqServiceFlowSet OPTIONAL,

pduSnForHarqReordering         INTEGER (0..3) OPTIONAL,

rohcAttributes                 RohcAttributesSet OPTIONAL,

packetErrorRate                PacketErrorRateSet OPTIONAL,

pscAssignment                  INTEGER (0..64) OPTIONAL,

emergencyIndication            BIT STRING (SIZE(1)) OPTIONAL,

alertClass                     AlertClassSet OPTIONAL,

macHeaderType                  ENUMERATED {

agmh (0),

cmh  (1)} OPTIONAL,

embsService                    ENUMERATED {

embsNotProvided (0),

embsProvided    (1)} OPTIONAL,

embsZoneId                     INTEGER (0..127) OPTIONAL,

phyCarrierIndex                INTEGER (0..64) OPTIONAL

}

-- AAI_DSA-RSP Message

AAI-DSA-RSP ::= SEQUENCE {

messageType                    OCTET STRING (SIZE(1)),

confirmationCode               ConfirmationCodeSet}

-- AAI_DSA-ACK Message

AAI-DSA-ACK ::= SEQUENCE {

messageType                    OCTET STRING (SIZE(1)),

confirmationCode               ConfirmationCodeSet}

-- AAI_DSC-REQ Message

AAI-DSC-REQ ::= SEQUENCE {

messageType                    OCTET STRING (SIZE(1)),

sfid                           INTEGER (0..4294967295) OPTIONAL,

fid                            INTEGER (0..15) OPTIONAL,

directionIndicator             DirectionIndicatorSet,

serviceClassName               ServiceClassNameSet OPTIONAL,

globalServiceClass             GlobalServiceClassName OPTIONAL,

qosPrarmeterSetType            QosPrarmeterSet OPTIONAL,

qoSParameters                  QosPrameterSet OPTIONAL,

csEncodingRule                 CsEncodingRuleSet OPTIONAL,

classifierDscAction            ClassifierDscActionSet OPTIONAL,

classificationRules            ClassificationRuleSet OPTIONAL,

mbsService                     MbsServiceSet OPTIONAL,

sduInterArrival                INTEGER (0..65535) OPTIONAL, -- 0.5ms

timeBase                       INTEGER (0..65535) OPTIONAL, -- ms

mbsZoneIdentifier              INTEGER (0..64) OPTIONAL,

snFeedback                     SnFeedbackSet OPTIONAL,

rohcAttributes                 RohcAttributesSet OPTIONAL,

packetErrorRate                PacketErrorRateSet OPTIONAL,

pscAssignment                  INTEGER (0..64) OPTIONAL,

emergencyIndication            BIT STRING (SIZE(1)) OPTIONAL,

alertClass                     AlertClassSet OPTIONAL}

-- AAI_DSC-RSP Message

AAI-DSC-RSP ::= SEQUENCE {

messageType                    OCTET STRING (SIZE(1)),

confirmationCode               ConfirmationCodeSet}

-- AAI_DSC-ACK Message

AAI-DSC-ACK ::= SEQUENCE {

messageType                    OCTET STRING (SIZE(1)),

confirmationCode               ConfirmationCodeSet}

-- AAI_DSD-REQ Message

AAI-DSD-REQ ::= SEQUENCE {

messageType                    OCTET STRING (SIZE(1)),

fid                            INTEGER (0..15) OPTIONAL}

-- AAI_DSD-RSP Message

AAI-DSD-RSP ::= SEQUENCE {

messageType                    OCTET STRING (SIZE(1)),

fid                            INTEGER (0..15) OPTIONAL,

confirmationCode               ConfirmationCodeSet}

ServiceClassNameSet ::= CHOICE {

scName2Bytes                   OCTET STRING (SIZE(2)),

scName8Bytes                   OCTET STRING (SIZE(8)),

scName16Bytes                  OCTET STRING (SIZE(16)),

scName32Bytes                  OCTET STRING (SIZE(32)),

scName64Bytes                  OCTET STRING (SIZE(64)),

scName96Bytes                  OCTET STRING (SIZE(96)),

scName128Bytes                 OCTET STRING (SIZE(128))}

GlobalServiceClassName ::= SEQUENCE {

directionIndicator             DirectionIndicatorSet OPTIONAL,

maxSustainedRate               INTEGER (0..63) OPTIONAL,

maxTrafficBurst                INTEGER (0..63) OPTIONAL,

minReservedTrafficRate         INTEGER (0..63) OPTIONAL,

maxLatency                     INTEGER (0..63) OPTIONAL,

sduLengthIndicator             SduLengthIndicatorSet OPTIONAL,

pagingPreference               PagingPreferenceSet OPTIONAL,

reqTransmissionPolicy          ReqTransmissionPolicySet OPTIONAL,

ulGrantScheduling              UlGrantSchedulingSet OPTIONAL,

toleratedJitter                INTEGER (0..63) OPTIONAL,

trafficPriority                INTEGER (0..7) OPTIONAL,

unsolicitedGrantInterval       INTEGER (0..63) OPTIONAL,

unsolicitedPollingInterval     INTEGER (0..63) OPTIONAL}

QosPrarmeterSet ::=                BIT STRING {

provisionedSet (0),

admittedSet    (1),

activeSet      (2)} (SIZE(3))

DirectionIndicatorSet ::=          ENUMERATED {

uplink (0),

downlink (1)}

ConfirmationCodeSet ::=            ENUMERATED {

successful (0),

failure    (1)}

QosPrameterSet ::= SEQUENCE {

trafficPriority                INTEGER (0..7) OPTIONAL,

maxSustainedRate               INTEGER (0..4294967295) OPTIONAL, -- bps

maxTrafficBurst                INTEGER (0..4294967295) OPTIONAL, -- bytes

minReservedTrafficRate         INTEGER (0..4294967295) OPTIONAL, -- bps

maxLaytency                    INTEGER (0..4294967295) OPTIONAL, -- ms

sduLengthIndicator             SduLengthIndicatorSet OPTIONAL,

pagingPreference               PagingPreferenceSet OPTIONAL,

reqTransmissionPolicy          ReqTransmissionPolicySet OPTIONAL,

ulGrantScheduling              UlGrantSchedulingSet OPTIONAL,

toleratedJitter                INTEGER (0..4294967295) OPTIONAL, -- ms

unsolicitedGrantInterval       INTEGER (0..65535) OPTIONAL, -- ms

unsolicitedPollingInterval     INTEGER (0..65535) OPTIONAL, -- ms

priGrantPollingInterval        INTEGER (0..65535) OPTIONAL, -- ms

secGrantPollingInterval        INTEGER (0..65535) OPTIONAL, -- ms

priGrantSize                   INTEGER (0..65535) OPTIONAL, -- bytes

secGrantSize                   INTEGER (0..65535) OPTIONAL, -- bytes

adaptationMethod               ENUMERATED {

implicit (0),

explicitSustained (1),

explicitOneTime (2)} OPTIONAL,

accessClass                    INTEGER (0..3) OPTIONAL,

differentiatedBrTimer          INTEGER (0..63) OPTIONAL,

quickAccessBrIndex             INTEGER (0..255) OPTIONAL,

macInOrderDelivery             ENUMERATED {

notPreserved (0),

preserved (1)} OPTIONAL

}

UlGrantSchedulingSet ::=           ENUMERATED {

undefined  (1),

bestEffort (2),

nrtPs      (3),

rtPs       (4),

ertPs      (5),

ugs        (6)}

ReqTransmissionPolicySet ::=       BIT STRING {

broadcastBr     (0),

multicastBr     (1),

piggyback       (2),

noFragmentation (3),

noPhs           (4),

noPcking        (5),

noCrc           (6),

noRohc          (7)} (SIZE(8))

PagingPreferenceSet ::=            ENUMERATED {

noPagingGeneration (0),

pagingGeneration (1)}

SduLengthIndicatorSet ::=          ENUMERATED {

variableLength (0),

fixedLength (1)}

DataDeliveryServicesSet ::=        ENUMERATED {

ugs    (1),

rtVrs  (2),

nrtVrs (3),

be     (4),

ertVrs (5)}

CsSpecificationSet ::=             ENUMERATED {

gpcs               (0),

packetIpv4         (1),

packetIpv6         (2),

packetEthernet     (3),

packetIpv4Ethernet (4),

packetIpv6Ethernet (5),

atm                (6),

packetIpv4OrIpv6   (7),

multiProtocol      (8)}

CsEncodingRuleSet ::=              BIT STRING {

atm                (0),

packetIpv4         (1),

packetIpv6         (2),

packetEthernet     (3),

packetIpv4Ethernet (4),

packetIpv6Ethernet (5),

gpcs               (6),

packetIpv          (7)} (SIZE(8))

ClassifierDscActionSet ::=         ENUMERATED {

dscAddClassifier     (0),

dscReplaceClassifier (1),

dscDeleteClassifier  (2)}

FsnSizeSet ::=                     ENUMERATED {

fsn3Bits  (0),

fsn11bits (1)}

MbsServiceSet ::=                  ENUMERATED {

noMbs              (0),

mbsInServingBs     (1),

noMacroDiversity   (2),

withMacroDiversity (3)}

SnFeedbackSet ::=                  ENUMERATED {

snFeedbackDisable (0),

snFeedbackEnable  (1)}

HarqServiceFlowSet ::=             ENUMERATED {

noHarq   (0),

withHarq (1)}

PacketErrorRateSet ::=             SEQUENCE {

perMeasurement               ENUMERATED {

postArqHarq   (0),

beforeArqHarq (1)},

perInterpretation            ENUMERATED {

binary         (0),

negExponential (1)},

per                          INTEGER (0..64)}

AlertClassSet ::=                  BIT STRING {

priority0 (0),

priority1 (1),

priority2 (2),

priority3 (3)} (SIZE(4))

ArqAttributeSet ::= SEQUENCE {

arqEnable                      ENUMERATED {

arqNotRequested (0),

arqRequested    (1)} OPTIONAL,

-- > 0 and <= (ARQ_BSN_MODULUS/2)

arqWindowSize                  INTEGER (1..65535) OPTIONAL, --

arqBlockLifeTime               INTEGER (0..65535) OPTIONAL, -- 100us

arqSyncLossTimeout             INTEGER (0..65535) OPTIONAL, -- 100us

arqPurgeTimeout                INTEGER (0..65535) OPTIONAL, -- 100us

arqSubBlockSize                INTEGER (0..65535) OPTIONAL,

arqErrorDetectionTimeout       INTEGER (0..65535) OPTIONAL,

arqFbkPollRetryTimeout         INTEGER (0..65535) OPTIONAL,

arqAckProcssingTime            INTEGER (0..255) OPTIONAL

}

ClassificationRuleSet ::= SEQUENCE {

priority                       INTEGER (0..255) OPTIONAL,

protocolField                  INTEGER (0..255) OPTIONAL,

ipMaskedSrcAddr                CHOICE {

ipV4Addr                     INTEGER (0..255),

ipV6Addr                     INTEGER (0..4294967295)} OPTIONAL,

ipMaskedDestAddr               CHOICE {

ipV4Addr                     INTEGER (0..255),

ipV6Addr                     INTEGER (0..4294967295)} OPTIONAL,

protocolSrcPortrange           SEQUENCE {

lowPortNumber                INTEGER (0..65535),

highPortNumber               INTEGER (0..65535)} OPTIONAL,

protocolDestPortrange          SEQUENCE {

lowPortNumber                INTEGER (0..65535),

highPortNumber               INTEGER (0..65535)} OPTIONAL,

destMacAddress                 SEQUENCE {

macAddr                      OCTET STRING (SIZE(6)),

macAddrMask                  OCTET STRING (SIZE(6))} OPTIONAL,

secMacAddress                  SEQUENCE {

macAddr                      OCTET STRING (SIZE(6)),

macAddrMask                  OCTET STRING (SIZE(6))} OPTIONAL,

layer3ProtocolId               CHOICE {

type0                        NULL,

type1                        SEQUENCE {

ePort1                     INTEGER (0..65535),

ePort2                     INTEGER (0..65535)},

type2                        SEQUENCE {

ePort1                     INTEGER (0..255),

ePort2                     INTEGER (0..255)}} OPTIONAL,

userPriorityrange              SEQUENCE {

lowPriority                  INTEGER (0..255),

highPriority                 INTEGER (0..255)} OPTIONAL,

vlanId                         INTEGER (0..4095) OPTIONAL,

associatedPhsiIndex            INTEGER (0..255) OPTIONAL,

classificationRuleIndex        INTEGER (0..65535) OPTIONAL,

ipV6FlowLabel                  INTEGER (0..1048575) OPTIONAL,

classificationAction           ENUMERATED {

none    (0),

discard (1)} OPTIONAL,

dscp                           INTEGER (0..64) OPTIONAL,

phsRule                        PhsRuleSet OPTIONAL

}

PhsRuleSet ::= SEQUENCE {

phsiField                      INTEGER (0..255) OPTIONAL,

phsfField                      OCTET STRING (SIZE(256))OPTIONAL,

-- phsmField[x] = 0 don't surpress x's byte,

-- phsmField[x] = 1 subpress x's byte

phsmField                      BIT STRING (SIZE (256)) OPTIONAL,

phssField                      INTEGER (0..255) OPTIONAL,

phsvField                      ENUMERATED {

verify      (0),

donotVerify (1)} OPTIONAL

}

RohcAttributesSet ::= SEQUENCE {

maxContextId                   INTEGER (0..65535) OPTIONAL,

largeContextId                 ENUMERATED {

smallCid (0),

largeCid (1)} OPTIONAL,

rohcMrru                       ENUMERATED {

noSegmentation (0),

mrru           (1)} OPTIONAL

}

------------------------------------------------- End of proposed text IX --------------------------------------------------