| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | AAI_L2-XFER message clean-up |  |
| Date Submitted | 2010-03-05 |  |
| Source(s) | Joey Chou Muthaiah Venkatachalam Intel | E-mail: joey.chou@intel.com |
| Re: | TGm AWD: |  |
| Abstract | This contribution proposes text of AAI_L2-XFER message clean-up |  |
| Purpose | Adopt proposed text. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

- AAI_L2-XFER message clean-up

- Joey Chou, Muthaiah Venkatachalam

- Intel

- Introduction

This contribution cleans up AAI_L2-XFER message.

- Proposed text

1. 	Proposed Text I

16.2.3.28 L2 Transfer message (AAI_L2_XFER)

------------------------------------------------- Start of proposed text I --------------------------------------------------

[Replace Table 706 as trhe following]

Table 706: AAI_L2-XFER

| M/O | Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| M | Control Message Type | TBD | The type value of this message | Present always |
| M | L2-Xfer Type | 8 | a)Transfer-Type = 1; GNSS assistance (DL) b)Transfer-Type = 2; LBS measurement [Terrestrial meas. and GNSS pseudo ranges] (UL) c)Transfer-Type = 3; Device Bootstrap (DL/UL) d)Transfer-Type = 4; WirelessMAN-OFDMA network boundary indication (DL) e)Transfer-Type = 5; ORAT-MSG (DL) f)Transfer-Type = 6: SMS g)Transfer-Type = 7: MIH Frame h)Transfer-Type = 8; ASN control messages for Relay support (DL/UL) i)Transfer-Type = 9; Emergency Alert j)Transfer-Type = 10-127; reserved k)Transfer-Type = 128-255; Vendor specific types | Present always |
| O | L2-Xfer SubType | 4 | e)Transfer-Type = 5; ORAT-MSG (DL) i)Sub-Type = 1: GERAN (GSM/GPRS/EGPRS) ii) Sub-Type = 2: UTRAN iii) Sub-Type = 3: E-UTRAN iv) Sub-Type = 4: TDSCDMA v) Sub-Type = 5: CDMA2000 f)Transfer-Type = 6: SMS i)Sub-Type = SMS data ii)Sub-Type = SMS confirmation g)Transfer-Type = 7: MIH Frame i)Sub-Type =1 : ES/CS MIH Capability Discovery ii)Sub-Type =2 : Event Service iii)Sub-Type = 3 : Command Service iv)Sub-Type = 4 : Information Service | Present only for Transfer Types 5,6,7 |
| O | L2-Xfer Payload | Variable | Variable |  |

------------------------------------------------- End of proposed text I --------------------------------------------------