| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | AAI_MSG-ACK and AAI_RES-CMD messages clean up |  |
| Date Submitted | 2010-03-05 |  |
| Source(s) | Joey Chou Intel | E-mail: joey.chou@intel.com |
| Re: | TGm AWD: |  |
| Abstract | This contribution proposes text to Clean up AAI_MSG-ACK and AAI_RES-CMD messages |  |
| Purpose | Adopt proposed text. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

- AAI_MSG-ACK and AAI_RES-CMD message clean up

- Joey Chou, Intel

- Introduction

This contribution is intended to clean up AAI_MSG-ACK and AAI_RES-CMD messages by providing missing information, such as size, value, conditions, that are required to build the message.

- Proposed text

II.1 Proposed text 1

16.2.3.35 AAI MSG-ACK

The ABS and AMS may use AAI_MSG-ACK to indicate the reception of a MAC control message. When receiving a message over control connection with Polling bit set to 1 in MCEH, the ABS and AMS shall transmit AAI_MSG-ACK as an acknowledgement to the reception of the message. The AAI_MSG-ACK shall include the following information:

—SN retrieved from MCEH of the received MAC PDU

—Control Connection Channel ID (CCC ID) that the MAC control message is received

------------------------------------------------- Start of proposed text I --------------------------------------------------

[Add a new table xxx]

Table xxx: AAI_MSG-ACK

| M/O | Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| M | Message type | 8 |  |  |
| M | SN | 8 | Message sequence number |  |
| M | CCC ID | 1 | Channel ID to identify separate fragmentation / reassembly state machines 0: channel 1 1: channel 2 |  |

------------------------------------------------- End of proposed text I --------------------------------------------------

II.2 Proposed text 2

16.2.3.49 AAI_RES-CMD (Reset command)

The AAI_RES-CMD message shall be transmitted by the ABS to force the AMS to reset itself, reinitialize its MAC, and repeat initial system access. This message may be used if an AMS is unresponsive to the ABS or if the ABS detects continued abnormalities in the UL transmission from the AMS. It contains no information except MAC message name.

------------------------------------------------- Start of proposed text II --------------------------------------------------

[Add a new table yyy]

Table yyy: AAI_RES-CMD

| M/O | Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| M | Message type | 8 | AMS reset command |  |

------------------------------------------------- End of proposed text II --------------------------------------------------

II.3 Proposed text 3

Annex P.

(Normative)

Definition of AAI MAC control messages

Annex P.2 MAC Control Message Definitions

------------------------------------------------- Start of proposed text III --------------------------------------------------

[Add the following ASN.1 code to Annex P.2]

[Add the following ASN.1 code and Table after table 687]

-- Message Acknowledge Message

AAI-MSG-ACK ::= SEQUENCE {

messageType                    OCTET STRING (SIZE(1)),

sequenceNumber                 INTEGER (0..255),

cccId                          ENUMERATED {

channel1 (0),

channel2 (1)}

}

------------------------------------------------- End of proposed text III --------------------------------------------------

II.4 Proposed text 4

Annex P.

(Normative)

Definition of AAI MAC control messages

Annex P.2 MAC Control Message Definitions

------------------------------------------------- Start of proposed text IV --------------------------------------------------

[Add the following ASN.1 code to Annex P.2]

-- Reset Command Message

AAI-RES-CMD ::= SEQUENCE {

messageType                    OCTET STRING (SIZE(1))

}

------------------------------------------------- End of proposed text IV --------------------------------------------------