| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposal to fix MLEH and other header issues |  |
| Date Submitted | 2010-03-15 |  |
| Source(s) | Joey Chou Aran Bergman Muthaiah Venkatachal Intel YihShen Chen MediaTek | E-mail: joey.chou@intel.com yihshen.chen@mediatek.com |
| Re: | TGm AWD: |  |
| Abstract | This contribution proposes text to fix MAC header issues |  |
| Purpose | Adopt proposed text. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

- MLEH and other Header issues

- Joey Chou,

- Aran Bergman,

- Muthaiah Venkatachal

- Intel

- Introduction

This contribution is intended to fix the following header issues:

- For an encrypted transport connection, the receiver needs to parse extended headers to know if MLEH is present, since the receiver has to know the MPDU length before the decryption.

- For a control connection, the receiver needs to parse MLEH and MCEH to know if the MPDU is cncrypted.

The above issues causes overhead to the implementation

- Proposed text

------------------------------------------------- Start of proposed text I--------------------------------------------------

II.1 Proposed text 1

16.2.2.1.1 Advanced Generic MAC Header (AGMH)

Table 653: AGMH Format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| Advanced Generic MAC Header() { |  |  |
| Flow ID | 4 | Flow identifier |
| EH | 1 | Extended header presence indicator; When set to ‘1’, this field indicates that an Extended Header is present following this AGMH. |
| Length | 11 | This field indicates the length in bytes of MAC PDU including the AGMH and extended header if present. If MLEH is present in a MPDU, Length field indicates 11 LSB of length in byte of MAC PDU. |
| } |  |  |

------------------------------------------------- End of proposed text I --------------------------------------------------

II.2 Proposed text 2

16.2.2.2 Extended header Group formats

------------------------------------------------- Start of proposed text II--------------------------------------------------

The inclusion of Extended Header group is indicated by EH bit in MAC Header. The extended headers format are defnined in Table 662 group (see Figure 386—), when used, shall always appear immediately after the MAC header. Extended headers format group shall not be encrypted. The fields of the extended header group format are defined in Table 662—.

[Editor Note: Replace Table 662 with the following table]

Table 662: Extended Headers Group Format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| Extended Headers Group () { |  |  |
| If (MAC Header == AGMH) { |  | AGMH |
| Extended length | 3 | Extend the MPDU length to 14 bits. If the length is not extended, they should be set to “0”. |
| If (FID == Transportation) { |  | Transport connection |
| Reserved | 4 |  |
| EH length flag | 1 | = 0, no EH = 1, include Total length of Extended Headers |
| } |  |  |
| else { |  | Control connection |
| EC | 1 | Encryption Control indicator 0 = Payload is not encrypted 1 = Payload is encrypted |
| Control Connection Channel ID (CCC ID) | 1 | Channel ID to identify separate fragmentation / reassembly state machines 0: channel 1 1: channel 2 |
| Polling | 1 | 0 = no acknowledgement required 1 = acknowledge required upon receiving the MAC message |
| Reserved | 1 |  |
| EH length flag | 1 | = 0, no EH = 1, include Total length of Extended Headers |
| EHLG = EH length flag |  |  |
| } |  |  |
| } |  |  |
| Else { |  | CMH |
| EHLG = 1 |  |  |
| } |  |  |
| if (EHLG == 1) { |  |  |
| Total length of Extended Headers | 8 | Total length of Extended Headers indicates the total length in bytes of all extended headers, and Total length of Extended Headers. |
| End = 0 |  | Indicator of the end of EH |
| For (i=1 ; !End ; i++) { |  |  |
| Type of EH[i] | 4 | Type of extended header as defined in Table 663. |
| Body of EH[i] | variable | The size of the extended header is determined by extended header type as specified in Table 663. The extended header including the extended header type is byte aligned. |
| Padding | variable | For byte alignment |
| If (Total length of Extended headers -1 <= (length(EH[1]) + … + length(EH[i]))) { |  |  |
| End = 1 |  |  |
| } |  |  |
| } |  |  |
| } |  |  |
| } |  |  |

------------------------------------------------- End of proposed text II --------------------------------------------------

------------------------------------------------- Start of proposed text III--------------------------------------------------

II.3 Proposed text 3

Delete Figure 386

II.4 Proposed text 4

Delete subcaluse 16.2.2.2.9 MAC PDU length extended header (MLEH)

II.5 Proposed text 5

Delete subcaluse 16.2.2.2.2 MAC Control extended header (MCEH)

------------------------------------------------- End of proposed text III--------------------------------------------------