| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed text to clarify broadcast A-A-MAP IE design and usage (16.3.6) |  |
| Date Submitted | 2010-3-5 |  |
| Source(s) | Yi Hsuan, Hujun Yin, Shantidev Mohanty, Tom Harel Intel Corporation |  |
| Re: | Category: AWD comments / Area: Chapter 16.3.6 (DL-CTRL) “Comments on AWD 16.3.6 DL-CTRL” |  |
| Abstract | The contribution proposes text to improve broadcast message coverage |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16 |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed text to clarify broadcast A-A-MAP IE design and usage (16.3.6)

Yi Hsuan, Hujun Yin, Shantidev Mohanty, Tom Harel

Intel Corporation

- Introduction

Broadcast A-A-MAP IE accepted in Session #65 allows a broadcast burst to be transmitted at a lower code rate than the regular data bursts. The lower code rate is achieved through

- Time domain repetition like HARQ with Chase combining so that the effective code rate is lower than the code rate in each transmission, and

- Signaling the burst size directly instead of ISizeOffset like regular data burst.

The current broadcast A-A-MAP design is generic enough to allow various ways to transmit different broadcast messages. In practice, certain broadcast messages should be limited to specific transmission format and some unnecessary transmission options should be removed for implementation simplicity.

- Reasons of proposed changes on broadcast A-A-MAP IE

- “Periodicity of Repetition” is not needed. When time repetition is applied, it is preferable to complete transmission of all repetition as soon as possible. Therefore the periodicity of repetition should be fixed to 1 frame. In other words, the repetition should happen at the same subframe(s) of the next frame.

- The supported burst size doesn’t need to be as large as a data burst. From our analysis [1], burst sizes up to 1 kilo bytes are sufficient for broadcast messages. Therefore we only need to support the first 44 entries (up to 1056 bytes) of Table 931 in D4. Reducing the supported burst size saves the buffer size required for broadcast busts.

- Although it’s commonly understood, D4 text doesn’t mention the modulation scheme used for broadcast bursts. QPSK should be specified.

- Some broadcast messages should not be repeated in time. Those messages should be spelled out clearly including: traffic indication (AAI_TRF-IND), paging advertisement (AAI_PAG-ADV), and paging group advertisement (PGID_Info).

- With up to 4 repeated transmissions of a broadcast burst in 4 frames, BS can avoid concurrent broadcast bursts transmission in the same time period. This would simplify the receiver processing such that AMS doesn’t need to track different broadcast process at the same time and thus BCID can be removed from the broadcast A-A-MAP IE. However the traffic indication message can happen quite often, potentially once every frame. Because traffic indication message cannot be repeated in time, it is not considered as a HARQ process. It is therefore reasonable to only allow transmission of broadcast bursts without time domain repetition during the period when another broadcast burst is transmitted repetitively. A flag needs to be added in the broadcast A-A-MAP to indicate the burst has time repetition or not, so that AMS knows which burst to combine if there is more than one in a subframe. For this reason, we should also remove the following sentence in D4: “All broadcast MAC control messages to be transmitted in an AAI subframe shall be included in a broadcast burst.”

-------------------------------  Start Text Proposal ---------------------------------------------------

- [ Recommended AWD Text Proposal (16.3.6.5.2.4.13, page 524, line 1) ]

16.3.6.5.2.4.13 Broadcast Assignment A-MAP IE

Broadcast assignment A-MAP allocates resource for a broadcast burst. All broadcast MAC control messages to be transmitted in a AAI subframe shall be included in a broadcast burst. A broadcast burst contains one or more broadcast MAC control messages. Table 839 describes the fields in a broadcast assignment A-MAP IE.

Table 829 – Broadcast Assignment A-MAP IE*

| Syntax | Size in bits | Description/Notes |
| --- | --- | --- |
| Broadcast-A-MAP_IE() { | - | - |
| A-MAP IE Type | 4 | Broadcast A-MAP IE |
| Function Index | 1 | 0: This IE carries broadcast assignment information 1: This IE carriers ranging channel allocation information. |
| If (Function Index == 0){ |  |  |
| Burst Size | 6 | Burst size as indicated in the first 44 64 entries of Table 923 |
| Resource Index | 11 | 5 MHz: 0 in first 2 MSB bits + 9 bits for resource index 10 MHz: 11 bits for resource index 20 MHz: 11 bits for resource index Resource index includes location and allocation size |
| Long TTI Indicator | 1 | Indicates number of AAI subframes spanned by the allocated resource. 0b0: 1 AAI subframe (default) 0b1: 4 DL AAI subframes for FDD or all DL AAI subframes for TDD |
| BCID | 2 | Broadcast channel ID |
| Transmission Format | 1 | 0b0: no time domain repetition 0b1: with time domain repetition |
| Repetition | 2 | 0b00: no more repetition of the same burst 0b01: the same burst will be transmitted one more time 0b10: the same burst will be transmitted two more times 0b11: the same burst will be transmitted three more times |
| Periodicity of Repetition | 2 | 0b00: 1 frame periodicity 0b01: 2 frame periodicity 0b10: 3 frame periodicity 0b11: 4 frame periodicity |
| Reserved | TBD | Reserved bits |
| } else if (Function Index == 1){ |  |  |
| Number of Ranging Opportunities (N) | 2 | TBD |
| For (i=0; i<N; i++){ |  |  |
| Subframe index | 3 |  |
| } |  |  |
| Reserved |  |  |
| } |  |  |
| } |  |  |

*A 16 bit CRC is generated based on the contents of the Broadcast Assignment A-MAP IE.

The broadcast burst message signaled by the Broadcast Assignment A-MAP IE is always transmitted using SFBC as the MIMO encoder format and QPSK as the modulation scheme.

During the period when a broadcast burst is transmitted repetitively in time, other broadcast bursts can only be transmitted without time domain repetition. For each broadcast burst transmission, there is a broadcast A-A-MAP IE to indicate the resource index and remaining repetition number. SPID is always 0 for broadcast burst transmission.

-------------------------------------- End of Text Proposal -------------------------------------------