| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Equations for A-A-MAP Group Size Indication with Coding Rate (1/8, 1/2, 1/4) (AWD-16.3.6.5.2.1) |  |
| Date Submitted | 2010-03-14 |  |
| Source(s) | Ping Wang Yi Hsuan Hujun Yin Intel Corporation | ping.wang@intel.com yi.hsuan@intel.com hujun.yin@intel.com |
| Re: | Category: AWD comments / Area: Chapter 16.3.6.5.2.1 (DL-CTRL) “Comments on AWD 16.3.6.5.2.1 DL-CTRL” |  |
| Abstract | The contribution proposes the addition of coding rate (1/8, 1/2, 1/4) A-A-MAP group size indication. |  |
| Purpose | To be discussed and approved by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Equations for A-A-MAP Group Size Indication with Coding Rate (1/8, 1/2, 1/4) (AWD-16.3.6.5.2.1)

Ping Wang, Yi Hsuan, and Hunjun Yin

Intel Corporation

- 1. Introductions

The following texts [1] at the IEEE 65th meeting on January 11th to 15th, 2010 were approved and adopted in  P802.16m D4, page 448, line 32:

- QPSK 1/2 and QPSK 1/8 can be used for assignment A-MAP and QPSK 1/4 is used for A-A-MAP in the power-boosted reuse 3 partition of FFR.

The coding rate set (1/8, 1/2, 1/4) instead of the coding rate set (1/8, 1/2, 1/2) is used to improve the reception reliability of A-A-MAP IEs.

But the generation equations of group size indication is used for the coding rate set (1/8, 1/2, 1/4) in the current P802.16m D4. So it is necessary to re-design the generation equations of group size indication for the coding rate set (1/8, 1/2, 1/4) in order to replace the coding rate set (1/8, 1/2, 1/2) in the current P802.16m D4. Additionally, it is noted that the design assumptions in the contribution IEEE C802.16m-09/3070r2 are also applied for the generation equation of the coding rate set (1/8, 1/2, 1/4).

- 2. Proposed Equations to Generate A-A-MAP Group Size Indication for Coding Rate Set (1/8, 1/2, 1/4) Instead of (1/8, 1/2, 1/2)

The following describes how to generate the resource allocation indication for A-A-MAP to meet the resource wastes as few as possible:

Ntotal=[4∗G1(I)+G2(I)+2∗G3(I)]≤48

wherein Ntotal (0≤Ntotal≤48) is the total number of MLRUs for A-A-MAP; G1(I) [0≤G1(I)≤12] is the number of FFR Reuse 1 A-A-MAP in Group 1 using QPSK 1/8; G2(I) [0≤G2(I)≤48] is the number of FFR Reuse 1 A-A-MAP in Group 2 using QPSK 1/2; G3(I) [0≤G3(I)≤24] is the number of FFR Reuse 3 A-A-MAP in Group 3 using QPSK 1/4.

Group 1 is not adjacent to data transmission in FFR Reuse 1, Group 2 is adjacent to data transmission in FFR Reuse 1, and Group 3 is adjacent to data transmission in FFR Reuse 3. Therefore unoccupied MLRUs in Group 1 are wasted. In order to reduce the waste as much as possible, the following rules are applied to remove the combinations first:

  - G1(I) may be from 0 to floor(Ntotal /4), and Ntotal = 0:48;

  - G2(I) may be Ntotal - 4* G1(I) – 2*G3(I);

  - G3(I) may be floor[(Ntotal - 4* G1(I))/3];

The total number of the combinations is reduced to 325, which is still larger than the requirement of 8-bit signaling overhead.

  - All combinations with Ntotal no larger than 24 shall be retained by considering the 5, 7, 8.75, and 10MHz bandwidths, which are about 91.

  - The 165 (=256-91) combinations can be generated by retaining the number [6,6,6,7,7,7,7,7,7,7,7,7,7,7,7,7,7,7,7,7,7,7,7,7] of A-A-MAP for Group 1 when Ntotal is from 25 to 48 MLRUs, respectively.

Based on the above descriptions, the resultant number of Assignment A-MAPs for Group 1, Group 2, and Group 3 can be formulated as follows:

G1(I)

IIb(Ntotal)+kIb(Ntotal)0,Ntotal=0Ib(Ntotal−1)+(Ntotal−1)4+1,1≤Ntotal≤25Ib(Ntotal−1)+6,26≤Ntotal≤28Ib(Ntotal−1)+7,29≤Ntotal≤480,otherwise

Wherein G1(I) is the number of Assignment A-MAPs for Group 1 using QPSK 1/8; G2(I) is the number of Assignment A-MAPs for Group 2 using QPSK 1/2; G3(I) is the number of Assignment A-MAPs for Group 3 using QPSK 1/4; I is the 8-bit index at non-user-specific A-MAP IE, which ranges from 0 to 255. Ntotal (0≤Ntotal≤48) is the total number of MLRUs for A-A-MAP; Ib(Ntotal) is the base index for a given Ntotal value.

Figure 1(a): Average Resource Wastes and Average Extra Blind Detection for the Proposed Equations

Figure 1(b): Maximum Blind Detection for the Proposed Equations

# Conclusions

It is proposed to adopt the subsequently recommended texts in this contribution, which can make the upcoming P802.16m texts more consistent.

- References

[1] IEEE C802.16m-10/0011r1: Proposed text to improve A-A-MAP reliability in FFR (16.3.6), Intel, January 11th, 2010.

[2] IEEE C802.16m-09/3070r2: Text Cleanup on Non-user-specific A-MAP IE (AWD-16.3.6.5.2.1), Intel, January 9th, 2010.

[3] P802.16m/D4: Advanced Air Interface, February 3rd, 2010.

[4] IEEE C802.16m-09/1940r1: A-A-MAP Group Size Indication in Non-User Specific A-MAP IE (AWD-15.3.6), Intel, LGE, and Samsung, September 1st, 2009.

[5] IEEE C802.16m-09/1365r1: A-A-MAP Group Size Indication in Non-User Specific A-MAP IE (AWD-15.3.6), Intel, July 9th, 2009.

[6] IEEE C802.16m-09/1361r2: Proposal for non-user specific A-MAP design in the 802.16m AWD (AWD-15.3.6), Intel, July 14th, 2009.

[7] IEEE C802.16m-09/1419: Proposed Text of A-MAP structure and signaling for IEEE 802.16m Amendment, LGE, July 6th, 2009.

-------------------------------  Start Text Proposal ---------------------------------------------------

- [ ----- Replace all the text from Line 39 of Page 457 to Line 11 of Page 458 by  the following recommended AWD text-----]

For FFR configuration with Group 1 using QPSK 1/8, Group 2 using QPSK 1/2, and Group 3 using QPSK 1/4, the lookup table can be generated using Equation (211) to Equation (215) and looping through all values of Ntotal and k.

G1(I)

IIb(Ntotal)+k(214)Ib(Ntotal)0,Ntotal=0Ib(Ntotal−1)+(Ntotal−1)4+1,1≤Ntotal≤25Ib(Ntotal−1)+6,26≤Ntotal≤28Ib(Ntotal−1)+7,29≤Ntotal≤480,otherwise(215)

------------------------------------- End of proposed texts ------------------------------------------------------------------