| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Listening Window extending control in IEEE 802.16m |  |
| Date Submitted | 2010-03-18 |  |
| Source(s) | Guanghui Fan, Lei Jin, Bin Chen, Jia Lin Huawei Technologies Giwon Park LG Electronics Yeongmoon Son Samsung Electronics | E-mail: fanguanghui@huawei.com jinlei60020191@huawei.com binchen@huawei.com linjia@huawei.com |
| Re: | Call for LB #31 on “ P802.16m/D4”: Target topic: “ 16.2.16.2.3.2” |  |
| Abstract | This contribution proposes the behavior of the AMS/ABS when the Listening Window is extended explicitly in IEEE 802.16m. |  |
| Purpose | To be discussed and adopted by TGm for 802.16m Letter Ballot 31. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Listening Window extending control in IEEE 802.16m

Guanghui Fan, Lei Jin, Bin Chen, Jia Lin

Huawei Technologies

Giwon Park

LG Electronics

Yeongmoon Son

Samsung Electronics

Introduction

When the ABS/AMS sends an explicit listening window extending indication using SCH/SCEH, the corresponding receiver AMS/ABS has better to accept the request, as the request is mainly induced by traffic needs. But in current text, the receiver may choose to terminate the listening window when it has received a explicit listening window extending request. It’s not reasonable.

At the same time, both the SCH/SCEH request for extending and terminating the listening window should be ACKed by SCH/SCEH with response indication =1, otherwise it may leads to nonsynchronous of the sleep cycle control.

Text proposal for inclusion in the P802.16m/D4

========================== Start of Proposed Text ==============================

[Editor’s Note: Modify text in page 302 in “16.2.16.2.3.2” as follows]

16.2.16.2.3.2 Listening Window extension

In order to provide scheduling flexibility and to take advantage of radio link conditions and to reduce control signaling latency of AMSs, the Listening Window may also be extended explicitly. The ABS may send an explicit signaling in Sleep Control Extended header or Sleep Control header SCH or SCEH including the number of frame for extended listening window to control extension of Listening Window during the Default Listening Window. Upon receiving the explicit signaling, the AMS shall send SCH/SCEH with "Response Indication" bit =1 and extend its Listening Window until the frame specified in signaling or terminate the listening window. The AMS may also send an explicit signaling in Sleep Control Extended header or Sleep Control header SCH or SCEH including the number of frame for extended listening window to control extension of Listening Window during the Default Listening Window. Upon receiving the explicit signaling to extend Listening Window, the ABS shall send SCH/SCEH with "Response Indication" bit =1 and regard extend the AMS's as extending the Listening Window until the frame specified in signaling SCH or SCEH or terminate the listening window.

The ABS may send an explicit indication using Sleep Control Extended Header or Sleep Control Header SCH or SCEH to terminate the current Listening Window. The AMS may send an explicit indication using Sleep Control Extended Header or Sleep Control Header SCH or SCEH to terminate the current Listening Window. In this case, the ABS shall send SCH/SCEH with "Response Indication" bit =1 and regard the AMS as returning to sleep (i.e. the Sleep Window starts). When an ABS has a last PDU in the DL buffer during the listening window, the ABS may transmit an explicit indication using Sleep Control Extended Header or Sleep Control Header SCH or SCEH with "Response Indication" bit =0 provided that it allows to terminate the current Listening Window. In this case, Upon receiving the SCH/SCEH with "Response Indication" bit =1 from the AMS, the ABS shall send SCH/SCEH with "Response Indication" bit =0 to acknowledge the request and regard the AMS as returning to sleep (i.e., the Sleep Window starts).

========================== End of Proposed Text ==============================