| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | LBS Support for Multi-carrier in 16m |  |
| Date Submitted | 2010-03-04 |  |
| Source(s) | Jia Lin, Jianmin Lu, Bin Chen, Junxian Mo Huawei Technologies | E-mail: linjia@huawei.com lujianmin@huawei.com binchen@huawei.com |
| Re: | Call for LB #31 on “ P802.16m/D4”: Target topic: “16.8.3” |  |
| Abstract | This contribution proposes LBS Support for Multi-carrier in 16m. |  |
| Purpose | To be discussed and adopted by TGm for 802.16m LB#31. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

LBS Support for Multi-carrier in 16m

Jia Lin, Jianmin Lu, Bin Chen, Junxian Mo

Huawei Technologies

# Introduction

Location based service (LBS) are information services accessible with mobile devices through the mobile network and utilizing the ability to make use of the location of the mobile device.

As D4 describes for non-GPS measurement method, in DL, the AMS receives signals which are existing signals (e.g. preamble sequence) or new signals designed specifically for the LBS measurements, if it is needed to meet the requirement from the serving/attached ABS and multiple neighboring ABSs/ARSs. The AMS accurately calculates the required measurements, even in the presence of multi-path channel and heavy interference environment, and then estimates its location accordingly.

In UL, Various approaches can be utilized at the serving/attached ABS/ARS to locate the AMS such as TOA and AOA. These measurements are supported via existing UL transmissions (e.g. ranging sequence) or new signals designed specifically for the LBS measurements.

Since Multi-carrier is introduced into IEEE802.16m, in this contribution, we propose LBS techniques which take advantage of the multi-carrier operation.

# 2. LBS support for Multi-carrier

For non-GPS measurement method, in DL/UL, AMS receives/transmits signals from/to multiple ABSs for LBS measurement. As is known for single carrier operation, most LBS measurement methods (such as TOA and TDOA) need to be performed between AMS and multiple ABSs at different times. It means that AMS only receives/transmits measurement signals from/to one ABS per transmission/reception opportunity. For example, when U-TDOA measurement is being performed, serving ABS should negotiate with neighbor ABSs to get dedicated ranging resource for location measurement, and send these measurement parameters to AMS. Then, AMS can use the dedicated ranging information and measurement parameters to send dedicated CDMA codes to these ABSs through single carrier at different rendezvous times.

For Multi-carrier scenario, we propose that LBS measurement can be performed through multiple carriers at the same time, which means that it can reduce signaling overhead and time delay. The ABS can inform the AMS to perform LBS measurement procedures (such as TOA, TDOA) through one or multiple AMS’s carriers (primary carrier or secondary carriers) for LBS measurement. The message may include carrier indices, LBS measurement methods, ABS IDs, Rendezvous time, CDMA codes, Tx opportunity offsets, etc.

Meanwhile, when data is being transmitted between AMS and ABS through one or multiple carriers, LBS measurement can be performed in the AMS’s other carriers at the same time, which helps maintain QoS of data traffic.

# 3. Summary

LBS support in Multi-carrier provides the following benefits.

1) Reduced time delay for perform LBS measurement

2) Increased LBS measurement accuracy

3) Reduced signaling overhead for perform LBS measurement

4) Maintain QoS of data traffic

# 4 Proposed Text

Text proposal for inclusion in the P802.16m/D4

================================Start of Proposed Text ==============================

16.8.3 Enhanced LBS Support

The Enhanced LBS (ELBS) is an optional LBS capability involving the following functions:

•ABS ability to transmit special ELBS waveforms to be used for location specific measurements to enable more accurate location determination.

•ABS’s ability to signal to the AMS the location of the special ELBS waveforms.

•AMS’s ability to detect, measure and report relevant measurements to the ABS.

•LBS measurement can be performed through multiple carriers.

==================================== End of the Text ================================