| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text for Trigger function in Multi-carrier Operation in 802.16m (16.2.6.2) |  |
| Date Submitted | 2010-03-04 |  |
| Source(s) | Jia Lin, Lei Jin, Bin Chen, Junxian Mo Huawei Technologies | E-mail: linjia@huawei.com jinlei60020191@huawei.com binchen@huawei.com |
| Re: | Call for LB #31 on “ P802.16m/D4”: Target topic: “16.2.6.2” |  |
| Abstract | This contribution proposes amendment text describing trigger function in multi-carrier scenario in 802.16m. |  |
| Purpose | To be discussed and adopted by TGm for 802.16m LB#31. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

- Proposed Text for Trigger function of Multicarrier Scenario

- in 802.16m

Jia Lin, Lei Jin, Bin Chen, Junxian Mo

- Huawei Technologies

- Introduction

- This contribution proposes amendment text describing the Trigger function when a 16m ABS and AMSs support multiple carriers.

- In Multicarrier Scenario, AMS may use multiple carriers for data transmission. When AMS scans carriers of Serving ABS and Neighbor ABS's, it may trigger the actions such as respond on trigger with AAI_SCN-REP or AAI_HO-REQ. Since AMS need to communicate with ABS by multiple carriers, if AMS find there are other carriers of Neighbor ABS or Serving ABS satisfying trigger functions, it is possible that AMS is triggered to report these conditions or response with HO-REQ.

- In 16m/D4, it only considers trigger conditions definitions for single carrier scenario, but not for multicarrier scenario. In this case, Total bandwidth of carriers (The metric of these carriers is greater than absolute value) in neighbor ABS and Serving ABS should be measured and calculated.

- References

- IEEE P802.16-2009 “IEEE Standard for Local and Metropolitan Area Networks: Air Interface for Broadband Wireless Access”.

- IEEE 802.16m-08/003r9, “The Draft IEEE 802.16m System Description Document”

- IEEE P802.16m/D4, “Part 16: Air Interface for Fixed and Mobile Broadband Wireless Access Systems – DRAFT Amendment to IEEE Standard for Local and metropolitan area networks”

- Text proposal for inclusion in the P802.16m/D4

========================== Start of Proposed Text ==========================

[Note to Editor: Add the following contents in Table 745 in “16.2.6.2 Trigger condition definitions”]

- 16.2.6.2 Trigger condition definitions

Table 732—Trigger; Type/Function/Action Description

| Name | Length (Bits) | Value |
| --- | --- | --- |
| Type | 2 (MSB) | Trigger metric type: 0x0: CINR metric 0x1: RSSI metric 0x2: RTD metric 0x3: Number of missed superframes metric |
| Function | 34 | Computation defining trigger condition: 0x0: Reserved 0x1: Metric of neighbor ABS is greater than absolute value 0x2: Metric of neighbor ABS is less than absolute value 0x3: Metric of neighbor ABS is greater than serving ABS metric by relative value 0x4: Metric of neighbor ABS is less than serving ABS metric by relative value 0x5: Metric of serving ABS greater than absolute value 0x6: Metric of serving ABS less than absolute value 0x7: Reserved 0x8: Total bandwidth of those neighbor ABS’s carriers whose metric are greater than absolute value is more than specific value 0x9: Total bandwidth of those Serving ABS’s carriers whose metric are greater than absolute value is less than specific value. 0x10: Total bandwidth of those neighbor ABS’s carriers whose metric are greater than absolute value is more than total bandwidth of those serving ABS’s carriers whose metric are greater than specific value. 0x11: Total bandwidth of those neighbor ABS’s carriers whose metric are greater than absolute value is more than total bandwidth of those serving ABS’s carriers whose metric are greater than absolute value by relative value. 0x12-0x15: Reserved NOTE-0x1-0x4 and 0x8-0x11 not applicable for RTD trigger metric NOTE-When type 0x1 is used together with function 0x3 or 0x4, the threshold value shall range from -32 dB (0x80) to +31.75 dB (0x7F). When type 0x1 is used together with function 0x1, 0x2, 0x5 or 0x6, the threshold value shall be interpreted as an unsigned byte with units of 0.25 dB, such that 0x00 is interpreted as -103.75 dBm and 0xFF is interpreted as -40 dBm NOTE-Type 0x3 can only be used together with function 0x5 or function 0x6 NOTE- Functions 0x8-0x11 are only used for Multicarrier scenario. |
| Action | 3(LSB) | Action performed upon reaching trigger condition: 0x0: Reserved 0x1: Respond on trigger with AAI_SCN-REP 0x2: Respond on trigger with AAI_HO-REQ 0x3: Respond on trigger with AAI_SCN-REQ 0x4: Declare ABS unreachable: If this ABS is the serving ABS, AMS sends AAI_HO-IND with code 0x03 to the serving ABS and proceeds as specified in section 15.2.5.2.4. If this ABS is a target ABS, the AMS needs not take immediate action when this trigger condition is met for a single ABS. The AMS shall act only when this condition is met for all target ABSs included in AAI-HO-CMD during HO execution. The specific actions are described in section 15.2.5.2.4. 0x5: Cancel HO 0x6 and 0x7: Reserved NOTE-0x3 is not applicable when neighbor ABS metrics are defined (i.e., only Function values 0x5 or 0x6 are applicable). |

[Note to Editor: Add the following contents in Table 732 in “16.2.3.12 AAI_NBR-ADV”]

- 16.2.3.12 AAI_NBR-ADV

Table 732—Trigger; AAI_NBR-ADV message format

| Syntax | Size (Bits) | Note |
| --- | --- | --- |
| Neighbor -specific trigger TLVs | variables | Optional neighbor- specific triggers with encoding defined in Table 744 |
| Neighbor-MC-specific trigger TLVs | variables | Optional Neighbor MC-specific triggers with encoding defined in Table 744 |

[Note to Editor: Add the following contents in “16.2.3.12 AAI_NBR-ADV” (Page 206, line 8)]

The ABS may define neighbor-specific triggers/ neighbor-MC-specific triggers by including neighbor-specific trigger TLVs/neighbor-MC-specific trigger TLVs in the AAI_NBR-ADV message. The AMS evaluates neighbor-specific triggers only for the specific neighbor BS metric, and evaluates neighbor-MC-specific triggers only for the specific neighbor BS’s MC metric.

Neighbor-specific trigger TLVs use the format in Table 744—, where only function types 0x1, 0x2, 0x3 and 0x4 and actions types 0x1 and 0x2 are allowed. When present, neighbor-specific handover triggers override any general trigger TLVs (define in the AAI_SCD message) of the same type, function and action.

Neighbor-MC-specific trigger TLVs use the format in Table 744, where only function types 0x8, 0x9, 0x10 and 0x11 and actions types 0x1 and 0x2 are allowed. When present, neighbor-MC-specific handover triggers override any general trigger TLVs (define in the AAI_SCD message) of the same type, function and action.

========================== End of Proposed Text ===========================